3GPP TSG-RAN WG4 Meeting#97-e                                 R4-2016134

E-meeting,2nd Nov – 17th Nov, 2020

Agenda item:	11.1.3.2

Source: 	ZTE Corporation

Title: 	DL simulation results for 6425-7125MHz and 10-10.5GHz

Document for:	Discussion

Introduction

In the RAN4#96-e-Bis meeting, WF on simulation assumption [1] was approved for further coexistence evaluation work. Therefore in this contribution, we share some updated simulation results for further discussion.

Discussion

In the following sections, simulation results for both coordinated case and uncoordinated case in urban macro scenarios are provided based on the approved simulation assumption [1]. In addition, baseline ACIR for demonstration based on legacy FR1 NR system is assuming that using ACLR=45dBc at BS and ACS=33dBc at UE) at 7GHz and 10GHz, relative offset to baseline ACIR is used for further comparison and check whether relative relaxed requirements could be achieved.

## 2.1. Downlink

The DL simulation results for 7GHz and 10GHz are summarized in the following Table 1 and ACIR results are shown in Figure 1/2/3/4.

Table 1: DL throughput loss of victim UE at 7GHz and 10GHz

| Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| Coordinated case | Average throughput loss in % (7GHz) | 1.44 | 1.63 | 1.84 | 2.11 | 2.53 | 2.87 |
|  | 5%-tile throughput loss in % (7GHz) | 3.19 | 3.59 | 3.92 | 4.30 | 4.91 | 6.06 |
| Uncoordinated case | Average throughput loss in %(7GHz) | 1.65 | 1.89 | 2.10 | 2.33 | 2.63 | 2.99 |
|  | 5%-tile throughput loss in % (7GHz) | 4.39 | 5.18 | 5.98 | 7.07 | 7.87 | 9.06 |
| Coordinated case | Average throughput loss in % (10GHz) | 1.17 | 1.39 | 1.61 | 1.84 | 2.10 | 2.47 |
|  | 5%-tile throughput loss in % (10GHz) | 1.77 | 2.01 | 2.84 | 3.94 | 4.80 | 5.86 |
| Uncoordinated case | Average throughput loss in % (10GHz) | 1.52 | 1.68 | 1.93 | 2.12 | 2.31 | 2.62 |
|  | 5%-tile throughput loss in % (10GHz) | 3.36 | 3.79 | 4.79 | 5.78 | 6.41 | 7.63 |

Figure 1. uncoordinated urban macro scenario @7GHz

Figure 2. uncoordinated urban macro scenario @10GHz

Figure 3. coordinated urban macro scenario @7GHz

Figure 4. coordinated urban macro scenario @10GHz

Conclusions

In this contribution, we shared updated simulation results for 6.425-7.125GHz and 10.0-10.5GHz for further discussion.

Proposal 1: for 7GHz, the downlink throughput loss of the victim UE in the urban macro scenario can still be limited to 5% with downlink ACIR offsets of -1dB;

Proposal 2: for 10GHz, the downlink throughput loss of the victim UE in the urban macro scenario can still be limited to 5% with downlink ACIR offsets of -3dB.

References

- R4-2011827, WF on the Simulations for the SI on 6.425-7.125GHz and 10.0-10.5GHz, approved.