3GPP TSG RAN WG4 Meeting #97-e	R4-2016171

Electronic Meeting, November 2-13, 2020

Agenda Item:	7.4.4

Source: 	Ericsson

Title:	Issues with IAB RRM requirements

Document for:	Discussion

- Introduction

The RRM core requirements for IAB were completed in the last meeting. However, few issues have been identified and are analyzed in this contribution.

# RRM requirement applicability in non-DRX

The RRM requirements defined for both local area IAB-MT and wide area IAB-MT in TS 38.174 are applicable in non-DRX. However, the definition of non-DRX or no DRX used in TS 38.174 is not clear.

When IAB-MT is not configured with DRX then it is obvious that the IAB-MT is in non-DRX and shall meet all the requirements defined in TS 38-174. However, even the IAB-MT is configured with DRX parameters can in non-DRX when the IAB-MT receiver is active due to different reasons like for the UE (section 3.6.1, TS 38.133).

We therefore suggest to define conditions under which the IAB-MT shall assume no DRX is used in TS 38.174 as follows:

The IAB-MT shall assume that no DRX is used provided the following conditions are met:

-	DRX parameters are not configured or

-	DRX parameters are configured and

-	drx-InactivityTimer is running or

-	drx-RetransmissionTimerDL is running or

-	drx-RetransmissionTimerUL is running or

-	ra-ContentionResolutionTimer is running or

-	a Scheduling Request sent on PUCCH is pending or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the preamble not selected by the MAC entity

- Observation 1: All IAB-MT RRM requirements are applicable when no DRX is used. But definition of no DRX is missing in TS 38.174.

- Proposal 1: Conditions under which IAB-MT shall assume no DRX is used are defined in TS 38.174.

- Proposal 2: In proposal 1, conditions are the same as defined for the UE in section 3.6.1, TS 38.133.

# Measurement gaps for signalling characteristics

The RLM and link recovery requirements defined for local area IAB-MT depend on measurement gap configuration. More specifically the parameter, P, used in the evaluation periods of RLM and BFD depends on whether SMTC occasion does not overlap at all, partially overlaps or fully overlaps with measurement gap. The level of overlap depends on MGL, MGRP, relation between MGRP and SSB periodicity or relation between MGRP and CSI-RS periodicity.

However, it is not clear which measurement gap configurations are applicable for local area IAB-MT when performing RLM and beam management. This needs to be clarified.

- Observation 2: IAB-MT RLM and link recovery requirements are impacted by the measurement gap configuration. But applicable measurement gaps for local area IAB-MT for meeting RLM and link recovery requirements are missing in TS 38.174.

- Proposal 3: Specify applicable measurement gap configurations for local area IAB-MT for meeting RLM and link recovery requirements in TS 38.174.

- Summary

In this paper we have identified several issues related to IAB-MT RRM requirements. Based on the analysis following are the main observation and proposals:

- Observation 1: All IAB-MT RRM requirements are applicable when no DRX is used. But definition of no DRX is missing in TS 38.174.

- Proposal 1: Conditions under which IAB-MT shall assume no DRX is used are defined in TS 38.174.

- Proposal 2: In proposal 1, conditions are the same as defined for the UE in section 3.6.1, TS 38.133.

- Observation 2: IAB-MT RLM and link recovery requirements are impacted by the measurement gap configuration. But applicable measurement gaps for local area IAB-MT for meeting RLM and link recovery requirements are missing in TS 38.174.

- Proposal 3: Specify applicable measurement gap configurations for local area IAB-MT for meeting RLM and link recovery requirements in TS 38.174.

The CR to TS 38.174 on the definition of non-DRX is provided in [2].

- References

- R4-2012211, Email discussion summary for [96e][211] NR_IAB_RRM, ZTE

- R4-2016170, “Symbols, abbreviations and definitions for IAB RRM in 38.174”, Ericsson