3GPP TSG-WG RAN4 Meeting #97-e	R4-2016191

# Electronic meeting, 2nd – 13th November, 2020

Source: 		Nokia, Nokia Shanghai Bell

Title: 			TP to TR 38.847: BS RF requirements

Agenda Item: 	10.28.2

Document for:	Approval

# Introduction

During RAN4#96-e, document [1] with the following proposal was approved:

Proposal 1: Introduce the following step frequencies for defining the radiated Tx/Rx spurious emission limits for n262:

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |
| n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |

This document provided text proposal to TR 38.847 on n262 BS requirements.

# Text proposal

### 8.2.1	Transmitter characteristics

On top of generic FR2 BS RF and test requirements, the following 38.104/38.141-2 transmitter requirements changes are expected due to introduction of n262:

- Introduction of step frequencies for defining the radiated Tx spurious emission limits for n262 (impact to both 38.104 and 38.141-2)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |
| n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |

- Update test requirements for radiated transmit power/TRP measurement results range/measured mean EIRP spectral density (impact to 38.141-2)

|  | Normal test environment | Extreme test environment |
| --- | --- | --- |
| BS type 1-H | f ≤ 3 GHz: ± 3.3 dB | N/A |
|  | 3 GHz < f ≤ 6 GHz: ± 3.5 dB |  |
| BS type 1-O | f ≤ 3 GHz: ± 3.3 dB | f ≤ 3 GHz: ± 5.2 dB |
|  | 3 GHz < f ≤ 6 GHz: ± 3.5 dB | 3 GHz < f ≤ 4.2 GHz: ± 5.3 dB |
|  |  | 4.2 GHz < f ≤ 6 GHz: ± 5.3 dB |
| BS type 2-O | 24.15 GHz < f ≤ 29.5 GHz: ± 5.1 dB 37 GHz < f ≤ [48.2] GHz: ± 5.4 dB … | 24.15 GHz < f ≤ 29.5 GHz: ± 7.6 dB - GHz < f ≤ [48.2] GHz: ± 7.8 dB |

-	within +5.4 dB and –5.4 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP for carrier frequency 37 GHz < f ≤ [48.2] GHz.

The measured mean EIRP spectral density according to subclause 6.5.2.4.2 shall be less than -32.7 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 37 GHz < f ≤ [48.2] GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

- Define (0.2dB difference to 43.5GHz is proposed to reflect larger measurement uncertainty for higher frequencies) test tolerances for FR2 OTA transmitter tests to be applicable up to 48.2GHz (impact to 38.141-2)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See TS 38.104 [2], subclause 9.2 | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz [2.2] dB, 43.5GHz < f ≦ 48.2GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz [3.5] dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 OTA base station output power | See TS 38.104 [2], subclause 9.3 | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43.5GHz [2.6] dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4 OTA output power dynamics | See TS 38.104 [2], subclause 9.4 | 0.4 dB | Formula: Total power dynamic range – TT |
| 6.5.1 OTA transmitter OFF power | See TS 38.104 [2], subclause 9.5.2 | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz [3.5] dB, 43.5GHz < f ≦ 48.2GHz | Formula: Minimum Requirement + TT |
| 6.6.2 OTA frequency Error | See TS 38.104 [2], subclause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.6.3 OTA Modulation quality (EVM) | See TS 38.104 [2], subclause 9.6.2 | 1 % | Formula: EVM limit + TT |
| 6.6.4 OTA time alignment error | See TS 38.104 [2], subclause 9.6.3 | 25 ns |  |
| 6.7.2 OTA occupied bandwidth | See TS 38.104 [2], subclause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | See TS 38.104 [2], subclause 9.7.3 | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦ 43.5GHz [2.8] dB, 43.5GHz < f ≦ 48.2GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦ [48.2]GHz | Formula: Relative limit - TT Absolute limit +TT |
| 6.7.4 OTA operating band unwanted emissions | See TS 38.104 [2], subclause 9.7.4 | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦ [48.2]GHz 0.1*BWcontiguous f < fmax 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category A | See TS 38.104 [2], subclause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category B | See TS 38.104 [2], subclause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |

### 8.2.2	Receiver characteristics

On top of generic FR2 BS RF and test requirements, the following 38.104/38.141-2 receiver requirements changes are expected due to introduction of n262:

- Introduction of step frequencies for defining the radiated Rx spurious emission limits for n262 (impact to both 38.104 and 38.141-2)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |
| n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |

- Define test tolerances for FR2 OTA receiver tests to be applicable up to 48.2GHz (impact to 38.141-2)

| Subclause | Maximum OTA Test System uncertainty |
| --- | --- |
| 7.3 OTA reference sensitivity level | ±2.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±2.4 dB, 37 GHz < f ≤ 48.2 GHz |
| 7.5.1 OTA adjacent channel selectivity | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 48.2 GHz |
| 7.5.2 In-band blocking (General) | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 48.2 GHz |
| 7.6 OTA out-of-band blocking | ±4.1 dB |
| 7.7 OTA receiver spurious emissions | ±2.5 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |
| 7.8 OTA receiver intermodulation | ±3.9 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.9 dB, 37 GHz < f ≤ 48.2 GHz |
| 7.9 OTA in-channel selectivity | ±3.4 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.4 dB, 37 GHz < f ≤ 48.2 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

# Conclusion

This contribution provides the text proposal on summary of expected changes to 38.104 and 38.141-2. It is proposed to agree on this text proposal.

# Reference

[1] R4-2011412		BS RF requirements for 47 GHz band, Nokia, RAN4#96e