3GPP TSG-RAN WG4> Meeting # 97-e	R4-2016240

# Electronic Meeting,  2 - 13 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | CRNum | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133: Adding L1-SINR accuracy requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Perf |  |  |  |  |  | Date: |  |  | 2-11-2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Following the approval of the L1-SINR measurement requirements, the L1-SINR accuracy requirements need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR provides the text proposal for L1-SINR absolute accuracy requirements for FR1 and FR2, which is in accordance with the agreed WF (R4-1915850) and simulation results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The technical specification is incomplete because L1-SINR measurement requirements are defined in TS 38.133. In addition, measurement accuracy of UE cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 10.1.x	L1-SINR accuracy requirements for FR1

#### 10.1.x.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured

##### 10.1.x.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-SINR in this clause apply to all CSI-RS resources configured as CMR and no dedicated resource configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements in Table 10.1.x.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	The bandwidth of CSI-RS as CMR is 48 PRBs and the density is 3.

-	An AWGN channel.

The performance with larger bandwidth of CSI-RS as CMR is equal to or better than the accuracy requirements in Table 10.1.x.1.1-1.

#### Table 10.1.x.1.1-1: L1-SINR absolute accuracy for CSI-RS based CMR only in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition | CSI-RS CMR Ês/Iot |  | IMR | Io Note 1 range |  |  |  |  |
|  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB |  | dB | dB |  | dB |  | dBm / SCSCSI-RS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz |  |  |
|  |  |  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | [-121] | [-118] | N/A | [-70] |
|  |  |  |  |  |  | NR_FDD_FR1_B | [-120.5] | [-117.5] | N/A | [-70] |
|  |  |  |  |  |  | NR_TDD_FR1_C | [-120] | [-117] | N/A | [-70] |
| TBD |  | TBD | -3 |  | N/A | NR_FDD_FR1_D, NR_TDD_FR1_D | [-119.5] | [-116.5] | N/A | [-70] |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | [-119] | [-116] | N/A | [-70] |
|  |  |  |  |  |  | NR_FDD_FR1_F | [-118.5 | [-115.5] | N/A | [-70] |
|  |  |  |  |  |  | NR_FDD_FR1_G | [-118] | [-115] | N/A | [-70] |
|  |  |  |  |  |  | NR_FDD_FR1_H | [-117.5] | [-114.5] | N/A | [-70] |
| TBD |  | TBD | -3 |  | N/A | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | [-70] | [-50] |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

#### 10.1.x.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured

##### 10.1.x.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-SINR in this clause apply to all SSBs configured as CMR and dedicated resources configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements are defined in Table 10.1.x.2.1-1 for SSB based CMR and NZP-IMR and in Table 10.1.x.2.1-2 for SSB based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.x.2.1-1 and 10.1.x.2.1-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	An AWGN channel.

The performance with larger bandwidth of NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.x.2.1-1 and 10.1.x.2.1-2.

Table 10.1.x.2.1-1: L1-SINR absolute accuracy for SSB based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | NZP-IMR Ês/Iot |  | Io Note 1 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | [-121] | [-118] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_B | [-120.5] | [-117.5] | N/A | [-70] |
|  |  |  |  |  | NR_TDD_FR1_C | [-120] | [-117] | N/A | [-70] |
| TBD | TBD | 0 |  | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | [-119.5] | [-116.5] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | [-119] | [-116] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_F | [-118.5] | [-115.5] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_G | [-118] | [-115] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_H | [-117.5] | [-114.5] | N/A | [-70] |
| TBD | TBD | 0 |  | 0 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, | N/A | N/A | [-70] | [-50] |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.x.2.1-2: L1-SINR absolute accuracy for SSB based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | ZP-IMR Ês/Iot |  | Io Note 1 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | [-121] | [-118] | [N/A] | [-70] |
|  |  |  |  |  | NR_FDD_FR1_B | [-120.5] | [-117.5] | N/A | [-70] |
|  |  |  |  |  | NR_TDD_FR1_C | [-120] | [-117] | N/A | [-70] |
| TBD | TBD | -3 |  | N/A | NR_FDD_FR1_D, NR_TDD_FR1_D | [-119.5] | [-116.5] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | [-119] | [-116] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_F | [-118.5] | [-115.5] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_G | [-118] | [-115] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_H | [-117.5] | [-114.5] | N/A | [-70] |
| TBD | TBD | -3 |  | N/A | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, | N/A | N/A | [-70] | [-50] |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

#### 10.1.x.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured

##### 10.1.x.3.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-SINR in this clause apply to all CSI-RS resources configured as CMR and dedicated resources configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements are defined in Table 10.1.x.3.1-1 for CSI-RS based CMR and NZP-IMR and in Table 10.1.x.3.1-2 for CSI-RS based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.x.3.1-1 and 10.1.x.3.1-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	The bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	An AWGN channel.

The performance with larger bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.x.3.1-1 and 10.1.x.3.1-2.

Table 10.1.x.3.1-1: L1-SINR absolute accuracy for CSI-RS based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR | NZP-IMR Ês/Iot |  | Io Note 1 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dB |  | dBm / SCSCSI-RS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz |  |  |
|  |  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | [-121] | [-118] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_B | [-120.5] | [-117.5] | N/A | [-70] |
|  |  |  |  |  | NR_TDD_FR1_C | [-120] | [-117] | N/A | [-70] |
| TBD | TBD | 0 |  | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | [-119.5] | [-116.5] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | [-119] | [-116] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_F | [-118.5] | [-115.5] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_G | [-118] | [-115] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_H | [-117.5] | [-114.5] | N/A | [-70] |
| TBD | TBD | 0 |  | 0 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | [-70] | [-50] |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.x.3.1-2: L1-SINR absolute accuracy for CSI-RS based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR | ZP-IMR Ês/Iot |  | Io Note 1 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dB |  | dBm / SCSCSI-RS |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz |  |  |
|  |  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | [-121] | [-118] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_B | [-120.5] | [-117.5] | N/A | [-70] |
|  |  |  |  |  | NR_TDD_FR1_C | [-120] | [-117] | N/A | [-70] |
| TBD | TBD | -3 |  | N/A | NR_FDD_FR1_D, NR_TDD_FR1_D | [-119.5] | [-116.5] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | [-119] | [-116] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_F | [-118.5] | [-115.5] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_G | [-118] | [-115] | N/A | [-70] |
|  |  |  |  |  | NR_FDD_FR1_H | [-117.5] | [-114.5] | N/A | [-70] |
| TBD | TBD | -3 |  | N/A | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | [-70] | [-50] |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

<End of Change 1>

<Start of Change 2>

### 10.1.y	L1-SINR accuracy requirements for FR2

#### 10.1.y.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured

##### 10.1.y.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-SINR in this clause apply to all CSI-RS resources configured as CMR and no dedicated resource configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements in Table 10.1.y.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	The bandwidth of CSI-RS as CMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	An AWGN channel.

The performance with larger bandwidth of CSI-RS as CMR is equal to or better than the accuracy requirements in Table 10.1.y.1.1-1.

Table 10.1.y.1.1-1: L1-SINR absolute accuracy for CSI-RS based CMR only in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | IMR Ês/Iot | Io Note 1 range |  |  |
|  |  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dB | dBm / SCSCSI-RS Note 2 | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 120kHz |  |  |
| TBD | TBD | ≥-3 | N/A | TBD | N/A | [-70] |
| TBD | TBD | ≥-3 | N/A | N/A | [-70] | [-50] |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

#### 10.1.y.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured

##### 10.1.y.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-SINR in this clause apply to all SSBs configured as CMR and dedicated resources configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements are defined in Table 10.1.y.2.1-1 for SSB based CMR and NZP-IMR and in Table 10.1.y.2.1-2 for SSB based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.y.2.1-1 and 10.1.y.2.1-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	An AWGN channel.

The performance with larger bandwidth of NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.y.2.1-1 and 10.1.y.2.1-2.

Table 10.1.y.2.1-1: L1-SINR absolute accuracy for SSB based CMR and NZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB CMR Ês/Iot | NZP-IMR Ês/Iot | Io Note 1 range |  |  |
|  |  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dB | dBm / SCSSSB Note 2 | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120kHz |  |  |
| TBD | TBD | ≥0 | ≥0 | TBD | N/A | [-70] |
| TBD | TBD | ≥0 | ≥0 | N/A | [-70] | [-50] |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

Table 10.1.y.2.1-2: L1-SINR absolute accuracy for SSB based CMR and ZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB CMR Ês/Iot | ZP-IMR Ês/Iot | Io Note 1 range |  |  |
|  |  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dB | dBm / SCSSSB Note 2 | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120kHz |  |  |
| TBD | TBD | ≥-3 | N/A | TBD | N/A | [-70] |
| TBD | TBD | ≥-3 | N/A | N/A | [-70] | [-50] |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

#### 10.1.y.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured

##### 10.1.y.3.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-SINR in this clause apply to all CSI-RS resources as CMR and dedicated resources configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements are defined in Table 10.1.y.3.1-1 for CSI-RS based CMR and NZP-IMR and in Table 10.1.y.3.1-2 for CSI-RS based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.y.3.1-1 and 10.1.y.3.1-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	The bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

-	An AWGN channel.

The performance with larger bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 10.1.y.3.1-1 and 10.1.y.3.1-2.

Table 10.1.y.3.1-1: L1-SINR absolute accuracy for CSI-RS based CMR and NZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | NZP-IMR Ês/Iot | Io Note 1 range |  |  |
|  |  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dB | dBm / SCSCSI-RS Note 2 | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 120kHz |  |  |
| ±6.5 | ±9.5 | ≥0 | ≥0 | TBD | N/A | [-70] |
| 8.5 | 11.5 | ≥0 | ≥0 | N/A | [-70] | [-50] |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

Table 10.1.y.3.1-2: L1-SINR absolute accuracy for CSI-RS based CMR and ZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | ZP-IMR Ês/Iot | Io Note 1 range |  |  |
|  |  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dB | dBm / SCSCSI-RS Note 2 | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 120kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | N/A | TBD | N/A | [-70] |
| 8.5 | 11.5 | ≥-3 | N/A | N/A | [-70] | [-50] |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

<End of Change 2>