3GPP TSG-RAN4 Meeting # 97-e	R4-2016263

# Electronic Meeting, 2nd – 13th November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.809 | CR | <CR#> | rev | 0 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Tx signal quality related requirements chapter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  | Date: |  |  | 2020-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Weaken the frequency error requriement reasoning, there are different synchronization implementation depending on different architecture design. |  |  |  |  |  |  |  |  |
| Summary of change: |  | weaken the statement of the frequency error requriement reasoning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only one implementation is allowed from reading of TR text which should be corrected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.1, 9.6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < start of changes >

#### 7.5.2.1	Frequency error

As IAB-MT function is more like a UE, UE frequency error correction can be a reference. When BS transmits DL signal to UE, UE does frequency error correction algorithm to make sure UE follows BS with a relative low residual frequency error.The residual frequency error after compensation should be less than one percent of the subcarrier interval.

If 15KHz SCS is used, one percent is 150 Hz which is 0.1 ppm for 1.5 GHz and less than 0.1 ppm of the higher carrier frequency. For higher modulation such as 256 QAM, the residual frequency error should be much smaller. Therefore, in order to support high modulation, UE frequency offset correction algorithm should make the residual frequency error less than +/- 0.1ppm. The UE frequency offset correction algorithm follows BS carrier frequency through DL signals that's why the UE frequency error should be defined as relative frequency error not absolute frequency error.

When IAB-MT receives IAB-DU DL signal, the similar frequency error correction algorithm could be used to make high modulation support possible. IAB-MT frequency error requirement could reuse UE requirement which is defined as relative requirement as +/-0.1 PPM relative to received signal from parent node.

Therefore, IAB-MT frequency error requirement is defined to reuse UE requirements as ± 0.1 PPM compared to the carrier frequency received from the parent node.

## << Unchanged part is omitted>>

#### 9.6.2.1	Frequency error

IAB-MT OTA transmitted signal quality requirement analysis is the same as the conducted requirements in 7.5.2.1 that IAB-MT could follow parent node's center frequency. IAB-MT type 1-O and type 2-O OTA frequency error reuses UE requirements to be +/-0.1 PPM relative to received signal from parent node.

## < end of changes >