3GPP TSG-RAN WG4 Meeting #97-e	R4-2016289

Electronic Meeting, 2 – 13 November 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussions on TRP procedures

Agenda item:	4.5.4

Document for:	Discussion

# 1	Introduction

This contribution discusses the open issue related two TRP measurement procedures, namely two orthogonal cuts with pattern multiplication and beam-based directions. This is in accordance with the RAN4 meeting guidelines [1] in which there is a maximum number of one discussion paper per agenda item per company.

The two orthogonal cuts with pattern multiplication procedure is specified in TR 37.941 [2] and TS 38.141-2 [3]. The procedure is used to test the OTA BS output power, SEM and OBUE requirements. A shortcoming is identified in the specification of the two orthogonal cut procedure, which makes it not directly usable for computing TRP estimates from a set of discrete data. This document highlights the shortcoming, aiming to initiate discussion.

The beam-based directions procedure is also specified in TR 37.941 [2] and TS 38.141-2 [3]. The open issue with the beam-based directions procedure is its applicability to compute TRP estimate for in-band unwanted emissions. [5] outlined an approach to determine whether the radiation pattern of in-band unwanted emissions is correlated with that of the wanted signal. To address the feedback concerning the approach at the last meeting, we present comprehensive simulation results that are used to validate the proposed approach using the simulation model in [4].

# 2	Discussion

## 2.1	Two orthogonal cuts with pattern multiplication

According to Sections 6.3.2.2.2 and 6.3.2.3.2 in TR 37.941 [1], the procedure for the two orthogonal cuts with pattern multiplication is as follows:

Use this method when the antenna symmetries are compatible with pattern multiplication, see clause 6.3.2.5. Following steps are performed during the measurement:

1.	Calculate the reference angular steps as described in clause 6.3.2.1.

2.	Align the BS to allow for proper pattern multiplication. Measure EIRP on two orthogonal cuts with steps smaller or equal to the reference steps according to step 1.

3.	Apply pattern multiplication to extrapolate the two cuts data to full-sphere.

4.	Apply numerical integration to obtain the TRP estimate.

Referring to Step 4 above, numerical integration should be applied to compute the TRP estimate from the set of discrete data obtained through extrapolation in Step 3. However, the formula provided in Section 6.3.4.5 for computing TRP estimate is as follows:

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

The above integral is not in a numerical form where it can be directly applied to compute the TRP estimate from the discrete data.

Observation 1: The TRP formula for the two orthogonal cuts with pattern multiplication requires continuous data points, which is not suitable to integrate a set of discrete data samples.

In comparison with the other grid-based procedures in Section 6.4.4, TR 37.941, the expression for computing TRP estimate is defined in a numerical form as summations of discrete data that is measured in a test chamber. For example, the formula for the spherical equal angle and spherical equal area grids is defined as

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn and TRPEstimate=1Nn=1NEIRP(θn,ϕn), respectively.

Consequently, a numerical approximation of the TRP integral for the two orthogonal cuts with pattern multiplication is needed.

Proposal 1: A numerical form of the TRP integral for the two orthogonal cuts with pattern multiplication is defined to allow computation of TRP estimate from discrete data samples.

## 2.2	Beam-based directions

### 2.2.1	Background and summary

Signal correlation model

From [4], the radiation power pattern R(θ,ϕ) produced by a uniform linear array of N elements can be expressed as

Rθ,ϕ=E(θ,ϕ)2n=1Nwnvn2 										(1)

where E is the element factor, wn is the excitation vector given in Equation (2) and vn is the element location phasor vector given in Equation (3).

vn=exp⁡(j2πfc0(sinθcosϕ xn+sinϕyn+cosθ zn)) 				(2)

wn=ρ+1-ρ2(xn+jyn) 												(3)

Observation 1: xn and yn in Equation (3) are independent Gaussian random values, while xn, yn and zn in Equation (2) provides the position of each array element in the Cartesian coordinate system. To avoid confusion, a different symbol could be used to represent the Gaussian random value.

Question 1: Is the correlation model assumed by Equation (3) realistic or only valid for some AAS BS implementation variants?

Radiation pattern correlation

Based on the observations made in [5], we concluded that correlation between wanted and in-band unwanted emissions exists if the following 4 criteria are met:

- Maximum radiation of unwanted emissions occurs in the same direction as the wanted signal.

- The main lobe of the wanted signal and the unwanted emissions with respect to the axis of maximum radiation should have the same symmetry.

- HPBW in the azimuth and elevation direction for the unwanted emissions should correspond to those of the wanted signal.

- The directivity-beamwidth product of the unwanted emissions should correspond to that for the wanted signal.

Using the above criteria, 5 measurement points were derived in [5], which are measured in the radiation pattern of the unwanted signal:

- Measure EIRP(θ,ϕ), where (θ,ϕ) corresponds to the direction of maximum EIRP for the wanted signal.

- Measure EIRP(θ1) and EIRP(θ2) in the azimuth (θ) plane, where θ1 and θ2 correspond to the direction which is derived from the half-power angles of the wanted signal.

- Measure EIRP(ϕ1) and EIRP(ϕ2) the elevation (ϕ) plane, where ϕ1 and ϕ2 correspond to the direction which is derived from the half-power angles of the wanted signal.

Before the above measurements can be carried out, the direction of maximum radiation and half-power levels of the wanted signal must be known.

### 2.2.2	Evaluations

In this subsection, we evaluate the validity of the proposed criteria using radiation patterns generated using the simulation model in [4]. A similar simulation setup to [4] was used, consisting of an 8×8 uniform rectangular array. The array element spacing was set to 0.5λ, where the operating frequency was set to fo = 3.5 GHz. Note in [4], the same frequency was used for all ρ values. Even though this assumption is not realistic, but it simplifies the analysis.

Following the same way as in [4], the radiation intensity is used in our analysis instead of EIRP. U(ρ,θ,ϕ) denotes radiation intensity for ρ at angles θ and ϕ.

Case 1: ρ=1

Substituting ρ=1 into Equation (3), we get

wn=1 for all n.

Observation 2: wn has only a real part which is constant.

Thus, Equation (1) can be simplified as

Rθ,ϕ=E(θ,ϕ)2n=1Nvn2

Figure 1 shows the radiation pattern in the θ and ϕ directions, which is made at ϕ = 0º and θ = 90º, respectively.

Figure 1: Radiation patterns for ρ=1

As outlined in subsection 2.2.1, the radiation intensity U(1,θ,ϕ) for the following 5 measurement points are determined:

- The direction of maximum radiation intensity Umax1,90°,0°= 24.08 dB/sr occurs at θmax = 90º and ϕmax = 0º.

- In the θ plane, the half-power radiation intensity Uhp1,θ(1,83.6°,0°) = Uhp2,θ(1,96.4°,0°) = 21.07 dB/sr occurs at θ1 = 83.6º and θ2 = 96.4º.

- In the ϕ plane, the half-power radiation intensity Uhp1,ϕ(1,90°,-6.4°) = Uhp2,ϕ(1,90°,6.4°) = 21.07 dB/sr occurs at ϕ1 = -6.4º and ϕ2 = 6.4º.

The directions obtained in 1, 2 and 3 above are summarized in Table 1, which correspond to the wanted signal since ρ=1.

Table 1: Summary of radiation intensity at different angles for wanted signal

| Direction | Remark |
| --- | --- |
| θmax = 90º and ϕmax = 0º | Direction of maximum radiation intensity |
| θ1 = 83.6º and θ2 = 96.4º | Half-power angles in the θ plane |
| ϕ1 = -6.4º and ϕ2 = 6.4º | Half-power angles in the ϕ plane |

The directivity for ρ = 1 is D(ρ=1) = 22.98 dB.

For ρ=1, the unwanted signals generated in different transceiver paths are identical. As a result, the radiation pattern of the unwanted signal is identical to that of the wanted signal. As such, all the criteria (a)-(d) in subsection 2.1 are obviously met.

Observation 3: When ρ=1, the radiation pattern of the unwanted signal is identical to the wanted signal. As a result, the beam-based directions procedure can be applied without causing TRP estimation errors.

Case 2: ρ=0

Substituting ρ=0 into Equation (3), we get

wn=12xn+jyn

Observation 4: wn is a random complex number.

Based on the above observation, the radiation pattern (see Figure 2) is uncorrelated with that of the wanted signal (Figure 1). The radiation pattern in the θ and ϕ plane is made at ϕ = 0º and θ = 90º, respectively.

Figure 2: Radiation patterns for ρ=0

The radiation intensity U(0,θ,ϕ) is determined for the 5 measurement points using the direction in Table 1 according to subsection 2.1:

- The radiation intensity at (θmax = 90º and ϕmax = 0º) is

U(0,90°,0°) = 10.10 dB/sr.

- The radiation intensity at (θ1 = 83.6º and θ2 = 96.4º) in the θ plane is

Uhp1,θ(0,83.6°,0°) = 9.57 dB/sr,	 		Uhp2,θ(0,96.4°,0°) = 4.87 dB/sr

- The radiation intensity at (ϕ1 = -6.4º and ϕ2 = 6.4º) in the ϕ plane is

Uhp1,ϕ(0,90°,-6.4°) = 7.9 dB/sr,		Uhp2,ϕ(0,90°,6.4°) = 9.49 dB/sr

The directivity is D(ρ=0) = 11.22 dB.

To determine if criteria (a)-(c) are met, it is sufficient to compute the following from the above radiation intensity:

- The difference between the radiation intensity using 1, 2, and 3 above as follows:

In the θ plane:

U0,90°,0°-Uhp1,θ0,83.6°,0°=10.10-9.57=0.53 dB<3.0 dB (for ρ=1)

U0,90°,0°-Uhp2,θ0,96.4°,0°=10.10-4.87=5.23 dB>3.0 dB (for ρ=1)

In the ϕ plane:

U0,90°,0°-Uhp1,ϕ0,90°,-6.4°=10.10-7.9=2.2 dB<3.0 dB (for ρ=1)

U0,90°,0°-Uhp2,ϕ0,90°,6.4°=10.10-9.49=0.61 dB<3.0 dB (for ρ=1)

The difference is either smaller or greater than 3.0 dB in the θ and ϕ planes, which does not correspond to 3 dB for the correlated case (ρ = 1).

- The absolute difference between radiation intensity using 2 and 3 above as follows:

In the θ plane:

Uhp1,θ0,83.6°,0°-Uhp2,θ0,96.4°,0°=9.57-4.87=4.70 dB>0 dB (for ρ = 1)

In the ϕ plane:

Uhp1,ϕ0,90°,-6.4°-Uhp2,ϕ0,90°,6.4°=7.9-9.49=1.59 dB>0 dB (for ρ = 1)

The absolute difference is greater than 0 dB in the θ and ϕ planes, which does not correspond to 0 dB for the correlated case (ρ = 1). It is also obviously from the radiation intensity plots in Figure 2 that the maximum radiation intensity does not occur at θmax = 90º and ϕmax = 0º. The main lobe of ρ = 1 and ρ = 0 with respect to the axis of maximum radiation (θmax = 90º and ϕmax = 0º) do not have the same symmetry and the HPBW in the θ and ϕ planes for ρ = 0 does correspond to those for ρ = 1.

Observation 5: For ρ = 0, it can be concluded that criteria (a)-(c) are not met based on the above analysis, which implies criterion (d) is also not met.

Case 3: ρ = 0.9

Substituting ρ=0.9 into Equation (3), we get

wn=310+510xn+j510yn

Observation 6: wn is a random complex number but the real part is composed of a constant 310 and a random number.

Figure 3 shows the radiation pattern in the θ and ϕ directions. The radiation pattern in the θ and ϕ planes is cut at ϕ = 0º and θ = 90º, respectively.

Figure 3: Radiation patterns for ρ=0.9

The radiation intensity U(0.9, θ,ϕ) is determined for the 5 measurement points using the direction in Table 1 according to subsection 2.1:

- The radiation intensity at (θmax = 90º and ϕmax = 0º) is

U(0.9,90°,0°) = 23.63 dB/sr

- The radiation intensity at (θ1 = 83.6º and θ2 = 96.4º) in the θ plane is

Uhp1,θ(0.9,83.6°,0°) = 20.32 dB/sr,	 		Uhp2,θ(0.9,96.4°,0°) = 21.06 dB/sr

- The radiation intensity at (ϕ1 = -6.4º and ϕ2 = 6.4º) in the ϕ plane is

Uhp1,ϕ(0.9,90°,-6.4°) = 20.40 dB/sr,		Uhp2,ϕ(0.9,90°,6.4°) = 21.13 dB/sr

The directivity is D(ρ=0.9) = 22.53 dB.

To determine if criteria (a)-(c) are met, it is sufficient to compute the following from the above radiation intensity:

- The difference between the radiation intensity using 1, 2 and 3 above as follows:

In the θ plane:

U0.9,90°,0°-Uhp1,θ0.9,83.6°,0°=23.63-20.32=3.31 dB>3.0 dB (for ρ=1)

U0.9,90°,0°-Uhp2,θ0.9,96.4°,0°=23.63-21.06=2.57 dB<3.0 dB (for ρ=1)

In the ϕ plane:

U0.9,90°,0°-Uhp1,ϕ0.9,90°,-6.4°=23.63-20.40=3.23 dB>3.0 dB for ρ = 1

U0.9,90°,0°-Uhp2,ϕ0.9,90°,6.4°=23.63-21.13=2.5 dB<3.0 dB ( for ρ = 1)

The difference is either smaller or greater than 3 dB in the θ and ϕ planes, which does not correspond to 3 dB for the correlated case (ρ = 1). However, the difference is relatively smaller as compared with the case ρ = 0.

- The absolute difference in radiation intensity using 2 and 3 above as follows:

In the θ plane:

Uhp1,θ0.9,83.6°,0°-Uhp2,θ0.9,96.4°,0°=20.32-21.06=0.74 dB>0 dB (for ρ = 1)

In the ϕ plane:

Uhp1,ϕ0.9,90°,-6.4°-Uhp2,ϕ0.9,90°,6.4°=20.40-21.13=0.73 dB>0 dB (for ρ = 1)

The absolute difference is greater than 0 dB in the θ and ϕ planes, which does not correspond to 0 dB for the correlated case (ρ = 1). However, the difference is relatively smaller as compared with the case ρ = 0 because the axis of maximum radiation is the same as for ρ = 1.

Observation 7: For ρ = 0.9, it can be concluded that criteria (b)-(c) are not met based on the above analysis, which implies criterion (d) is also not met. Consequently, the beam-based directions procedure cannot be used for computing TRP estimate.

Case 4: ρ = 0.4

Substituting ρ=0.4 into Equation (3), we get

wn=210+310xn+j310yn

Observation 8: wn is a random complex number but the real part is composed of constant 210 and a random number similar to the case ρ = 0.9. However, the constant is smaller as ρ decreases.

Figure 4 shows the radiation pattern in the θ and ϕ directions. The radiation pattern in the θ and ϕ planes is cut at ϕ = 0º and θ = 90º, respectively.

Figure 4: Radiation patterns for ρ=0.4

The radiation intensity U(0.4, θ,ϕ) is determined for the 5 measurement points using the direction in Table according to subsection 2.1:

- The radiation intensity at (θmax = 90º and ϕmax = 0º) is

U(0.4,90°,0°) = 20.25 dB/sr

- The radiation intensity at (θ1 = 83.6º and θ2 = 96.4º) in the θ plane is

Uhp1,θ(0.4,83.6°,0°) = 16.14 dB/sr,	 		Uhp2,θ(0.4,96.4°,0°) = 18.27 dB/sr

- The radiation intensity at (ϕ1 = -6.4º and ϕ2 = 6.4º) in the ϕ plane is

Uhp1,ϕ(0.4,90°,-6.4°) = 17.18 dB/sr,		Uhp2,ϕ(0.4,90°,6.4°) = 18.35 dB/sr

The directivity is D(ρ=0.4) = 19.16 dB.

To determine if criteria (a)-(c) are met, the following is computed from the above radiation intensity:

- The difference between the radiation intensity using 1, 2 and 3 above as follows:

In the θ plane:

U0.4,90°,0°-Uhp1,θ0.4,83.6°,0°=20.25-16.14=4.11 dB>3.0 dB (ρ = 1)

U0.4,90°,0°-Uhp2,θ0.4,96.4°,0°=20.25-18.27=1.98 dB<3.0 dB (ρ = 1)

In the ϕ plane:

U0.4,90°,0°-Uhp1,ϕ0.4,90°,-6.4°=20.25-17.18=3.07 dB>3.0 dB ρ = 1

U0.4,90°,0°-Uhp2,ϕ0.4,90°,6.4°=20.25-18.35=1.9 dB<3.0 dB (ρ = 1)

The difference is either smaller or greater than 3.0 dB in the θ and ϕ planes, which does not correspond to 3 dB for the correlated case.

- The absolute difference in radiation intensity using 2 and 3 above as follows:

In the θ plane:

Uhp1,θ0.4,83.6°,0°-Uhp2,θ0.4,96.4°,0°=16.14-18.27=2.13 dB>0 dB (for ρ = 1)

In the ϕ plane:

Uhp1,ϕ0.4,90°,-6.4°-Uhp2,ϕ0.4,90°,6.4°=17.18-18.35=1.17 dB>0 dB (for ρ = 1)

The absolute difference is greater than 0 dB in the θ and ϕ planes, which does not correspond to 0 dB for the correlated case (ρ = 1). However, the difference relatively smaller than the case ρ = 0 but greater than ρ = 0.9. This could due to the axis of maximum radiation which is slightly shifted as depicted in Figure 4.

Observation 9: For ρ = 0.4, it can be concluded that criteria (b)-(c) are not met based on the above analysis, which implies criterion (d) is also not met. Consequently, the beam-based directions procedure cannot be used for computing TRP estimate.

The simulations results demonstrate the feasibility of such an approach to determine if correlation exists between the in-band unwanted and wanted emissions. Thus, it is proposed to add the criteria for determining if correlation exists to the TR.

Proposal 2: Criteria for determining whether correlation exists before applying the beam-based directions procedure should be added to the TR 37.941 as background information.

# 3	Conclusions

## 3.1	Two orthogonal cuts with pattern multiplication

The paper has identified a shortcoming with the current equation for the two orthogonal cuts with pattern multiplication for computing TRP estimates. The following observation has been made:

Observation 1: The TRP formula for the two orthogonal cuts with pattern multiplication requires continuous data points, which is not suitable to integrate a set of discrete data samples.

Therefore, the following is proposed

Proposal 1: A numerical form of the TRP integral for the two orthogonal cuts with pattern multiplication is defined to allow computation of TRP estimate from discrete data samples.

## 3.2	Beam-based directions

Simulation results for four different correlation levels (namely, 100% correlation, i.e., ρ = 1, high correlation, i.e., ρ = 0.9, low correlation, i.e., ρ = 0.4 and 0% correlation, i.e., ρ = 0) have been presented. The following observations have been made:

Observation 1: xn and yn in Equation (3) are independent Gaussian random values, while xn, yn and zn in Equation (2) provides the position of each array element in the Cartesian coordinate system. To avoid confusion, a different symbol could be used to represent the Gaussian random value.

Question 1: Is the correlation model assumed by Equation (3) realistic or only valid for some AAS BS implementation variants?

Observation 2: wn has only a real part which is constant.

Observation 3: When ρ=1, the radiation pattern of the unwanted signal is identical to the wanted signal. As a result, the beam-based directions procedure can be applied without causing TRP estimation errors.

Observation 4: wn is a random complex number.

Observation 5: For ρ = 0, it can be concluded that criteria (a)-(c) are not met based on the above analysis, which implies criterion (d) is also not met.

Observation 6: wn is a random complex number but the real part is composed of a constant 310 and a random number.

Observation 7: For ρ = 0.9, it can be concluded that criteria (b)-(c) are not met based on the above analysis, which implies criterion (d) is also not met. Consequently, the beam-based directions procedure cannot be used for computing TRP estimate.

Observation 8: wn is a random complex number but the real part is composed of constant 210 and a random number similar to the case ρ = 0.9. However, the constant is smaller as ρ decreases.

Observation 9: For ρ = 0.4, it can be concluded that criteria (b)-(c) are not met based on the above analysis, which implies criterion (d) is also not met. Consequently, the beam-based directions procedure cannot be used for computing TRP estimate.

The simulations results have demonstrated the feasibility of such an approach to determine if correlation exists between the in-band unwanted and wanted emissions. Thus, it is proposed the following:

Proposal 2: Criteria for determining whether correlation exists before applying the beam-based directions procedure should be added to the TR 37.941 as background information, which are as follows:

- Maximum radiation of unwanted emissions occurs in the same direction as the wanted signal.

- The main lobe of the wanted signal and the unwanted emissions with respect to the axis of maximum radiation should have the same symmetry.

- HPBW in the azimuth and elevation direction for the unwanted emissions should correspond to those of the wanted signal.

- The directivity-beamwidth product of the unwanted emissions should correspond to that for the wanted signal.

Views from other companies are welcome for the above two related topics and open issues.

# References

- RAN4 leadership, RAN4 Meeting efficiency improvements,

- TR 37.941, Radio frequency (RF) conformance testing background for radiated base station (BS) requirements

- TS 38.141-2, Base Station (BS) conformance testing Part 2: Radiated conformance testing

- R4-1914521, On correlation and beamwidth, Ericsson

- R4-1912483, Further discussion on beam-based directions for TRP estimates of in-band unwanted emissions, Nokia, Nokia Shanghai Bell