3GPP RAN WG4 Meeting #97e		R4-2016296

Electronic meeting, 2nd – 13th November 2020

Agenda item:	10.28.1

Source:	Apple Inc.

Title:	Peak EIRP and Peak EIS for band n262

WI/SI:	NR_47GHz_Band-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The work item [1] agreed on the introduction of NR 47 GHz (47.2 to 48.2 GHz). As part of the framework it was agreed to study peak EIRP and peak EIS budget for n262. In this contribution we have estimated the peak EIRP and peak EIS considering the key parameters for the transmitter and receiver, we have as well provided a brief explanation on the challenges for the support of multi-band in FR2 including n262.

# 2	Discussion

For the estimation of the peak EIRP and peak EIS we need to consider the impact on the TX and Rx component when supporting a higher carrier frequency such as 47 GHz. In the transmitter, the goal of the PA is to provide as much output power as possible, while keeping a high efficiency and good linearity. All these parameters become more difficult to achieve at higher frequencies, as the frequency increases the PA will present larger distortion and efficiency degradation. In the receiver, the LNA is a key component since its NF dominates the overall noise performance of the receiver. The LNA should provide a wideband impedance matching to the antenna, gain, good linearity and low noise figure (NF). These parameters become more challenging when supporting higher carrier frequency.

## 2.1	Peak EIRP for n262

Collecting all the parameters for the peak EIRP, we have estimated the minimum peak EIRP for n262 is 12,7 dBm.

| Parameter | Unit | Proposal |  |
| --- | --- | --- | --- |
|  |  | Nominal | Tolerance |
| Band number |  | n262 | n262 |
| Frequency range | GHz | 47.2 - 48.2 | 47.2 - 48.2 |
| Pout per element | dBm | 9,5 |  |
| # of antennas in an array |  | 4 |  |
| Total conducted power per polarization | dBm | 16 | -2 |
| Average antenna element gain | dBi | 3,75 |  |
| Antenna gain roll-off loss versus frequency | dB | -1,5 |  |
| Realized antenna array gain | dBi | 8,3 | 0 |
| Polarization gain | dB | 2,8 |  |
| Mismatch and transmission line loss including load pull | dB | -2,6 | -0,7 |
| Beamforming loss (phase shifter and amplitude error) | dB | -2 | -0,6 |
| Form factor integration losses | dB | -4 | -2 |
| Total implementation loss (nominal) | dB | -8,6 | -3,3 |
| Total implementation loss (worst case) | dB |  |  |
| Peak EIRP (Nominal) | dBm | 18,0 |  |
| Tolerance (+/-) | dB |  | -4,3 |
| Peak EIRP (Minimum) | dBm | 12,7 |  |

Proposal 1:	The single-band minimum peak EIRP requirement for band n262 is 12.7 dBm.

## 2.2	REFSENS for n262

Collecting all the parameters for the REFSENS for n262, we have estimated the REFSENS budget -78.98 dBm/50 MHz.

| Parameter | Unit | Proposal |
| --- | --- | --- |
| Band number |  | n262 |
| Frequency range | GHz | 47.2 - 48.2 |
| SNR Requirement | dB | -1 |
| Bandwidth | MHz | 50 |
| Thermal Noise | dBm/Hz | -174 |
| Noise Figure | dB | 16 |
| # of antennas in an array |  | 4 |
| Average antenna element gain | dBi | 3,75 |
| Realized antenna array gain |  | 8,3 |
| Diversity Gain | dB | 0 |
| Antenna gain roll-off loss versus frequency | dB | -1,5 |
| Mismatch and transmission line loss |  | -3,3 |
| Beamforming Loss | dB | -2 |
| Total Insertion Loss | dB | -11,3 |
| Form factor Integration losses |  | -6 |
| REFSENS | dBm | -78,98 |

Proposal 2:	The single-band minimum peak EIS requirement for band n262 is -78.98 dBm/50 MHz, -75.98 dBm/100 MHz, -72.98 dBm/200 MHz, -69.98 dBm/400 MHz

## 2.3	Multi-band relaxation requirement for n262

For UE supporting multiple FR2 bands, the MBR requirements gives a relaxation for peak EIRP and EIRP spherical coverage. In Rel-17 the band n262 will be included in the multi-band framework, extending the previous frequency range support from 26.5 to 43.5 GHz (n257, n258, n259, n260, n261), to a new wider range from 26.5 to 48.2 GHz.

Some UE implementations are anticipated to extend the frequency coverage of their antenna solution to span bands n260, n259, and n262. For the study of the multi-band relaxation for n262, we need to consider the antenna array performance when evaluating the antenna integration of 39 GHz and 47 GHz.  In [2] we have shown in the trade-off between the space thickness and the array gain across the 28 GHz and 39 GHz bands. A similar study is needed when considering the multi-band antenna array integration for 39 GHz and 47 GHz.

The wide bandwidth will introduce new challenges to provide a high gain and directivity. In addition, the impedance matching between the antenna and the transmission line becomes more difficult at wide bandwidth. Depending on the quality of the impedance matching, the energy from the transmitter to the antenna could be reflected, which results in the reduction of the radiated energy from the antenna. These challenges shall be considered for the multi-band relaxation requirement definition at n262.

Observation 1:	Degradation of the radiated energy from the antenna due to poor impedance matching when supporting 39 GHz + 47 GHz wide band antenna is anticipated.

Proposal 3:	Study of multi-band relaxation requirement shall consider wide-band solution (39 GHz + 47 GHz).

# 3	Conclusions

This contribution provides our view on the peak EIRP and peak EIS considering the key parameters for the transmitter and receiver, as well as a brief explanation on the challenges for the support of multi-band in FR2 including n262. In summary, we have made the following observations and proposals:

Proposal 1:	The single-band minimum peak EIRP requirement for band n262 is 12.7 dBm.

Proposal 2:	The single-band minimum peak EIS requirement for band n262 is -78.98 dBm/50 MHz, -75.98 dBm/100 MHz, -72.98 dBm/200 MHz, -69.98 dBm/400 MHz

Observation 1:	Degradation of the radiated energy from the antenna due to poor impedance matching when supporting 39 GHz + 47 GHz wide band antenna is anticipated.

Proposal 3:	Study of multi-band relaxation requirement shall consider wide-band antenna solution (39 GHz + 47 GHz).

# 4	References

RP-201232, “New WID: introduction of NR 47 GHz band.”, T-Mobile USA, Dish Network, July 2020

R4-1814702, “Further views on extending FR2 power class requirements to multi-band UEs”, Apple Inc., 3GPP RAN4 Meeting #89, November 2018