3GPP TSG-RAN WG4 Meeting #97-e	R4-2016302

Electronic Meeting, 02 November – 13 November 2020

Source:	Ericsson, Telstra

Title:	TP to TR 37.717.11-21: Addition of DC configurations for DC_3_n78-n258

Agenda item:	10.7.3

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 to add DC_3_n78-n258 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## DC_3_n78-n258

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n78-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A_n78A-n258A DC_3A_n78A-n258G DC_3A_n78A-n258H DC_3A_n78A-n258I DC_3A_n78A-n258J DC_3A_n78A-n258K DC_3A_n78A-n258L DC_3A_n78A-n258M | DC_3A_n78A DC_3A_n258A | 3 | - | 920 |
|  |  | n78 | - |  |
|  |  | n258 | - |  |

### 6.x.3	Co-existence studies

Based on co-existence studies of DC_3_n78 and DC_3_n258 specified in TR 37.717-11-21, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.x.4	∆TIB and ∆RIB values

For DC_3_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n258 | 0 |

### 6.x.5	MSD

There is no additional MSD issue for DC_3_n78-n258.

##### ---End of changes---

# Reference

[1]	RP-201477, “Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics