3GPP TSG-RAN WG4 Meeting #97-e	R4-2016320

Electronic Meeting, 02 November – 13 November 2020

Source:	Ericsson, Verizon

Title:	TP to TR 37.717.11-21: Addition of DC configurations for DC_13_n48-n66

Agenda item:	10.7.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 to add DC_13_n48-n66 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_13_n48-n66

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n48-n66 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n48 | 3550 MHz | – | 3800 MHz | 3550 MHz | – | 3800 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n48A-n66A | DC_13A_n48A DC_13A_ n66A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Based on co-existence studies, 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n48.

Based on co-existence studies, 3rd order IMD generated by dual uplink of 13_n48 may fall into own Rx of band n66.

### 6.x.4	∆TIB and ∆RIB values

For DC_13_n48-n66, the TIB,c and RIB,c values are derived from DC_13-48_n66 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n48-n66 | 13 | 0.3 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n48-n66 | 13 | 0 |
|  | n48 | 0.5 |
|  | n66 | 0.2 |

### 6.x.5	MSD

As mentioned in 6.x.3, 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n48. Relaxation is given in the TR [2] and proposed in table below.

As mentioned in 6.x.3, 3rd order IMD generated by dual uplink of 13_n48 may fall into own Rx of band n66. Relaxation is derived from CA_13A-48A-66A and DC_13A-66A_n48A and proposed in table below.

Table 6.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n48A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3584 | 5 | 25 | 3584 | 2.8 | IMD5 |
|  | n66 | 1716 | 5 | 25 | 2116 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |

##### ---End of changes---

# Reference

[1]	RP-201477, “Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]	R4-2011885, “TR37.717-11-21 v0.1.0 for DC LTE x Bands DL_1UL (x=1,2,3,4) and 2 NR bands DL_1UL basket WI”, LG Electronics