3GPP TSG-RAN WG4 Meeting #97-e	R4-2016328

Electronic Meeting, 02 November – 13 November 2020

Source:	Ericsson, Verizon

Title:	TP to TR 37.717.11-21: Addition of DC configurations for DC_66_n66-n77

Agenda item:	10.7.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 to add DC_66_n66-n77 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_66_n66-n77

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n66-n77 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n66A-n77A | DC_66A_ n77A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Based on co-existence studies, 2nd and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n66.

### 6.x.4	∆TIB and ∆RIB values

For DC_66_n66-n77, the TIB,c and RIB,c values are derived from CA_n66-n77 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n66-n77 | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n66-n77 | 66 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

### 6.x.5	MSD

As mentioned in 6.x.3, 2nd and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n66. Relaxation are derived from CA_n66A-n77A and proposed in table below.

Table 6.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n66A-n77A | 66 | 1730 | 5 | 25 | 2140 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2170 | 31 | IMD2 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2140 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2170 | 5.0 | IMD5 |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | N/A |

##### ---End of changes---

# Reference

[1]	RP-201477, “Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics