3GPP TSG-RAN WG4 Meeting #97-e	R4-2016367

# Electronic Meeting, 2 – 13 November, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR | 0210 | rev |  | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.105 on NR+UTRA support for AAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When AAS BS specs were developed fully in Rel-15, there was support included for LTE and UTRA and multi-RAT operation with LTE+UTRA. GSM/EDGE was implicitly excluded. NR support was later introduced in 2018-12 (CR in R4-1808429), but only in combination with LTE. It is not explicitly stated which RATs or RAT combinations that are not covered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is added explicitly to the General section that for AAS BS, GSM/EDGE operation is not supported and that UTRA operation is only supported as single-RAT or in combination with E-UTRA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of RAT and RAT configurations is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 4	General

## 4.1	Relationship between the AAS BS specification and non-AAS BS single RAT & MSR specifications

An AAS BS is distinguished from a non-AAS BS by including a dedicated antenna system in its design.

The transceiver to antenna RF interface of the AAS BS (referred to as the transceiver array boundary) comprises one or several TAB connectors. There is no general one-to-one relationship between non-AAS BS antenna connectors and AAS BS TAB connectors, but it can be expected the number of TAB connectors in AAS BS will be greater than or equal to the number of non-AAS BS antenna connectors for respective base stations with corresponding functionality.

For the conducted requirements, the AAS BS requirements are formulated in such a way that the impact of what they are meant to regulate is expected to be "equivalent" to the corresponding non-AAS BS requirements; i.e. the outside world sees the same level of emissions, power levels, or can expect the same demodulation performance at the UE from the AAS BS as it would from a non-AAS BS with equivalent functionality. All non-AAS BS RF requirements (referenced to the non-AAS BS antenna connector) have been adapted to AAS BS conducted requirements. Many requirements are also direct references to the non-AAS BS RF specifications 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] and 3GPP TS 37.104 [5].

NOTE 1:	Applying non-AAS BS antenna connector requirements directly to individual TAB connectors renders certain AAS BS requirements excessively strict from a system perspective.

NOTE 2:	Both specific references and non-specific references occur.

In addition, some new OTA requirements are introduced. These requirements have no corresponding non-AAS BS requirements.

For a OTA AAS BS there are no conducted requirements. The radiated requirements have been derived based on the principle that they offer the same level of performance and protection as the hybrid AAS BS requirements. The radiated requirements therefore use the same equivalence to the non-AAS requirements. The non-AAS BS RF requirements have therefore been further adapted to apply to OTA metrics in the far field. Some requirements are direct references to the non-AAS BS RF specifications 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] and 3GPP TS 37.104 [5]. Some co-location requirements which have been developed from assumptions on BS-to-BS coupling do not have direct OTA equivalents. The radiated co-location requirements use the same scenarios used to develop the non-AAS RF requirements.

In this specification, the term "requirements for single RAT operation" refers to requirements that are derived from the 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], or 3GPP TS 36.104 [4] specifications baseline, whilst "requirements for MSR operation" refers to requirements derived from the 3GPP TS 37.104 [5] specification baseline (including NR operation as part of MSR).

GSM/EDGE operation is not supported by AAS BS. UTRA operation is only supported as single-RAT or in combination with E-UTRA.

NB-IoT in-band, NB-IoT guard band, or standalone NB-IoT operation is not supported by AAS BS. When referring to standalone E-UTRA requirements for single RAT operation in 3GPP TS 36.104 [4] or to E-UTRA requirements for MSR operation in 3GPP TS 37.104 [5], any requirements specified in those specifications for E-UTRA with NB-IoT (in-band or guard band) or for standalone NB-IoT, shall not be considered for the AAS BS. Unless otherwise stated, the exclusion of the NB-IoT requirements in this specification applies to all operation modes (i.e. in-band NB-IoT operation, guard band NB-IoT operation and standalone NB-IoT operation).NR requirements for single RAT operation are not specified in the present document because AAS RF requirements for single RAT NR are fully captured in the NR specification 38.104 [27].

Band 46 operation and Band 49 operation is not supported by AAS BS. When referring to standalone E-UTRA requirements for single RAT operation in 3GPP TS 36.104 [4] or to E-UTRA requirements for MSR operation in 3GPP TS 37.104 [5], any requiremetns specified in those specifications for Band 46 or Band 49 operation shall not be considered for the AAS BS with E-UTRA.