3GPP TSG-RAN WG4 Meeting #97-e	R4-2016387

Electronic Meeting, 2-13 Nov., 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On the high-speed train deployment scenario in FR2

Agenda item:	12.7.2 [NR_HST_FR2_enh-Core]

Document for:	Discussion

Introduction

In the RAN#89-e meeting, the WI on NR support for high-speed train scenario in FR2 was approved [1].

HST scenarios in more challenging higher frequency band above 3.5 GHz have not been considered in the previous 3GPP working and study items. mmWave bands can add many new spectrum resources for high-speed connections for passengers and HST special services. However, existing FR2 RRM and Demod requirements do not take into account high-speed deployments.

This contribution provides an overview of HST deployment scenarios in FR2. We collect main deployment parameters, highlight the magnitude and potential impact of the Doppler effect, and discuss channel models.

Discussion

WI background

Below we summarise the deployment-related assumptions already listed in the WI description [1]:

- UE equipment in 5G NR FR2 HST scenario is vehicle-roof mounted customer-premises equipment (CPE)

  - High-power device, power class 4

  - Single panel, i.e., only one active antenna panel at a time

- NR SA single carrier scenario in FR2

- The applicable target frequency is up to 30GHz.

  - Candidate bands are n261 (27.50 – 28.35GHz), n257 (26.50 – 29.50GHz), n258(24.25 – 27.50GHz)

- Target deployment scenario is multi-RRHs share the same cell-ID

Detailed parameters to be investigated and decided in the initial phase of the WI [1]:

- High-speed train scenario with up to 350 km/h (actual maximum supported speed to be decided within the WI)

- Number of RRHs per cell

- The distance between adjacent RRHs

- The distance between RRHs and railway track

- The number of SSB per RRH

Additionally, the following aspects of the HST FR2 scenario should be studied further [1]:

- The channel models out of two possible options:

  - HST single Tap channel

  - Uni/bi-directional SFN channel

- The Maximum Doppler frequency

HST FR2 SFN deployments

Single Frequency Network (SFN) scenario refers to a deployment planning in which the same signals are sent and received with the same frequency through multiple remote radio heads (RRH) installations covering the cell area [2]. This scenario is suitable for high-speed moving mobile environments because there is less interference from adjacent cells, and the cell coverage area is large, which results in less frequent handovers (HO).

In general, we can identify two main types of SFN deployments:

- The unidirectional SFN scenario (Option 1)

- The HST SFN with bidirectional coverage (Option 2a/b)

Following the TR [3], the intention of unidirectional SFN scenario is to provide a stable downlink carrier frequency as experienced by the UE when traveling at high speed. This can be achieved by arranging the RRHs in such a manner that the strongest signal received by the UE has a nearly constant Doppler shift without sign-alternation. A stable downlink frequency, as experienced by the UE, leads to that uplink transmissions from the same UE are received by the RRHs with a nearly constant frequency offset.

Unidirectional SFN scenario is shown in Figure 1. It is the only one considered for HST 30GHz deployment in TR [4]. In this scenario, 3 RRH are connected over the fiber to the same base-band unit (BBU).

Figure 1: Unidirectional SFN scenario.

Another significant HST SFN deployment uses bidirectional coverage, where UE receives simultaneously multiple paths of the same signal with different signs of Doppler frequency.

In SFN Option 2.a (Figure 2), the transmission scheme with two RRHs per BBU is similar to the SFN model defined in LTE, TS [5] section B.3A, and for NR HST deployment in FR1 [4]. Here, two RRHs sending the same signals are connected to one BBU. When the mobile terminal passes through RRH1 and RRH2, the signal arriving from RRH1 has a negative Doppler frequency shift effect, while the signal coming from RRH2 has a positive Doppler frequency shift effect.

Figure 2: HST SFN scenario with bidirectional coverage, 2 RRHs per BBU.

Another variation of bidirectional SFN is Option 2.b shown in Figure 3. In this deployment, six RRHs are connected to one BBU. In a traditional SFN scheme, all RRHs of the BBU have the same cell ID and are sending the same signal. However, more advanced approaches are being considered, where Doppler shift can be estimated and compensated individually for each of RRHs based on T-RSs (CSI-RS with tracking flag enabled) and DM-RSs with multiple TCI states [6].

Figure 3: HST SFN scenario with bidirectional coverage, 6 RRHs per BBU.

Unidirectional SFN deployment is a more reliable solution. It is more challenging for a UE to track the sudden Doppler frequency shift from negative to positive in bidirectional SFN deployments. Robustness can be enhanced by using multiple TCI states for the signals belonging to the same cell.

RAN4 to evaluate both unidirectional and bidirectional SFN deployments. Consider robustness enhancements in bidirectional SFN.

Deployment parameters

It is easy to evaluate the distance traveled by a UE between the consecutive SSB measurements. These values are collected in Table 1 for two train speeds: 260 kmph (~160mph) and 350kmph.

Table 1: Distance traveled by the train between consecutive SSB bursts.

| SSB burst period, ms | Speed: 260kmph | Speed: 350kmph |
| --- | --- | --- |
| 5 | 0.36m | 0.49m |
| 20 | 1.44m | 1.94m |
| 40 | 2.89m | 3.89m |

FR2 deployments usually assume that RRH sites are relatively close to the railway track, e.g., 5-10 meters away. In this case, one of the beams shall be directed almost parallel to the track. From Figure 4, it can be seen that a typical mmWave beam with a width of around 10-20 degrees can cover the major part of the track till the next RRH site. Only the distance of the order of 10s of meters next to the RRH along the track is available for the other beams.

Figure 4: Angular directions from the RRHs located 5m and 50m from the railway track.

Even for relatively mmWave narrow beams, it makes sense to consider multi-SSB deployments only when the RRH sites are more than 50 meters away from the railway track.

When RRH sites are 5-10 meters away from the railway track, it is sufficient consider only 1 SSB per RRH.

From the mobility point of view, it is necessary to evaluate if, assuming minimum UE requirements, the UE can be assumed to be capable of identifying, measuring, and reporting newly detectable cells in a timely manner to the network, such that the network can prepare the handover with the target cell and transmit the handover command to the UE in time.

It is necessary to consider minimum UE capabilities to identify, measure, and report new cells while deciding about inter RRH site distance and the number of SFN RRHs per BBU.

The main HST deployment parameters are collected in Table 2. The reference values of the parameters are presented for two scenarios previously described in [4], [7] for FR2 and in the recent NR_feMIMO summary [8].

Table 2: Main parameters of HST deployment in FR2.

| Parameter | Value [TR 38.913] | Value [R1-2007151] | Meaning |
| --- | --- | --- | --- |
| DS | 1732m | - | Distance between BBH sites |
| DR, D*R | 580m, 580m, 572m | 200m | Inter RRH site distance |
| Dmin | 5m | 5m | RRH site to railway track distance |
| hT | 3.38m | 1.5m | Height of the train (UE antenna panel height) |
| hR | - | 2.5m | Hight of the RRU antenna |
| DT | 6m | 6m | Distance between the railway tracks |
| υ | Max 500kmph | 256kmph | Train speed |
| SCS | - | 120kHz | Sub-carrier spacing |
| BW | - | 80MHz | Bandwidth |
| CF | Around 30 GHz | 30GHz | Carrier Frequency |
| φ | - | ±20 degrees | RRH panel bearing angle |
| NRRH | 3 | 3 | Number of RRH per cell |

There are few essential HST deployment parameters for which several values are potentially possible, e.g., inter RRH site distance, RRH and UE antenna height and orientation, etc.

RAN4 to evaluate two typical scenarios with longer (500-600m) and shorter(200-300m) inter RRH site distance.

Doppler effect

Doppler frequency depends on the angle θt between the velocity vector v of the moving UE and the direction of radio wave propagation between the UE and the RU

fst=fdcosθt=vcfccosθt,

where fd is the maximum Doppler shift, v is UE speed in (m/s), c is the speed of light (m/s), fc is the carrier frequency in Hz.

For the time-varying channel, as in the high-speed trains scenario, the accuracy of Doppler shift estimation depends on the number of reference (usually DM-RS) symbols distributed in the time domain. The minimum time interval ∆t between two reference symbols in one slot sets the theoretical upper limit of Doppler shift that can be accurately estimated [R4-1908282]. If UE and BS clock frequency drift is not taken into account, the theoretical maximum Doppler shift that can be estimated is given by
femax≈12∆t.

The values of femax for several intervals ∆t between reference symbols and 120kHz SCS is shown in Table 3.

Table 3: Maximum estimated Doppler shift, 120 kHz SCS.

| ∆t between reference symbols | femax |
| --- | --- |
| 1 symbol (8.92 us) | 56.05 kHz |
| 2 symbols | 28.03 kHz |
| 3 symbols | 18.68 kHz |
| 4 symbols | 14.01 kHz |
| 8 symbols | 7.00 kHz |
| 14 symbols (slot) | 4.0 kHz |

The absolute value of maximum Doppler shift fd depends on the UE speed and for 30 GHz carrier frequency equals

- 7.23 kHz for 260 kmph,

- 9.73 kHz for 350 kmph.

The Doppler shift of the signal send in UL direction is two times higher than in DL because the UE gets synchronized from the DL signals that already contain frequency offset, giving

- 14.45 kHz for 260 kmph,

- 19.46 kHz for 350 kmph.

Moreover, in HST bidirectional deployment, depending on the implementation of frequency offset evaluation (FOE) and compensation (FOC) in the gNB (e.g., both done per UE and in the baseband) and the behaviour of the UE, the worst case mismatch between the received signal and the last FOE value can be up to two times higher resulting in

- 28.91 kHz for 260 kmph,

- 38.92 kHz for 350 kmph.

As it is described in section B.3 of TS [5], the actual values of the Doppler frequency depend on the network deployment, e.g., on the distances DS. and Dmin. Nevertheless, presented analysis provides feasible coarse evaluations for HST scenario in FR2. It can be noticed that 350 kmph case is more demanding and may require a very high density of reference symbols.

The distance between consecutive DM-RS symbols shall not be above 3 to support 350kmph train speed. Hence, at least 3 DM-RS symbols per slot should be used. It is also necessary to consider the utilization of PT-RS in addition to DM-RS. Link level simulations are needed for more accurate evaluations.

RAN4 to evaluate two maximum train speeds: 260 and 350 kmph.

Channel models

We expect LoS propagation conditions for beamformed signals in FR2, and for a UE located on top of the train. Multipath fading models are not used for HST demodulation performance requirements in FR1 either.

In the UL direction, assuming that the reception is beamformed by a UE and that the reception is done by a single RRH, a single-tap channel model with a given Doppler shift profile is a feasible approach.

In the DL direction, an alternative approach would be to use a multi-tap channel model, where the number of taps depends on the number of RRH transmitting jointly the same signals. However, multi-tap modeling is considerably more complicated because the delay and frequency offset of each tap is changing continuously with the position of the train.

RAN4 to consider using a single-tap per RRH channel model for performance requirements in UL direction and both single- and multi-tap models in DL direction.

- For RRM system-level evaluations in FR1, NR RMa LoS model defined in TR [9] was used. There is a concern regarding the RMa model applicability for FR2. Following the 3GPP TR 38.901:

- The applicable frequency range of the PL formula is in the range 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios.

- It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz.

- Scenario RMa is for up to 7GHz, while others are for up to 100GHz.

- Table 7.5-6 Part-2 states that Channel model parameters for RMa (up to 7GHz) and Indoor-Office.

In the RAN1 simulation discussion for Rel-17 FeMIMO studies, there are parameters also listed for HST in FR2 [8]:

- Table 4 Baseline assumptions for SLS: Additional simulation assumptions for HST scenario (FR2), mainly from TR 38.802, e.g. Table A.2.1-2

- UMa LOS model is agreed there.

Adopt UMa LoS model from 3GPP TR 38.901 as a baseline for system-level studies of HST in FR2.

Conclusion

This contribution provides an overview of HST deployment scenarios in FR2. We collect main deployment parameters, highlight the magnitude and potential impact of the Doppler effect, and discuss channel models.
The following observations and proposals are made:

- Unidirectional SFN deployment is a more reliable solution. It is more challenging for a UE to track the sudden Doppler frequency shift from negative to positive in bidirectional SFN deployments. Robustness can be enhanced by using multiple TCI states for the signals belonging to the same cell.

- RAN4 to evaluate both unidirectional and bidirectional SFN deployments. Consider robustness enhancements in bidirectional SFN.

Even for relatively mmWave narrow beams, it makes sense to consider multi-SSB deployments only when the RRH sites are more than 50 meters away from the railway track.

When RRH sites are 5-10 meters away from the railway track, it is sufficient consider only 1 SSB per RRH.

It is necessary to consider minimum UE capabilities to identify, measure, and report new cells while deciding about inter RRH site distance and the number of SFN RRHs per BBU.

There are few essential HST deployment parameters for which several values are potentially possible, e.g., inter RRH site distance, RRH and UE antenna height and orientation, etc.

RAN4 to evaluate two typical scenarios with longer (500-600m) and shorter(200-300m) inter RRH site distance.

The distance between consecutive DM-RS symbols shall not be above 3 to support 350kmph train speed. Hence, at least 3 DM-RS symbols per slot should be used. It is also necessary to consider the utilization of PT-RS in addition to DM-RS. Link level simulations are needed for more accurate evaluations.

RAN4 to evaluate two maximum train speeds: 260 and 350 kmph.

RAN4 to consider using a single-tap per RRH channel model for performance requirements in UL direction and both single- and multi-tap models in DL direction.

Adopt UMa LoS model from 3GPP TR 38.901 as a baseline for system-level studies of HST in FR2.

- References

- RP-202037, “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia, RAN#89-e.

- T. Takada, K, Teshima, H. Harada, “Performance enhancement technologies in High-speed moving mobile environment in LTE-Advanced Release 14”, NTT DOCOMO Technical Journal, Vol. 19, No 3, Jan. 2018, pp. 64-70.

- TR 36.878, “Study on performance enhancements for high speed scenario in LTE”.

- TR 38.913, “Study on scenarios and requirements for next generation access technologies”.

- TS 36.101, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”.

- RP-192729, “Motivation for high-speed train enhancement in Rel-17 NR MIMO”, CMCC, RAN#86.

- TR 38.802, “Study on new radio access technology Physical layer aspects”.

- R1-2007151, “Moderator summary#2 for multi-beam enhancement: EVM”, RAN1#102-e.

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.