3GPP TSG-RAN WG4 Meeting#97-e	R4- 2016463

Online, 2nd Nov – 13th Nov, 2020

Source: 	Qualcomm Incorporated

Title: 	Views on Power Imbalance Tests

Agenda item:	7.16.1.3

Document for:	Discussion

# Introduction

In WF [1], a lot of the test parameters were finalized, and some issues were still open and different options were provided for open issues. In this paper, we discuss our views on open issues for power imbalance test cases.

# MCS

Based on our calculations, we observe the following:

Observation 1: Slots containing TRS have effective code rate greater than 0.95 for MCS28.

Based on above observation, peak throughput for MCS28 is lower than that of MCS27. Therefore, we propose the following:

Proposal 1: Maximum MCS for power imbalance test should be less than or equal to MCS27.

In Table 1, we present the required SNR to achieve 85% of peak throughput for FDD 1x2, MCS25, Rank1 under static channel condition, WB PRB Bundling. Requirement SNR is derived by adding 0.8dB to impairment results, similar to normal demodulation requirements for 64QAM.

Table 1: SNR to achieve 85% of peak throughput for FDD 15kHz 1x2, Rank1, MCS25 under static channel

| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Requirement SNR (dB) |
| --- | --- | --- | --- |
| 5 | 15.94 | 17.94 | 18.74 |
| 10 | 15.48 | 17.48 | 18.28 |
| 15 | 15.67 | 17.67 | 18.47 |
| 20 | 15.80 | 17.80 | 18.60 |
| 25 | 15.46 | 17.46 | 18.26 |
| 30 | 15.87 | 17.87 | 18.67 |
| 40 | 15.90 | 17.90 | 18.70 |
| 50 | 15.79 | 17.79 | 18.59 |

Based on above simulation results, we observe and propose following.

Observation 2: Requirement SNR for 64QAM MCS25 is very close to 19dB, as desired for power imbalance test cases.

Observation 3: As CBW changes, requirement SNR does not change significantly for 64QAM MCS25, Rank1.

Proposal 2: Use 64QAM MCS25, Rank1 to define the power imbalance requirements for 2Rx.

# Conclusions

This paper discusses open issues for power imbalance test cases. Following has been observed and proposed.

Observation 1: Slots containing TRS have effective code rate greater than 0.95 for MCS28.

Proposal 1: Maximum MCS for power imbalance test should be less than or equal to MCS27.

Observation 2: Requirement SNR for 64QAM MCS25 is very close to 19dB, as desired for power imbalance test cases.

Observation 3: As CBW changes, requirement SNR does not change significantly for 64QAM MCS25, Rank1.

Proposal 2: Use 64QAM MCS25, Rank1 to define the power imbalance requirements for 2Rx.

# References

[1] R4-2012691, “Way forward on UE power imbalance requirements for FR1 CA and EN-DC”, 3GPP TSG RAN WG4 Meeting #96-e, Online, Aug 17 - Aug 28, 2020.