3GPP TSG-RAN WG4 Meeting # 97-e	R4-2016465

# Electronic Meeting, 2-13 Nov., 2020

Source: 	Skyworks Solutions, Inc.

Title: 	Discussion on Single Carrier MPR versus Architecture

Agenda Item:	7.19.2.2 Power class related to UL MIMO and other related req. (MPR, SEM, etc) [TEI16 or NR_newRAT-Core]

Document for:	Approval

# Introduction

In the RAN#96 and RAN#98 e-meetings it was agreed that the work to develop MPR for Tx Diversity and UL MIMO should continue under TEI16. One aspect is related to how and where the MPR requirement should be handled. We provide here our input on how to distinguish the different MPRs and still limit the amount of tables.

# Discussion

In RAN4#96e Two CR [1, 2] were proposed to introduce single CC MPR for P2 and PC1.5, that are both based on 2 RF transmit paths and assume UL MIMO or TX Diversity. Only PC1.5 was agreed as this power class can only be supported with two TX while the other needed to be further discussed. These MPR requirements would come in addition to the single TX PC3 and PC2 MPR that have been developed very early on for NR and the recent addition [3] of single TX PC5 for NRU.

So far, it seems there is a consensus to use the same MPR table for 2 TX whether it implements TX Diversity or UL MIMO.

In order to understand how to best introduce these requirements, it is important to agree on the different cases and their behaviors. These are described in Table 1 and for now, we assume that 2 TX cases have the same power capability per path.

Table 1: Single CC power classes and their MPR status

| Power class | Number of TX paths and power capability | Status of MPR in 38.101-1-g50 | behavior |
| --- | --- | --- | --- |
| PC3 | 1xPC3 | Table 6.2.2-1 | Baseline |
| PC2 | 1xPC2 | Table 6.2.2-2 | Edge allocation added due to SEM limit |
| PC1.5 | 2xPC2 | Table 6.2.2-4 | = PC2+3dB with ceil at 0 but 16/64QAM inner seem better than 1TX PC2 while RIMD is impacting |
| PC2 | 2xPC2 | Assumes Table 6.2.2-2 | Same absolute power capability at antenna as PC1.5 but with 3dB lower reference for MPR |
| PC2 | 2xPC3 | Failed to be introduced so far | Slightly worse MPR than 1xPC2 because of 30dBc ACLR capability of PA versus -31 dBc PC2 target (0.3dB?) but moreover because of RIMD |
| PC3 | 2xPC3 | Assumes Table 6.2.2-1 | It’s the case for UL MIMO or fall back modes for duty cycle.. |
| PC5 | 1xPC5 | Table 6.2F.2-1 | For NRU waveforms with different requirement not comparable to NR MPR |
| PC3 | 2xPC5 | Future NRU? |  |
| … | Any hybrid |  |  |

Observations:

- Power class needs to be clarified in terms of number of TX paths:

  - May be two TX MPR should be in a different section from one TX MPR in UL MIMO section for example

  - Signaling or declaration is needed to know which MPR applies to the actual UE and how to test it.

- 2 Tx Hybrid forms are possible (such as PC2+PC3) but the following could be assumed:

  - If one path has the power capability equivalent to the power class, then in single port mode 1Tx is used (no transparent Tx diversity)

  - In UL MIMO mode, the two TX operate as if they had the lowest power capability

- When 2 TX with the same capability are used for two power classes like 2xPC2 for PC2 or PC1.5 it should be feasible to use the same table as the difference is only the Pmax reference and not the output power at which the emissions are met.

Proposals:

- 2 Tx MPR should be the same MPR requirement for TX Diversity and UL MIMO for the same power class.

- 2 Tx MPR table should be the same for different 2 TX power classes based on the same 2 TX paths as it is only a difference of Pmax reference.

- 2 Tx Hybrid forms should not have specific MPR but agreed behavior in single port and UL MIMO modes.

- FFS if 1 TX and 2 TX MPR tables should be in the same or separate clauses.

Note that this is very important as we are now already discussing TX architectures for UL CA combined with UL MIMO and PC3/PC2.

# Conclusions

In this contribution, we provide our views on possible power class and transmit chain architectures cases and their MPR specification and make the following proposals.

Proposals:

- 2 Tx MPR should be the same MPR requirement for TX Diversity and UL MIMO for the same power class.

- 2 Tx MPR table should be the same for different 2 TX power classes based on the same 2 TX paths as it is only a difference of Pmax reference.

- 2 Tx Hybrid forms should not have specific MPR but agreed behavior in single port and UL MIMO modes.

- FFS if 1 TX and 2 TX MPR tables should be in the same or separate clauses.

# References

[1]	R4-2011922, CR for TS 38.101-3 introduce new power class for EN-DC.docx, Huawei, RAN4#96e

[2] R4-2010060, CR for 38.101-1: Introduction of Power Class 1.5, T-Mobile US, RAN4#96e

[3] RP-202117, Introduction of NR-based access to unlicensed spectrum, Qualcomm Incorporated, Nokia, AT&T, Charter Communications, Inc., Verizon, Samsung, LGE, Skyworks Solutions, Inc.RAN#89e