3GPP TSG-RAN WG4 Meeting #97-e		R4-2016468

Electronic, 2nd – 13th November, 2020

Agenda item:	7.15.3.2.3

Source:	Intel Corporation

Title:	Simulation results for NR HST UL TA

Document for:	Discussion

# Introduction

Last meeting it was agreed to define HST UL TA demodulation requirements under multi-path fading propagation conditions with 120 km/h UE speed  - Scenario X [1]. In this contribution we provide corresponding simulation results and also present results for additional CBWs for scenario Y and Z.

# Discussion

## Simulation results for Scenarios Y and Z

In this section, the ideal and impairment results for HST UL TA scenarios Y and Z for 5 MHz and 10 MHz CBW are presented.

Table 1. Simulation results for Scenarios Y and Z

| Propagation Conditions | CBW, MHz | SCS, kHz | Mapping type | Ideal, dB | Impairment, dB |
| --- | --- | --- | --- | --- | --- |
| Scenario Y | 5 | 15 | A | 6.1 | 8.6 |
| Scenario Y | 5 | 15 | B | 6.1 | 8.6 |
| Scenario Y | 10 | 30 | A | 6.1 | 8.6 |
| Scenario Y | 10 | 30 | B | 6.1 | 8.6 |
| Scenario Z | 5 | 15 | A | 6.1 | 8.6 |
| Scenario Z | 5 | 15 | B | 6.1 | 8.6 |
| Scenario Z | 10 | 30 | A | 6.1 | 8.6 |
| Scenario Z | 10 | 30 | B | 6.1 | 8.6 |

## Simulation results for Scenario X

In Table 2 we provide link-level results for UL TA Scenario X for all considered SCS/CBW combinations.

Table 2. Simulation results for Scenario X

| Propagation Conditions | CBW, MHz | SCS, kHz | Mapping type | Ideal, dB | Impairment, dB |
| --- | --- | --- | --- | --- | --- |
| Scenario X | 10 | 15 | A | 8.9 | 11.4 |
| Scenario X | 10 | 15 | B | 9.9 | 12.4 |
| Scenario X | 40 | 30 | A | 9.6 | 12.1 |
| Scenario X | 40 | 30 | B | 10 | 12.5 |
| Scenario X | 5 | 15 | A | 8.3 | 10.8 |
| Scenario X | 5 | 15 | B | 8.3 | 10.8 |
| Scenario X | 10 | 30 | A | 8.7 | 11.2 |
| Scenario X | 10 | 30 | B | 8.8 | 11.3 |

# Conclusion

In this contribution we have provided simulation results for NR HST UL TA performance requirements definition.

# References

- R4-2012676 “WF on Rel-16 NR HST BS demodulation requirements”, Nokia, RAN4 #96-e, August 2020