3GPP TSG-RAN WG4 Meeting #97-e	R4-2016552

# Online Meeting, November 2 – 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7006 | rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for DLchannel quality report accuracy for eMTC UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Perf |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Rel-16 adds support for DL channel quality report for eMTC UE. Test cases to verify DL channel quality report accuracy requirements need to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test cases for DL channel quality report accuracy in RRC_IDLE and RRC_CONNECTED for FD-FDD, HD-FDD and TDD UE Cat-M1 under normal coverage and enhanced coverage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Accuracy requirements for DL channel quality report in RRC_CONNECTED could not be verified due to lack of tests |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change >

### A.9.14.5	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A

#### A.9.14.5.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.5.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.5.2-1 and A.9.14.5.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.5.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE in CE Mode A

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 8.13.2.8-2. |
| M-PDCCH repetition level |  | 1 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-3. |  |  |  |

Table A.9.14.5.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE in CE Mode A

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.17 FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |
| A, B |  | -3 |  |
| MPDCCH_RA | dB | 0 |  |
| MPDCCH_RB | dB | 0 |  |
| PBCH_RA | dB | -3 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 6 | 0 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.5.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A

#### A.9.14.6.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.6.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.6.2-1 and A.9.14.6.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.6.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE in CE Mode A

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 8.13.2.8-2. |
| M-PDCCH repetition level |  | 1 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-4. |  |  |  |

Table A.9.14.6.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE in CE Mode A

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.2 |  | R.7 HD-FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |
| A, B |  | -3 |  |
| MPDCCH_RA | dB | 0 |  |
| MPDCCH_RB | dB | 0 |  |
| PBCH_RA | dB | -3 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 6 | 0 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.6.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.7	E-UTRAN TDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A

#### A.9.14.7.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.7.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.7.2-1 and A.9.14.7.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.7.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE in CE Mode A

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 8.13.2.8-2. |
| M-PDCCH repetition level |  | 1 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-3. |  |  |  |

Table A.9.14.7.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE in CE Mode A

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| Special subframe configuration Note 5 |  | 6 |  |
| Uplink-downlink configuration Note 5 |  | 1 |  |
| MPDCCH parameters as defined in A.3.1.3.3 |  | R.15 TDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.11 TDD |  |
| A, B |  | -3 |  |
| MPDCCH_RA | dB | 0 |  |
| MPDCCH_RB | dB | 0 |  |
| PBCH_RA | dB | -3 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 6 | 0 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. |  |  |  |

#### A.9.14.7.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.8	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B

#### A.9.14.8.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.8.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.8.2-1 and A.9.14.8.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.8.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE in CE Mode B

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 8.13.3.8-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-1. |  |  |  |

Table A.9.14.8.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE in CE Mode B

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.4 |  | R.19 FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.8.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.9	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B

#### A.9.14.9.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.9.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.9.2-1 and A.9.14.9.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.9.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE in CE Mode B

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 8.13.3.8-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-2. |  |  |  |

Table A.9.14.9.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE in CE Mode B

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.5 |  | R.9 HD-FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.9.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.10	E-UTRAN TDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B

#### A.9.14.10.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in connected mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.10.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MAC CE-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.10.2-1 and A.9.14.10.2-2. There are two time periods T1 and T2 with different SNR levels. At the start of T2 the active cell should trigger a downlink channel quality report (“Regular DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.10.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE in CE Mode B

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 8.13.3.8-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 | s | 1 |  |
| T2 Note 1 | s | ≥ 1 |  |
| Note 1: The active cell should send a DCQR command MAC CE at the start of T2. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-1. |  |  |  |

Table A.9.14.10.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE in CE Mode B

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| Special subframe configuration Note 5 |  | 6 |  |
| Uplink-downlink configuration Note 5 |  | 1 |  |
| MPDCCH parameters as defined in A.3.1.3.6 |  | R.17 TDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.2 TDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. |  |  |  |

#### A.9.14.10.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.11	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 under normal coverage

#### A.9.14.11.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.11.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.11.2-1 and A.9.14.11.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.11.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE under normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-1. |  |  |  |

Table A.9.14.11.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE under normal coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.4 |  | R.19 FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -6 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.11.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.12	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 under normal coverage

#### A.9.14.12.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.12.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.12.2-1 and A.9.14.12.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.12.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE under normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-2. |  |  |  |

Table A.9.14.12.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE under normal coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.5 |  | R.9 HD-FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -6 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.12.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.13	E-UTRAN TDD Downlink channel quality reporting accuracy for UE Category M1 under normal coverage

#### A.9.14.13.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.23.

#### A.9.14.13.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.13.2-1 and A.9.14.13.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.13.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE under normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1A | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.23-1. |  |  |  |

Table A.9.14.13.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE under normal coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| Special subframe configuration Note 5 |  | 6 |  |
| Uplink-downlink configuration Note 5 |  | 1 |  |
| MPDCCH parameters as defined in A.3.1.3.6 |  | R.17 TDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.2 TDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -6 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. |  |  |  |

#### A.9.14.13.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.23.

### A.9.14.14	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 under enhanced coverage

#### A.9.14.14.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.14.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.14.2-1 and A.9.14.14.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.14.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE under enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-1. |  |  |  |

Table A.9.14.14.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN FD-FDD Category M1 UE under enhanced coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.4 |  | R.19 FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.14.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.15	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 under enhanced coverage

#### A.9.14.15.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.15.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.15.2-1 and A.9.14.15.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3. The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.15.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE under enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-2. |  |  |  |

Table A.9.14.15.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN HD-FDD Category M1 UE under enhanced coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| MPDCCH parameters as defined in A.3.1.3.5 |  | R.9 HD-FDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.6 FDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. |  |  |  |

#### A.9.14.15.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

### A.9.14.16	E-UTRAN TDD Downlink channel quality reporting accuracy for UE Category M1 under enhanced coverage

#### A.9.14.16.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy in idle mode is within the specified limits. This test will verify the requirements in Section 9.1.21.24.

#### A.9.14.16.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.16.2-1 and A.9.14.16.2-2. There are two time periods T1 and T2 with different SNR levels. The start of T2 coincides with the start of the channel quality measurement period defined in clause 4.7.3.  The UE transmits the downlink channel quality report (“Msg3 DCQR”) as described in clause 5.25 of TS 36.321. Upon receiving the DCQR from the UE, the active cell should re-configure MPDCCH according to the signaled aggregation and repetition levels.

Table A.9.14.16.2-1: General Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE under enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DCI format |  | 6-1B | As defined in Table 4.7.3-1. |
| M-PDCCH aggregation level |  | 24 |  |
| T1 Note 1 | s | 1 |  |
| T2 Note 2 | s | - |  |
| Note 1: Prior to this period the UE has camped on the active cell and is in idle mode. The UE initiates a random access procedure. Note 2: This period begins from the start of the random access response. Enough time should be allowed to verify both accuracy requirements in Table 9.1.21.24-1. |  |  |  |

Table A.9.14.16.2-2: Cell specific Test Parameters for Downlink channel quality reporting accuracy test for E-UTRAN TDD Category M1 UE under enhanced coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 10 |  |
| Special subframe configuration Note 5 |  | 6 |  |
| Uplink-downlink configuration Note 5 |  | 1 |  |
| MPDCCH parameters as defined in A.3.1.3.6 |  | R.17 TDD |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.2 TDD |  |
| MPDCCH_RA | dB | -3 |  |
| MPDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15 kHz | -98 |  |
| SNR Note 3 | dB | 0 | -12 |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Channel quality report Note 4 |  | As defined in Table 9.1.21.22-1 |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 4: SIB2 field mpdcch-CQI-Reporting set to ‘fourBits’ indicates 4-bit CQI reporting is allowed. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. |  |  |  |

#### A.9.14.16.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.21.24.

# < End of change >