3GPP TSG-RAN WG4 Meeting #97-e	R4-2016566

Online, 02 – 13 November, 2020

Agenda item:	7.1.7.1

Source: 	Qualcomm Incorporated

Title: 	RSSI Measurement accuracy requirements in NR-U

Document for:	Discussion

# Introduction

In this paper, we discuss RSSI measurement bandwidth and RSSI measurement accuracy requirements in NR-U.

# RSSI Measurement Bandwidth

The topic of RSSI Measurement Bandwidth was briefly discussed during RAN4#96-e and the following options were captured in the WF [1]:

RSSI measurement BW

Candidate options:

- Option 1: RSSI measurement bandwidth is the LBT bandwidth

- Option 2: The discussion can take place in the performance work

We believe this discussion should take place under performance and hence we present our view on this topic in this paper.

We believe that RSSI measurement bandwidth that UE used does not need to be specified. The only requirement from UE side is to meet the RSSI measurement accuracy requirement and it should be left to UE implementation on how to meet those requirements.

Proposal 1. There is no need to specify RSSI measurement bandwidth for the UE.

# RSSI Measurement Accuracy

The purpose of RSSI measurements in NR-U is the same as RSSI measurements in CLI and LTE-LAA, so we believe the same measurement accuracy requirements should apply.

Proposal 2. The RSSI measurement accuracy requirements for NR-U are the same as for CLI-RSSI as specified in Section 10.1.22.2 in TS 38.133 (and for RSSI measurements in Section 9.1.18.5 in TS 36.133)

# Conclusions

Proposal 1. There is no need to specify RSSI measurement bandwidth for the UE.

Proposal 2. The RSSI measurement accuracy requirements for NR-U are the same as for CLI-RSSI as specified in Section 10.1.22.2 in TS 38.133 (and for RSSI measurements in Section 9.1.18.5 in TS 36.133)

# References

[1] R4-2012293