3GPP TSG-RAN WG4 Meeting #97e                         	             R4-2016589

# Electronic meeting, 2nd – 13th November, 2020

Agenda item:	12.1.3.1

Source:	Huawei, HiSilicon

Title:	Discussion on open issues of NR MIMO OTA WI

Document for:	Approval

# Introduction

In RAN4#96-e meeting, WF on NR MIMO OTA was agreed [1]. Significant progress had been made. In this document, we discuss and provide our views on the left open issues, including “The metric for FR1 performance requirements”, “The metric for FR2 performance requirements” and “RMC for FR2 performance requirements”.

# 2	Discussion

## 2.1 The metric for FR1 MIMO OTA performance requirements

Regarding FR1 performance requirement metric, the WF [1] agreed in last meeting is cited as below.

(From [1])

- For FR1 MIMO OTA performance requirements:

  - Only one outage point of TP@ 70% is selected for the final performance metric

    - This agreement is based on LTE TRMS analysis

  - Further check whether how many PMODE can reach TP @ 90% or 95% could be an additional FoM

    - Option 1：TP@90% can pass 11 of total 12 rotations

    - Option 2：TP@95% can pass 10 of total 12 rotations

    - Other options are not precluded

Apart from TP@70% for the final performance metric, the achievable of TP@90% or TP@95% was left for further discussion, i.e. option1 or option2. Here we propose to adopt “Option 2: TP@95% can pass 10 of total 12 rotations”, the consideration is that compared with TP@90%, TP@95% can better demonstrate the UE’s capability to reach almost full performance in ideal situations e.g. close to base station, high SNR and so on. In our view, these use cases are not rare in FR1 deployment scenarios of micro, small cell or even Marco cell. Therefore it is useful to examine the UE capability on this aspect.

Proposal 1: in addition to the agreement that “Only one outage point of TP@ 70% is selected for the final performance metric”, select option 2 in the WF as another check point i.e. “TP@95% can pass 10 of total 12 rotations”

## 2.2 The metric for FR2 MIMO OTA performance requirements

Regarding FR2 performance requirement metric, the WF [1] agreed in last meeting is cited as below.

(From [1])

- For FR2 MIMO OTA performance requirements:

  - Select averaging all the values better than [50%] percentile of CCDF as the Figure of Merit for FR2 MIMO OTA requirement

  - Further check whether [50%] percentile value of the CCDF curve should also be a FoM of FR2

  - Analysis on number of test points vs uncertainty of FR2 MIMO OTA performance is encouraged

In previous discussions, concern about the deviation of testing result was raised due to the relative sparse grid points i.e. 36 compared with more than 200 in the test of EIS spherical coverage. The averaging method is a possible way to partially alleviate this problem. According to theory of probability and statistics, by averaging over N Independent and identically distributed data (IID), we can reduce the standard deviation by N times. E.g. if IID x1, x2, … , xN follow the Independent distribution of N(0,σ2) , then the averaging of x1+ x2+…+xNN follows the distribution of N(0,σ2N). And as point out in previous discussion, the agreed 36 test points are not enough to make a reasonable CCDF statistics. Therefore, based on the above agreement on the WF, we propose to select “averaging all the value better than [xx%, “xx” is the %-tile of spherical coverage requirements for different power classes as below table] percentile of CCDF as the only Figure of Merit for FR2 MIMO OTA requirement.

| UE Power class | UE type | %-tile CDF for spherical coverage |
| --- | --- | --- |
| 1 | Fixed wireless access (FWA) UE | 85% |
| 2 | Vehicular UE | 60% |
| 3 | Handheld UE | 50% |
| 4 | High power non-handheld UE | 20% |

And not to introduce “[50%] percentile of the CCDF curve” as another FoM, the considerations are further summarized as below:

- We anticipate the problem of larger deviation, because the agreed 36 evenly spaced test points are sparse. In 38.810, for FR2 EIS testing, results of 30 constant density measurement grips have the STD of 0.55 dB. Despite it cannot fully represent the situation in MIMO OTA, e.g. MIMO channel model vs FS,  4x2 vs 8x2 antenna array and so on, the large deviation from sparse test points are still expected to be a problem which prevent the usage of CCDF curve sampling method.

- The averaging method can reduce the standard deviation by N times for N i.i.d. variables. If the averaging method can be justified as fulfilling the testing purpose, i.e. judge “good” or “bad” UEs. We prefer not to have two requirements.

Proposal 2: clarify the agreement as “select averaging all the value better than [xx%, “xx” is the %-tile of spherical coverage requirements for different power classes] percentile of CCDF as the only Figure of Merit for FR2 MIMO OTA requirement”. And not to introduce “[50%] percentile of the CCDF curve” as another FoM.

(From 38.810 Table G.3.3.2.2-2: Statistical results of EIS50% CDF for the 8x2 antenna array for constant density measurement grids and the beam peak oriented in completely random orientations.)

|  | DL Power Step Size: infinitesimal |  |  | DL Power Step Size: 0.1dB |  |  | DL Power Step Size: 0.5dB |  |  | DL Power Step Size: 1dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of unique grid points | STD [dB] | ¦Mean Error¦ [dB] | Span [dB] | STD [dB] | ¦Mean Error¦ [dB] | Span [dB] | STD [dB] | ¦Mean Error¦ [dB] | Span [dB] | STD [dB] | ¦Mean Error¦ [dB] | Span [dB] |
| 30 | 0.56 | 0.16 | 3.85 | 0.56 | 0.23 | 4.20 | 0.55 | 0.57 | 3.50 | 0.53 | 1.03 | 3.63 |
| 40 | 0.50 | 0.15 | 3.87 | 0.50 | 0.23 | 3.40 | 0.49 | 0.58 | 3.77 | 0.47 | 1.04 | 3.67 |
| 50 | 0.41 | 0.11 | 3.25 | 0.41 | 0.18 | 3.10 | 0.39 | 0.54 | 2.87 | 0.38 | 1.03 | 2.83 |
| 70 | 0.25 | 0.06 | 2.31 | 0.24 | 0.14 | 2.20 | 0.24 | 0.51 | 2.02 | 0.23 | 1.02 | 2.00 |
| 100 | 0.20 | 0.03 | 1.50 | 0.20 | 0.11 | 1.58 | 0.19 | 0.50 | 1.41 | 0.18 | 1.00 | 1.35 |
| 150 | 0.17 | 0.02 | 1.36 | 0.17 | 0.10 | 1.43 | 0.17 | 0.50 | 1.33 | 0.15 | 1.00 | 1.37 |
| 200 | 0.10 | 0.02 | 0.80 | 0.10 | 0.10 | 0.86 | 0.10 | 0.50 | 0.79 | 0.09 | 1.01 | 0.74 |
| 300 | 0.08 | 0.01 | 0.65 | 0.08 | 0.10 | 0.66 | 0.08 | 0.50 | 0.56 | 0.07 | 1.01 | 0.67 |
| 400 | 0.06 | 0.01 | 0.54 | 0.06 | 0.10 | 0.49 | 0.06 | 0.50 | 0.44 | 0.05 | 1.01 | 0.48 |
| 500 | 0.06 | 0.01 | 0.53 | 0.06 | 0.10 | 0.47 | 0.06 | 0.50 | 0.43 | 0.05 | 1.01 | 0.42 |

## 2.3 RMC for FR2 MIMO OTA performance requirements

Regarding RMC for FR2 MIMO OTA performance requirements, the WF [1] agreed in last meeting is cited as below.

From [1]

- For FR2 MIMO OTA performance requirements:

  - Further discuss the down-selection of FR2 RMC, feasibility of 64 QAM for FR2 will be checked, and final conclusion will be made next meeting.

    - Technical input from companies is encouraged to analyze SNR range and feasibility of 64QAM

The following framework was discussed in [3] in which the SNR in 3D-MPAC consists of SNR with single probe and the contribution of multi-probe.

SNR3D-MPAC = SNRsingle-probe + ΔSNRmulti-probe

Based on the analysis in [2], for FR2 band n260, 100MHz channel bandwidth, direct far field (DFF) with the 0.75m measurement distance and the noise-limited test condition, the SNR upper bound of multi-band UE is 24.4dB for the beam peak direction.

By taking into account the difference between requirements of “Reference sensitivity power level” and “EIS spherical coverage” for FR2 power class 3 UE (12.6dB for n260), the upper bound SNRsingle-probe for N260 can be calculated as 24.4-12.6=11.8dB, as in [4].

For n257/n258/n261 the EIS requirements and path loss are different from N260.

- The “EIS spherical coverage” difference between n260 and n257/n258/n261 for power class 3 UE is 77.4-73.1=4.3dB.

- The frees-space path loss difference between 43.5GHz and 28GHz is about 3.8dB at 0.75m distance according to the formula PL=λ4πd2.

Therefore, the upper bound SNRsingle-probe for n257/n258/n261 can be roughly calculated as 11.8+4.3+3.8=19.9dB

According to the analysis in [2], the multi-probe SNR contribution (ΔSNRmulti-probe) is expected to be in the range of [0, 3.5]dB, taking into account the impact of the probe weights at TE side and the antenna directivity pattern at UE side.

As summary, according to SNR3D-MPAC = SNRsingle-probe + ΔSNRmulti-probe, we expect SNR3D-MPAC ~ 11.8+[0, 3.5]dB for n 260, and SNR3D-MPAC ~ 19.9+[0, 3.5]dB for n257/n258/n261.

Compared with below table from 38.101-4, and taking into account the SNR difference between TP@70% and TP@95, which is normally about 3~4dB [5], we can conclude that 16QAM with 100MHz bandwidth is feasible for n257/n258/n261 MIMO OTA e.g. R.PDSCH.5-2.2 TDD.

For n260, the SNR is very challenging, QPSK (R.PDSCH.5-4.1 TDD) can work, another possible way is to reduce the bandwidth for 16QAM (e.g. 16QAM with 25 or 20MHz bandwidth).

Proposal 3: use 16QAM with 100MHz bandwidth as FR2 MIMO OTA RMC for n257/n258/n261. For n260, consider QPSK, or reduce the bandwidth for 16QAM (e.g. 16QAM with 25 or 20MHz bandwidth).

From 38.101-4: Table 7.2.2.2.1-4: Minimum performance for Rank 2 (FRC)

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 2-1 | R.PDSCH.5-4.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 4.1 |
| 2-2 | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.4 |
| 2-3 | R.PDSCH.5-5.2 TDD | 50 / 120 | 16QAM,0.48 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 14.0 |
| 2-4 | R.PDSCH.5-2.3 TDD | 200 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.2 |
| 2-5 | R.PDSCH.4-1.1 TDD | 50 / 60 | 16QAM, 0.48 | FR2.60-1 | TDLA30-75 | 2x2 ULA Low | 70 | 14.3 |
| 2-6 | R.PDSCH.5-6.1 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 18.6 |

# 3	Conclusions

In this document, we shared our views on some open issues of the NR MIMO OTA WI discussion.

Proposal 1: in addition to the agreement that “Only one outage point of TP@ 70% is selected for the final performance metric”, select option 2 in the WF as another check point i.e. “TP@95% can pass 10 of total 12 rotations”.

Proposal 2: clarify the agreement as “select averaging all the value better than [xx%, “xx” is the %-tile of spherical coverage requirements for different power classes] percentile of CCDF as the only Figure of Merit for FR2 MIMO OTA requirement”. And not to introduce “[50%] percentile of the CCDF curve” as another FoM.

Proposal 3: use 16QAM with 100MHz bandwidth as FR2 MIMO OTA RMC for n257/n258/n261. For n260, consider QPSK, or reduce the bandwidth for 16QAM (e.g. 16QAM with 25 or 20MHz bandwidth).

# References

[1] RAN4#96, R4-2012707 WF on MIMO OTA, vivo, CAICT, Spirent

[2] RAN4#94 R4-2000505, Study feasible SNR ranges for NR FR2 MIMO OTA in 3D MPAC, Qualcomm

[3] RAN4#95 R4-2006431, Discussion on SNR analysis for FR2 3D-MPAC, RAN4#95, Samsung

[4] RAN4#96: R4-2011335 Consideratons on performance requiremetns for FR2 MIMO OTA, Qualcomm

[5] RAN4#90 R4-1900362, Summary of simulation results of NR UE demod (FR2), Intel