3GPP TSG-RAN WG4 Meeting # 97-e                                  		R4-2016749

Electronic Meeting, 2nd – 13th November, 2020

Source: 	Huawei, HiSilicon

Title: 	TP for TR 37.717-00-00 for CA_n1A_SUL_n78A-n84A

Agenda Item:	10.8.2

Document for:	Approval

- 1 Introduction

The WID for SUL band combinations was updated in RAN #89 meeting. This contribution provides a TP for CA_n1A_SUL_n78A-n84A for TR 37.717-00-00.

- References

[1]	RP-201729, “New WID on Rel-17 Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)”, RAN #89e

- Text Proposal

<TP for TR 37.717-00-00>

## 5.x	CA_n1A_SUL_n78A-n84A

### 5.x.1	Operating bands

Table 5.x.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_SUL_n78-n84 | n1, n78, n84 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.x.2	Channel bandwidths per operating band

Table 5.x.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n84A | SUL_n78A-n84A | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n84 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 5.x.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.x.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.x.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1_SUL_n78-n84.

Table 5.x.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n1 | DL frequency range | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n78 | UL/DL frequency range | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n84 | UL frequency range | 1920 | 1980 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |

There is no harmonic/harmonic mixing issue for this band combination.

### 5.x.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.x.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n841 | 15 | 25 | 50 | 75 | 100 | 128 | 128 | 128 | 128 |  |  |  |  |  |
| n78 | n84 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.x.6	∆TIB and ∆RIB values

For CA_n1_SUL_n78-n84, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1_SUL_n78-n84 | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n84 | 0.6 |

Table 5.x.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1_SUL_n78-n84 | n1 | 0.2 |
|  | n78 | 0.5 |

<End of TP >