3GPP TSG-RAN WG4 Meeting # 97-e												R4-2017009

Electronic Meeting, 2 – 13 November, 2020

Agenda item:			7.5.2.1, 7.5.3.1, 7.5.3.2

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [97e][210] LTE_NR_DC_CA_RRM_1

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

Core requirements are closed, and work needs to be finalized. Hence, RAN4 should strive to reach a compromise agreement for MR-DC EMR in this meeting.

To facilitate the discussion, it is proposed to go one step back and focus on the where the main open aspects are and solve those. Based on the contributions and work so far, the biggest open topics are:

- UE idle mode CA measurement requirements and s-NonIntraSearch.

- Overlapping and non-overlapping carriers.

There re other open items as well, but they depend in some degree on having agreements on the 2 aspects above. If these two aspects can be agreed in RAN4#97 it is also feasible continue the detailed performance work including test case development.

From the discussion, if RAN4 agree to define same measurement requirements for all carriers configured for EMR, then there is likely no reason to define overlapping and non-overlapping carriers for Rel-16 EMR.

Hence, to progress the work, it is proposed decouple the idle mode CA measurement requirements discussion from the discussion related to overlapping and non-overlapping carriers.

This way RAN4 would initially discuss the UE idle mode CA measurement requirements for idle mode CA measurements in a generic manner for a carrier which is configured for mobility and idle mode CA measurements.

Once agreement is reached on this scenario, RAN4 can continue the discussion and see if agreement can be reached that same UE idle mode CA measurement requirements can apply also for a carrier only configured for idle mode CA measurements (and not mobility).

Following the UE idle mode CA measurement requirements and s-NonIntraSearch discussion RAN4 can then discuss the issue of overlapping and non-overlapping carriers.

Some companies are proposing to define same measurement requirements for overlapping and non-overlapping carriers without explicitly stating whether to define overlapping and non-overlapping carriers. Other companies have explicitly stated there is no need to define overlapping and non-overlapping carriers. And yet other companies propose how to define the overlapping and non-overlapping carriers.

## Discussions in 1st round

Hence, in the 1st round discussion the focus is on discussing and reach agreements related to:

- Topic #1: UE idle mode CA measurement requirements and s-NonIntraSearch.

  - Sub-topic #1-1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are not configured

    - Issue 1-1-1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are not configured

  - Sub-topic #1-2: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured

    - Issue 1-2-1: Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ (high priority carrier not configured)

    - Issue 1-2-2: Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ (high priority carrier configured)

    - Issue 1-2-3: Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (high priority carrier configured)

    - Issue 1-2-4: Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (high priority carrier not configured)

- Topic #2: Overlapping and non-overlapping carriers.

  - Sub-topic 2-1: Need for RAN4 define different UE idle mode CA measurement requirements depending on whether the EMR carrier is measured for mobility or not

  - Sub-topic 2-2: UE measurement requirements and scaling according to measured carriers.

  - Sub-topic 2-3: Can the EMR carrier dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds?

  - Sub-topic 2-4: Is there a need for the definition of overlapping and non-overlapping carriers?

- Topic #3: Performance – accuracy requirements

  - Sub-topic 3-1: Introduction of accuracy requirements for all MR-DC EMR cases introduced.

  - Sub-topic 3-2: Re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements

  - sub-topic 3-3: Measurement accuracy relaxation compared to existing connected mode requirements

- Topic #4: Test cases for MD-DC EMR

  - Sub-topic 4-1: Time plan

  - Sub-topic 4-2: Test case list

If agreements are reached RAN4 can continue to other topics as listed in 2nd round discussion.

## Discussions in 2nd round

Based on the progress of 1st round discussions:

- Timer T331 length.

- UE Idle mode CA measurement requirements and UE conditions

- Measurement requirement for UE who supports the beam level EMR reporting

- Cell detected status

# Topic #1: UE idle mode CA measurement requirements and s-NonIntraSearch

Main technical topic overview. The structure can be done based on sub-agenda basis.

Moderator comment: In the following table all proposals and observations have been copied and those which were not seen relevant for this discussion has been ‘removed’ by use of strikethrough.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2014361 | Mediatek | Tdoc Title: Discussion on LTE CRS based and NR SSB based measurement in NR IDLE/INACTIVE mode Proposal 1: The same measurement requirements for both overlapping and non-overlapping EMR carriers are specified. Proposal 2: According to R4-2009264, different EMR measurement requirements are specified for Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and for Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. Proposal 3: Ran4 to follow the measurement rules of cell re-selection and update the overlapping and non-overlapping EMR carrier definitions as following table Table 3: Definition of overlapping and non-overlapping EMR carrier Inter-frequency/Inter-RAT E-UTRAN layers configured for both re-selection and EMR; higher priority layers; equal/lower priority layers. Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ; Overlapping; Non-overlapping. Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ; Overlapping; Overlapping. Proposal 4: Introduce measurement periodicity requirements for non-overlapping EMR carriers Proposal 5: For measurement requirement of EMR carriers, evaluating time should be multiplied by the total carrier number to be monitored, i.e., the summation of total overlapping EMR carrier number, total non-overlapping EMR carrier number, and total reselection carriers without EMR configured Proposal 6: RAN4 to reuse the evaluating time for overlapping inter-freq. and inter-RAT EMR carriers as the evaluating time for non-overlapping inter-freq. and inter-RAT EMR carriers Proposal 7: RAN4 to specify the measurement requirements for non-overlapping EMR carriers when serving cell quality is lower than the s-NonIntraSearch thresholds as below table Table 5: Measurement requirements for cell re-selection purpose and EMR purpose (Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ) Un-detected cell; detected cell. Inter-freq. overlapping; Kcarrier’ * Tdetect,NR_Inter; Kcarrier’ * Tevaluate,NR_Inter. Inter-freq. non-overlapping; Kcarrier’ * Tdetect,NR_Inter_nonOverEMR; Kcarrier’ * Tevaluate,NR_Inter_nonOverEMR. Inter-RAT overlapping; NEUTRA_carrier’ * Tdetect,EUTRAN; NEUTRA_carrier’ * Tevaluate,EUTRAN. Inter-RAT non-overlapping; NEUTRA_carrier’ * Tdetect,EUTRA_nonOverEMR; NEUTRA_carrier’ * Tevaluate,EUTRAN_nonOverEMR. Proposal 8: RAN4 to specify the measurement requirements for non-overlapping EMR carriers when serving cell quality is higher than the s-NonIntraSearch thresholds as below table Table 6: Measurement requirements for cell re-selection purpose and EMR purpose (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) Un-detected cell; detected cell. Inter-freq. overlapping; Kcarrier’ * (Thigher_priority_search + Tevaluate,NR_Inter); Kcarrier’ * Tevaluate,NR_Inter. Inter-freq. non-overlapping; Kcarrier’ * (Thigher_priority_search + Tevaluate,NR_Inter_nonOverEMR); Kcarrier’ * Tevaluate,NR_Inter_nonOverEMR. Inter-RAT overlapping; NEUTRA_carrier’ * (Thigher_priority_search + Tevaluate,EUTRAN); NEUTRA_carrier’ * Tevaluate,EUTRAN. Inter-RAT non-overlapping; NEUTRA_carrier’ * (Thigher_priority_search + Tevaluate,EUTRAN_nonOverEMR); NEUTRA_carrier’ * Tevaluate,EUTRAN_nonOverEMR. Proposal 9: RAN4 to introduce additional time period TtimeIndex,NR_Inter in measurement requirement for UE who supports the beam level EMR reporting, where TtimeIndex,NR_Inter equals to 3* Tmeasure,NR_Inter in FR1 and 5* Tmeasure,NR_Inter in FR2 |
| R4-2015587 | ZTE | Tdoc Title: Remaining issues on NR EMR Proposal 1: The same measurement requirements for both overlapping and non-overlapping EMR carriers are specified. Proposal 2: EMR carrier measurement interval is Thigher_priority_search if signal strength and signal quality is above search threshold. Proposal 3: EMR carrier measurement interval is (Tdetect,NR_Inter, Tmeasure,NR_Inter) if signal strength or signal quality is below search threshold. Proposal 4: The EMR carrier measurement accuracy requirements are irrelevant of search threshold. Proposal 5: LS RAN2 to ask for adding additional candidate value of timer T331. Proposal 6: The additional candidate value for timer T331 is up to 1200s. Proposal 7: UE additional time period in measurement requirement for UE who supports the beam level EMR reporting is TtimeIndex,NR_Interf = [3]* Tmeasure,NR_Interf. Proposal 8: EMR carriers are actively measured for EMR while T331 is running |
| R4-2015742 | Huawei | Tdoc Title: Discussion on remaining issues in EMR requirements Proposal 1: Define EMR measurement period requirements to be same as that for high priority mobility measurement. Proposal 2: The measurement period requirements should be scaled by the number of carriers actively measured for mobility plus the number of carriers that is actively measured for EMR only. Proposal 3: RAN4 to not define overlapping and non-overlapping carriers. |
| R4-2015881 | Nokia | Tdoc Title: Early Measurement Reporting - An overlapping carrier is an actively measured carrier configured by higher layer for mobility and early measurement reporting. A non-overlapping carrier is defined as an actively measured carrier configured by higher layer for early measurement reporting while not configured for mobility. A carrier configured for early measurement reporting is actively measured provided T331 has not expired, the serving cell is supporting early measurement reporting and the serving cell is in the validity area. - If Proposal 1 is not agreeable, the definitions of overlapping and non-overlapping carriers follow Rel-15 definition directly: - An overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements. - A non-overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements. Decouple the definitions of overlapping and non-overlapping carriers from s-NonIntraSearch thresholds. Rel-16 UE measurement requirements for EMR follow Rel-15 baseline and RAN4 requirements allow flexibility in applying the s-NonIntraSearch thresholds to prevent inter-frequency/RAT measurements with different periodicities. Allow UEs to measure, and perform EMR measurements, according to its preference, accounting any UE specific complexity related to measuring inter-frequency/RAT measurements with different measurement periods. Enable more advanced UEs to benefit from performing more advanced measurements for EMR. When s-NonIntraSearch thresholds are not configured (hence, s-NonIntraSearch is infinite and Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ conditions are always fulfilled) the UE shall measure all configured overlapping EMR carriers according to 4.2.2.4 (and not 4.2.2.7). When s-NonIntraSearch threshold are configured (hence, s-NonIntraSearch is not infinite) Srxlev > SnonIntraSearchP or Squal > SnonIntraSearchQ conditions may be fulfilled or not. In this case, the UE measurement requirements are as follows: - If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ: UE shall follow measurement requirements according to 4.2.2.4 (not 4.2.2.7) for overlapping EMR carriers. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and UE is not configured with any high priority carriers: UE shall follow measurement requirement according to 4.2.2.4 without applying s-NonIntraSearch threshold for EMR carriers. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and UE is configured with one or more high priority carriers: UE shall at least measure at least high priority carriers according to 4.2.2.7 and 7 and shall at least measure at least measure configured, overlapping EMR carriers according 4.2.2.7. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: and UE is configured with one or more high priority carriers: UE may measure configured carriers according to 4.2.2.4. Agree on proposal 7 and proposal 8 as a package. The UE indicates to the network if the reported EMR results are performed while applying s-NonIntraSearch thresholds to the EMR carriers or not. For Rel-16 the requirements in proposal 7 and proposal 8 applies. RAN4 to send LS to RAN2 to introduce means for UE to indicate whether the UE applied search thresholds when measuring the reported early measurement or not. Indication should be defined in Rel-16, if possible, otherwise in Rel-17 If RAN2 cannot introduce the solution in Rel-16, we propose that for Rel-16, if s-NonIntraSearch thresholds are configured, and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and UE is configured with one or more high priority carriers, the UE may measure overlapping EMR according to section 4.2.2.7 or 4.2.2.4. If RAN2 cannot introduce the solution in Rel-16, we propose that or Rel-16 the network will not be aware of UE which UE requirements the UE applies if s-NonIntraSearch thresholds are configured, and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and UE is configured with one or more high priority carriers Agree Proposals 11 - 14 as a package. Related to measurement requirements on EMR carriers a number of cases are missing. for those we suggest following: For non-overlapping NR FR2 carriers, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting. For non-overlapping LTE inter-RAT carriers, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting. Measurement requirements for an overlapping NR inter-RAT EMR carrier follow the principles of of proposal 15. For non-overlapping NR inter-RAT carriers, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting. For an inter-frequency carrier, the UE is required to acquire the index of the SSB within 3 DRX cycles. Hence, X = 3. For an inter-RAT NR carrier, the UE is required to acquire the index of the SSB within 3 DRX cycles. Hence, X = 3. - When a cell will remain detectable after entering idle mode after connection release should not be conditioned by a future condition which is unknown at the time of the connection release. The cell detected state conditions during transitioning to idle mode need to be conditioned by the cell conditions at the time of release and not on potential future cell conditions once the connection is established. New condition would be: The detected cell and SSBs remains detectable during the state transitioning to idle mode. |
| R4-2016573 | Qualcomm | Tdoc Title: Early measurement reporting in MR-DC Proposal 1: Whether overlapping or non-overlapping EMR carrier is dynamically defined depending on the following attributes: - Whether T331 is running or not - Whether S-criteria condition is met or not - Whether EMR carrier is also configured as a measurement carrier for idle/inactive mode or not, and if configured whether it is higher priority frequency layer than serving/camped cell or not - Based on the attributes, if the carrier is determined to be measured for EMR and cell reselection at a given condition, it is defined as overlapping EMR carrier Proposal 2: A unified minimum measurement interval requirement applies to both EMR carriers and cell reselection inter-frequency layers. - When S-criteria condition is met (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ), the carriers shall be measured at least every Thigher_priority_search(=Nlayers x 60s), and Nlayers is the number of higher priority frequency layers plus the number of non-overlapping EMR carriers - When S-criteria condition is not met (Srxlev ≤ SnonIntraSearchP and Squal ≤ SnonIntraSearchQ), the carriers shall be measured at least every Kcarrier x Tmeasure,NR_Inter, and Kcarrier is the number of higher/equal/lower priority frequency layers plus the number of non-overlapping EMR carriers |

## Open issues summary and views’ collection for 1st round

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

Introduction to the discussion:

When discussing the measurement requirements, it is important to take into account both UE aspects and network aspects, with the target of defining requirements which are acceptable to all parties and useful in real field deployments.

As pointed out in R4-2015587:

- RAN4 also agrees that UE will perform EMR measurements no matter the signal strength is above or below the threshold.

- The controversial part is how the requirements are specified when signal strength is above the threshold.

This is also clear from other contributions.

In R4-2016573 a good reference figure is given to illustrate the possible different scenarios:

And the different scenarios described in the different contributions can be summarized in a table like (modified from R4-2015881):

| SnonIntraSearchP/Q configured |  |  |  | SnonIntraSearchP/Q not configured |
| --- | --- | --- | --- | --- |
| Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ |  | Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ |  | N/A |
| High Priority carrier not configured | High Priority carrier configured | High Priority carrier not configured | High Priority carrier configured | N/A |
| 4.2.2.x | 4.2.2.x | 4.2.2.x | 4.2.2.x | 4.2.2.x |

RAN4 need to reach an agreement related to the UE measurement requirements for idle mode CA measurements for each of the scenarios in the table.

To get common baseline understanding among the group, it is important to understand how RAN2 has defined the procedures for idle mode CA measurements. 38.133 section 5.7.8 and mobility idle mode measurements in 38.304 section 5.2.4.2:

#### 5.7.8.2a	Performing measurements

When performing measurements on NR carriers according to this clause, the UE shall derive the cell quality as specified in 5.5.3.3 and consider the beam quality to be the value of the measurement results of the concerned beam, where each result is averaged as described in TS 38.215 [9].

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListEUTRA and the SIB1 contains idleModeMeasurementsEUTRA:

3>	for each entry in measIdleCarrierListEUTRA within VarMeasIdleConfig:

4>	if UE supports NE-DC between the serving carrier and the carrier frequency indicated by carrierFreqEUTRA within the corresponding entry:

5>	perform measurements in the carrier frequency and bandwidth indicated by carrierFreq and allowedMeasBandwidth within the corresponding entry;

This is allowed for idle mode mobility measurements as stated in 38.304 section 5.2.4.2:

5.2.4.2	Measurement rules for cell re-selection

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority;

-	Otherwise, the UE shall perform measurements of NR inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 38.133 [8].

-	If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements, as specified in clause 5.2.4.9.

RAN4 has agreed to use the measurements requirements in sections 4.2.2.4 and 4.2.2.5, and the open aspect is how to define the UE measurement requirements for idle mode CA measurements, if s-NonIntraSearch thresholds are configured.

To progress the work, it is proposed decouple the idle mode CA measurement requirements discussion from the discussion related to overlapping and non-overlapping carriers.

### Sub-topic 1-1

Sub-topic description: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are not configured

Open issues and candidate options before e-meeting:

Issue 1-1-1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are not configured Proposals

  - Option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are not configured, follow requirements in section 4.2.2.4 table 4.2.2.4-1

  - Option 2: -

- Recommended WF (original)

  - Agree on option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are not configured, follow requirements in section 4.2.2.4 table 4.2.2.4-1.

Agreed during GTW:

Agreement: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are not configured, follow requirements in section 4.2.2.4 table 4.2.2.4-1

Note: To progress the work, it is proposed decouple the idle mode CA measurement requirements discussion from the discussion related to overlapping and non-overlapping carriers.

| Company | Comments |
| --- | --- |
| Huawei | We support the Recommended WF, provided that all the carriers UE measures for mobility and EMR are accounted in the scaling factor of the measurement period requirements. |
| MTK | Agree with WF |
| Apple | Recommended WF is OK. Huawei’s concern is also valid. Carrier with EMR only (f5 in above figure) should be counted in the scaling factor. |
| ZTE | We support the recommended WF. |
| Nokia | We support the recommended WF. The concern related to scaling is valid and should be accounted. This would be discussed in sub-topic 2-2. |
| Qualcomm | Agree with WF. And for #freq layers that need to be accounted in the scaling factor, we share the same view as Huawei and Apple. |
| Ericsson | Agree with the WF |

### Sub-topic 1-2

Sub-topic description: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured

Open issues and candidate options before e-meeting:

Next focus point is then when s-NonIntraSearch thresholds are configured. When this is the case at least one complexity concern has been raised by some companies:

- If UE is configured with high priority carrier for mobility and idle mode CA measurements on EMR carrier(s), this may lead to that the UE need to measure different carriers with different measurement interval.

This problem arises from the fact that the UE may be configured with one or more high priority mobility carriers (and not configured for idle mode CA measurements) and one or more carriers for idle mode CA measurements.

RAN4 can in this case split the discussion into following four aspects:

- Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ (high priority carrier not configured)

- Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ (high priority carrier configured)

- Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (high priority carrier not configured)

- Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (high priority carrier configured)

Each of these are handled in next 4 issues.

Issue 1-2-1: Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ (high priority carrier not configured)

- Proposals

  - Option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, and when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ follow requirements in section 4.2.2.4 table 4.2.2.4-1.

  - Option 2: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, and when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ follow other requirements.

    - For companies preferring this option: list exactly which other requirements (section and possibly table).

- Recommended WF (original)

  - Agree on option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, and when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ follow requirements in section 4.2.2.4 table 4.2.2.4-1.

Agreed during GTW:

Agreement:

UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, and when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ follow requirements in section 4.2.2.4 table 4.2.2.4-1.

Same principles will apply for inter-RAT measurements

E-UTRAN measurements when UE is in NR IDLE or INACTIVE mode

NR measurements when UE is in LTE IDLE mode

Note: To progress the work, it is proposed decouple the idle mode CA measurement requirements discussion from the discussion related to overlapping and non-overlapping carriers.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support the Recommended WF, provided that all the carriers UE measures for mobility and EMR are accounted in the scaling factor of the measurement period requirements. |
| MTK | Agree with WF |
| Apple | Recommended WF is OK. |
| ZTE | We support the recommended WF. |
| Nokia | We can support the recommended WF. |
| Qualcomm | Agree with WF. And for a scaling factor, our view on Issue 1-1-1 applies here too. |
| Ericsson | Agree with the recommended WF |

Issue 1-2-2: Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ (high priority carrier configured)

- Proposals

  - Option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, and when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ follow requirements in section 4.2.2.4 table 4.2.2.4-1.

  - Option 2: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, and when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ follow other requirements.

    - For companies preferring this option: list exactly which other requirements (section and possibly table).

- Recommended WF (original)

  - Agree on option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, and when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ follow requirements in section 4.2.2.4 table 4.2.2.4-1.

Hence, same as proposed WF as for Issue 1-2-1.

Agreed during GTW:

Agreement:

UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, and when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ follow requirements in section 4.2.2.4 table 4.2.2.4-1.

Same principles will apply for inter-RAT measurements

E-UTRAN measurements when UE is in NR IDLE or INACTIVE mode

NR measurements when UE is in LTE IDLE mode

Note: To progress the work, it is proposed decouple the idle mode CA measurement requirements discussion from the discussion related to overlapping and non-overlapping carriers.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support the Recommended WF, provided that all the carriers UE measures for mobility and EMR are accounted in the scaling factor of the measurement period requirements. |
| MTK | Agree with WF |
| Apple | Recommended WF is OK. |
| ZTE | We support the recommended WF. |
| Nokia | We support the recommended WF. |
| Qualcomm | Agree with WF. And for a scaling factor, our view on Issue 1-1-1 applies here too. |
| Ericsson | Agree with the WF |

Issue 1-2-3: Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (high priority carrier configured)

- Proposals

  - Option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7.

  - Option 2: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, follow other requirements.

    - For companies preferring this option: list exactly which other requirements (section and possibly table).

- Recommended WF (original)

  - Agree on option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7.

Outcome from GTW:

Candidate agreement:

UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7.

Send LS to RAN2 to inform on the agreement and RAN4 observations that the measurement duration can exceed the maximum configurable duration of T331 timer but there is no consensus in RAN4 whether the timer value needs to be increased. It is up to RAN2 whether and how to resolve the issue.

- Moderator Propose following compromise agreement:

  - UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7.

  - Send LS to RAN2 to inform on the agreement and RAN4 the observation that the UE measurement duration can exceed the current maximum configurable duration of T331 timer.

Note: To progress the work, it is proposed decouple the idle mode CA measurement requirements discussion from the discussion related to overlapping and non-overlapping carriers.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support the Recommended WF, provided that all the carriers UE measures for mobility and EMR are accounted in the scaling factor of the measurement period requirements. It is noted that option 1 states “at least follow requirements in section 4.2.2.7”. Our preference is to define one unified set of requirements for EMR based on section 4.2.2.7 for the concerned scenario, and we can discuss possible enhancement in future release if justified. |
| MTK | Share the same view with Huawei. We prefer to remove “at least” |
| Apple | Recommended WF is ok. We also agree with Huawei’s suggestion. |
| ZTE | We can support the recommended WF under the condition that candidate configurable value for timer T331 is increased. If following requirements in section 4.2.2.7 it needs at least 60*7=420s for UE to measure all the frequency layers by considering minimum UE capability. When beam level report is configured the needed measurement time will be much longer. The maximum configurable value for timer T331 in current spec is 300s. So the feature would be broken. |
| Nokia | We can compromise to support the recommended WF to progress the work. This would then address the raised technical concern from some companies related to UE potentially performing measurements with different periodicities on different carriers simultaneously. However, we also believe this is not aligned with the RAN2 procedures and RAN4 would need to inform RAN2 in an LS about this UE behaviour - if this WF is agreed. |
| Qualcomm | Agree with WF, and share the same view as Huawei. |
| Ericsson | Agree with the WF |

Issue 1-2-4: Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (high priority carrier not configured)

- Proposals

  - Option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7.

  - Option 2: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.4 table 4.2.2.4-1.

- Recommended WF

  - More discussion needed.

Status after the GTW:

Option 1: MTK, Huawei, Apple, Qualcomm, Ericsson

Option 2: Nokia, ZTE

Note: To progress the work, it is proposed decouple the idle mode CA measurement requirements discussion from the discussion related to overlapping and non-overlapping carriers.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support option 1. It is true that without high priority carrier configured (for mobility only), there is no complexity issue due to different measurement intervals for different carriers, but we still prefer to define a common requirement for EMR, i.e. independent of whether there is a high priority carrier configured or not. This should simplify the UE implementation. On the other hand, there is no clear benefit to follow option 2. It is noted that option 1 does not always lead to longer cell detection delay than option 2, and actually for FR2 the cell detection delay is shorter with option 1. |
| MTK | Support option 1. If Issue 1-2-3 is agreeable and requirement in section 4.2.2.7 is applied for carrier with higher priority. There is no clear benefit to further ask UE to change scheduling method for carrier with equal/low priority |
| Apple | There seems to be some typos in both option 1 and 2: - Option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is NOT configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7. - Option 2: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is NOT configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.4 table 4.2.2.4-1. Our view is to apply 60*Nlayers seconds period for this scenario. From UE implementation perspective, we don’t expect dependency of EMR requirements on whether high priority carrier is configured or not. These two functionalities are for different purpose. |
| ZTE | We can support Option 1 under the condition that candidate configurable value for timer T331 is increased. Same reason as for Issue 1-2-3 |
| Nokia | Agree with Apple concerning the typos. We follow the header (Issue 1-2-4 description) and consider that high priority carrier is NOT configured. Option 2 is preferred. First of all, in this case the UE will not have to follow measurement on different with different measurement periodicities, which was the technical concern raised. Hence, there is no such additional UE complexity in this case. Secondly, this behaviour will be according to and follow the RAN2 procedures defined for idle mode CA measurements where it is clear that UE is not to apply any search thresholds. Another aspect to consider is the potential impact on the UEs not configured with Idle CA measurements or UEs which have been configured with Idle mode CA measurements after T331 has expired. Search thresholds are broadcasted parameters and applies to all UEs in the cell. Hence, search thresholds are not per UE configured. This means, if the idle mode CA measurements are too relaxed if search thresholds are configured in the cell, it will be difficult to use search thresholds in a cell where idle mode CA measurements are used. This then means that not only UE supporting idle mode CA measurements, but any UE in the cell, may be negatively impacted on the UE power saving in the cell. To illustrate: If we take the scenario in this issue 1-2-4, if the network would configure the UE with 1) EMR carriers, 2) carriers for mobility only (non-EMR carriers) and 3) search thresholds; Both UEs configured with EMR and UEs not configured with EMR could benefit from power savings from applying search thresholds: - UE configured with EMR while T331 is running: this UE is only required to measure EMR carrier according to 4.2.2.4 table 4.2.2.4-1 while UE is not required pure mobility carriers if the Srxlev of the serving cell is above the search threshold. - UE not configured with EMR (UE not supporting EMR or when T331 has expired) is not required to measure pure mobility carriers if Srxlev of the serving cell is above search threshold but can follow requirements according to 4.2.2.7. This would mean that if 1 carrier is configured for EMR only and rest (let’s just say 1 carrier) is configured for mobility and not EMR, the UE measurements when in good serving cell conditions could be reduced (e.g. after T331 has expired). However, if the network is limited in its use of the search thresholds in cells where EMR is used the two UEs described above would not be able to reduce measurements even in good conditions but would be required to measure all configured carriers according to 4.2.2.4 table 4.2.2.4-1. To address Huawei comment on option 2 vs option 1, we agree that for FR1 and FR2 the delays will be different, and we should look at both. For FR2 the UE minimum requirements when using option 1 could look attractive and provide shorter delays. However, this is not the case for FR1. Additionally, using option 1 would always lead to a fixed delay (N*60 seconds) also under very good conditions while this is not the case with option 2. We expect that under good conditions the UE will be able to detect an FR2 cell faster than what is the minimum requirement, which is based on cell detection and measurement in worst conditions. Our understanding is that option 1 will not be aligned with the RAN2 rules and would in many situations lead to a longer delay in cell detection and measurements. For MTK comment: in this case the UE is not configured with any high priority carriers. We agree that in this situation the UE would need to adopt the implementation compared to existing. But this is how the RAN2 procedure is defined. The system benefit will be that the UE will perform the measurements more frequently (as frequent as if the search thresholds were not configured in the cell). For Apple: RAN4 shall develop UE minimum requirements according to the RAN2 procedures. We agree that option 2 may in some implementations mean that a new rule is needed. However, it does not seem reasonable to expect that a new feature would not lead to some UE (and network) changes. We do however not expect considering whether UE is being configured with a high priority carrier or not when deciding the measurement routine will increase the UE complexity significantly. The UE is already required to be able to support measurements with and without high priority carrier configured and with and without search thresholds configured in the cell. It should be a matter of selection. As stated in former Issue 1-2-3, we can compromise, to address the UE raised concerns, and to have UE Idle mode CA measurement requirements accounting search thresholds (although against RAN2) to avoid that the UE may have measure with different periodicities (although UE is never required to do this). But there is no technical reason in this case (Issue 1-2-4) why the UE cannot measure as defined by RAN2 and according to option 2. The concern raised earlier is not present. Option 2 may in some cases lead to longer delays in FR2 (worst case scenario). However, in most cases (normal and good conditions in FR2 and in FR1) we expect that the latency is shorter with option 2 than with option 1. RAN4 can address optimising the FR2 latency in later release if needed. |
| Qualcomm | It is hard for us to tell whether and which option will bring how much benefit to UE and/or network and if that will be valid for all scenarios. And it is also unsure about likelihood of this case happening. We want to make it independent from whether higher priority carrier is configured or not, which is Option 1 with a correction “NOT”. On the other hand, we also agree with Nokia’s arguments “there is no such additional UE complexity in this case” and “follow the RAN2 procedures defined for idle mode CA measurements where it is clear that UE is not to apply any search thresholds”. We’re neutral on this, but slightly prefer Option 1 because it will be consistent at least RAN4 EMR requirement spec point of view. |
| Ericsson | Option 1 |

# Topic #2: Overlapping and non-overlapping carriers

Moderator comment: In the following table all proposals and observations have been copied and those which were not seen relevant for this discussion has been ‘removed’ by use of strikethrough.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2014361 | Mediatek | Tdoc Title: Discussion on LTE CRS based and NR SSB based measurement in NR IDLE/INACTIVE mode Proposal 1: The same measurement requirements for both overlapping and non-overlapping EMR carriers are specified. Proposal 2: According to R4-2009264, different EMR measurement requirements are specified for Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and for Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. Proposal 3: Ran4 to follow the measurement rules of cell re-selection and update the overlapping and non-overlapping EMR carrier definitions as following table Table 3: Definition of overlapping and non-overlapping EMR carrier Inter-frequency/Inter-RAT E-UTRAN layers configured for both re-selection and EMR; higher priority layers; equal/lower priority layers. Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ; Overlapping; Non-overlapping. Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ; Overlapping; Overlapping. Proposal 4: Introduce measurement periodicity requirements for non-overlapping EMR carriers Proposal 5: For measurement requirement of EMR carriers, evaluating time should be multiplied by the total carrier number to be monitored, i.e., the summation of total overlapping EMR carrier number, total non-overlapping EMR carrier number, and total reselection carriers without EMR configured Proposal 6: RAN4 to reuse the evaluating time for overlapping inter-freq. and inter-RAT EMR carriers as the evaluating time for non-overlapping inter-freq. and inter-RAT EMR carriers Proposal 7: RAN4 to specify the measurement requirements for non-overlapping EMR carriers when serving cell quality is lower than the s-NonIntraSearch thresholds as below table Table 5: Measurement requirements for cell re-selection purpose and EMR purpose (Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ) Un-detected cell; detected cell. Inter-freq. overlapping; Kcarrier’ * Tdetect,NR_Inter; Kcarrier’ * Tevaluate,NR_Inter. Inter-freq. non-overlapping; Kcarrier’ * Tdetect,NR_Inter_nonOverEMR; Kcarrier’ * Tevaluate,NR_Inter_nonOverEMR. Inter-RAT overlapping; NEUTRA_carrier’ * Tdetect,EUTRAN; NEUTRA_carrier’ * Tevaluate,EUTRAN. Inter-RAT non-overlapping; NEUTRA_carrier’ * Tdetect,EUTRA_nonOverEMR; NEUTRA_carrier’ * Tevaluate,EUTRAN_nonOverEMR. Proposal 8: RAN4 to specify the measurement requirements for non-overlapping EMR carriers when serving cell quality is higher than the s-NonIntraSearch thresholds as below table Table 6: Measurement requirements for cell re-selection purpose and EMR purpose (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) Un-detected cell; detected cell. Inter-freq. overlapping; Kcarrier’ * (Thigher_priority_search + Tevaluate,NR_Inter); Kcarrier’ * Tevaluate,NR_Inter. Inter-freq. non-overlapping; Kcarrier’ * (Thigher_priority_search + Tevaluate,NR_Inter_nonOverEMR); Kcarrier’ * Tevaluate,NR_Inter_nonOverEMR. Inter-RAT overlapping; NEUTRA_carrier’ * (Thigher_priority_search + Tevaluate,EUTRAN); NEUTRA_carrier’ * Tevaluate,EUTRAN. Inter-RAT non-overlapping; NEUTRA_carrier’ * (Thigher_priority_search + Tevaluate,EUTRAN_nonOverEMR); NEUTRA_carrier’ * Tevaluate,EUTRAN_nonOverEMR. Proposal 9: RAN4 to introduce additional time period TtimeIndex,NR_Inter in measurement requirement for UE who supports the beam level EMR reporting, where TtimeIndex,NR_Inter equals to 3* Tmeasure,NR_Inter in FR1 and 5* Tmeasure,NR_Inter in FR2 |
| R4-2015587 | ZTE | Tdoc Title: Remaining issues on NR EMR Proposal 1: The same measurement requirements for both overlapping and non-overlapping EMR carriers are specified. Proposal 2: EMR carrier measurement interval is Thigher_priority_search if signal strength and signal quality is above search threshold. Proposal 3: EMR carrier measurement interval is (Tdetect,NR_Inter, Tmeasure,NR_Inter) if signal strength or signal quality is below search threshold. Proposal 4: The EMR carrier measurement accuracy requirements are irrelevant of search threshold. Proposal 5: LS RAN2 to ask for adding additional candidate value of timer T331. Proposal 6: The additional candidate value for timer T331 is up to 1200s. Proposal 7: UE additional time period in measurement requirement for UE who supports the beam level EMR reporting is TtimeIndex,NR_Interf = [3]* Tmeasure,NR_Interf. Proposal 8: EMR carriers are actively measured for EMR while T331 is running |
| R4-2015742 | Huawei | Tdoc Title: Discussion on remaining issues in EMR requirements Proposal 1: Define EMR measurement period requirements to be same as that for high priority mobility measurement. Proposal 2: The measurement period requirements should be scaled by the number of carriers actively measured for mobility plus the number of carriers that is actively measured for EMR only. Proposal 3: RAN4 to not define overlapping and non-overlapping carriers. |
| R4-2015881 | Nokia | Tdoc Title: Early Measurement Reporting - An overlapping carrier is an actively measured carrier configured by higher layer for mobility and early measurement reporting. A non-overlapping carrier is defined as an actively measured carrier configured by higher layer for early measurement reporting while not configured for mobility. A carrier configured for early measurement reporting is actively measured provided T331 has not expired, the serving cell is supporting early measurement reporting and the serving cell is in the validity area. - If Proposal 1 is not agreeable, the definitions of overlapping and non-overlapping carriers follow Rel-15 definition directly: - An overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements. - A non-overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements. Decouple the definitions of overlapping and non-overlapping carriers from s-NonIntraSearch thresholds. Rel-16 UE measurement requirements for EMR follow Rel-15 baseline and RAN4 requirements allow flexibility in applying the s-NonIntraSearch thresholds to prevent inter-frequency/RAT measurements with different periodicities. Allow UEs to measure, and perform EMR measurements, according to its preference, accounting any UE specific complexity related to measuring inter-frequency/RAT measurements with different measurement periods. Enable more advanced Ues to benefit from performing more advanced measurements for EMR. When s-NonIntraSearch thresholds are not configured (hence, s-NonIntraSearch is infinite and Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ conditions are always fulfilled) the UE shall measure all configured overlapping EMR carriers according to 4.2.2.4 (and not 4.2.2.7). When s-NonIntraSearch threshold are configured (hence, s-NonIntraSearch is not infinite) Srxlev > SnonIntraSearchP or Squal > SnonIntraSearchQ conditions may be fulfilled or not. In this case, the UE measurement requirements are as follows: - If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ: UE shall follow measurement requirements according to 4.2.2.4 (not 4.2.2.7) for overlapping EMR carriers. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and UE is not configured with any high priority carriers: UE shall follow measurement requirement according to 4.2.2.4 without applying s-NonIntraSearch threshold for EMR carriers. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and UE is configured with one or more high priority carriers: UE shall at least measure at least high priority carriers according to 4.2.2.7 and 7 and shall at least measure at least measure configured, overlapping EMR carriers according 4.2.2.7. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: and UE is configured with one or more high priority carriers: UE may measure configured carriers according to 4.2.2.4. Agree on proposal 7 and proposal 8 as a package. The UE indicates to the network if the reported EMR results are performed while applying s-NonIntraSearch thresholds to the EMR carriers or not. For Rel-16 the requirements in proposal 7 and proposal 8 applies. RAN4 to send LS to RAN2 to introduce means for UE to indicate whether the UE applied search thresholds when measuring the reported early measurement or not. Indication should be defined in Rel-16, if possible, otherwise in Rel-17 If RAN2 cannot introduce the solution in Rel-16, we propose that for Rel-16, if s-NonIntraSearch thresholds are configured, and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and UE is configured with one or more high priority carriers, the UE may measure overlapping EMR according to section 4.2.2.7 or 4.2.2.4. If RAN2 cannot introduce the solution in Rel-16, we propose that or Rel-16 the network will not be aware of UE which UE requirements the UE applies if s-NonIntraSearch thresholds are configured, and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and UE is configured with one or more high priority carriers Agree Proposals 11 – 14 as a package. Related to measurement requirements on EMR carriers a number of cases are missing. For those we suggest following: For non-overlapping NR FR2 carriers, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting. For non-overlapping LTE inter-RAT carriers, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting. Measurement requirements for an overlapping NR inter-RAT EMR carrier follow the principles of of proposal 15. For non-overlapping NR inter-RAT carriers, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting. For an inter-frequency carrier, the UE is required to acquire the index of the SSB within 3 DRX cycles. Hence, X = 3. For an inter-RAT NR carrier, the UE is required to acquire the index of the SSB within 3 DRX cycles. Hence, X = 3. - When a cell will remain detectable after entering idle mode after connection release should not be conditioned by a future condition which is unknown at the time of the connection release. The cell detected state conditions during transitioning to idle mode need to be conditioned by the cell conditions at the time of release and not on potential future cell conditions once the connection is established. New condition would be: The detected cell and SSBs remains detectable during the state transitioning to idle mode. |
| R4-2016573 | Qualcomm | Tdoc Title: Early measurement reporting in MR-DC Proposal 1: Whether overlapping or non-overlapping EMR carrier is dynamically defined depending on the following attributes: - Whether T331 is running or not - Whether S-criteria condition is met or not - Whether EMR carrier is also configured as a measurement carrier for idle/inactive mode or not, and if configured whether it is higher priority frequency layer than serving/camped cell or not - Based on the attributes, if the carrier is determined to be measured for EMR and cell reselection at a given condition, it is defined as overlapping EMR carrier Proposal 2: A unified minimum measurement interval requirement applies to both EMR carriers and cell reselection inter-frequency layers. - When S-criteria condition is met (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ), the carriers shall be measured at least every Thigher_priority_search(=Nlayers x 60s), and Nlayers is the number of higher priority frequency layers plus the number of non-overlapping EMR carriers - When S-criteria condition is not met (Srxlev ≤ SnonIntraSearchP and Squal ≤ SnonIntraSearchQ), the carriers shall be measured at least every Kcarrier x Tmeasure,NR_Inter, and Kcarrier is the number of higher/equal/lower priority frequency layers plus the number of non-overlapping EMR carriers |

## Open issues summary and views’ collection for 1st round

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

Introduction to the discussion:

To facilitate the discussion, we can use following figure from R4-2016573

And the table (R4-2014361):

Table 3: Definition of overlapping and non-overlapping EMR carrier

| Inter-frequency/Inter-RAT E-UTRAN layers configured for both re-selection and EMR | higher priority layers | equal/lower priority layers |
| --- | --- | --- |
| Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Overlapping | Non-overlapping |
| Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Overlapping | Overlapping |

And from R4-2015881:

Based on these agreements, RAN4 then in [R4-2005847] agreed concerning overlapping and non-overlapping carriers:

- Definition of overlapping and non-overlapping (sub-topic 1-2) (R4-2005847)

  - An overlapping carrier is a carrier which the UE is actively measuring for EMR and mobility.

  - A non-overlapping carrier is a carrier which the UE is actively measuring for EMR.

  - Note: overlapping and overlapping carrier relates to a carrier configured for EMR.

Additionally, RAN4 made agreements related to the conditions for actively measured [R4-2005847]:

- At least following conditions apply for an actively measured EMR carrier (R4-2005847):

  - T331 timer is running.

  - Serving cell is in the validity area.

  - Serving cell support EMR.

In Rel-15 following definition was introduced mainly to ensure minimum impact on the UE power consumption:

- An overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements.

- A non-overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements.

During RAN4#97 following proposals related to overlapping/non-overlapping carriers have been put forward:

Proposals:

- Qualcomm:

  - Proposal 1: Whether overlapping or non-overlapping EMR carrier is dynamically defined depending on the following attributes:

    - Whether T331 is running or not

    - Whether S-criteria condition is met or not

    - Whether EMR carrier is also configured as a measurement carrier for idle/inactive mode or not, and if configured whether it is higher priority frequency layer than serving/camped cell or not

    - Based on the attributes, if the carrier is determined to be measured for EMR and cell reselection at a given condition, it is defined as overlapping EMR carrier

- MTK:

  - Proposal 3:

| Inter-frequency/Inter-RAT E-UTRAN layers configured for both re-selection and EMR | higher priority layers | equal/lower priority layers |
| --- | --- | --- |
| Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Overlapping | Non-overlapping |
| Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Overlapping | Overlapping |

  - Proposal 6: RAN4 to reuse the evaluating time for overlapping inter-freq. and inter-RAT EMR carriers as the evaluating time for non-overlapping inter-freq. and inter-RAT EMR carriers

- Huawei:

  - Proposal 3: RAN4 to not define overlapping and non-overlapping carriers.

- Nokia:

  - Proposal 1: An overlapping carrier is an actively measured carrier configured by higher layer for mobility and early measurement reporting. A non-overlapping carrier is defined as an actively measured carrier configured by higher layer for early measurement reporting while not configured for mobility. A carrier configured for early measurement reporting is actively measured provided T331 has not expired, the serving cell is supporting early measurement reporting and the serving cell is in the validity area.

  - Proposal 2: If Proposal 1 is not agreeable, the definitions of overlapping and non-overlapping carriers follow Rel-15 definition directly:

    - An overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements.

    - A non-overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements.

- ZTE:

  - Not to differentiate overlapping and non-overlapping EMR carriers

  - Proposal 1: The same measurement requirements for both overlapping and non-overlapping EMR carriers are specified.

- Ericsson:

  - An overlapping carrier is a carrier which the UE is actively measuring for EMR and mobility.

  - A non-overlapping carrier is a carrier which the UE is actively measuring for EMR.

Proposals 1, 4 and 6 target an actual definition of overlapping and non-overlapping carriers.

Proposals 2 and 5 target to have same measurement requirements for overlapping and non-overlapping carriers.

Proposal 3 suggest to not define overlapping and non-overlapping carriers.

From the contributions three main aspects which RAN4 group has to address:

- Is there a need for RAN4 define different UE idle mode CA measurement requirements depending on whether the EMR carrier is measured for mobility or not?

- Can the EMR carrier dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds?

- Is there a need for the definition of overlapping and non-overlapping carriers?

To progress the discussion – assuming proposed agreements are acceptable to companies – the open issues relates to whether there is need to differ UE idle mode CA measurements requirements between overlapping and non-overlapping carriers. Following table can be used as guide in the discussion:

| SnonIntraSearchP/Q configured |  |  |  | SnonIntraSearchP/Q not configured |
| --- | --- | --- | --- | --- |
| Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ |  | Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ |  | N/A |
| EMR measurement requirements when high Priority carrier not configured | EMR measurement requirements when high Priority carrier configured | EMR measurement requirements when high Priority carrier not configured | EMR measurement requirements when high Priority carrier configured | N/A |
| 4.2.2.2 (Kcarrier * Tdetect,NR_Inter, Kcarrier * Tmeasure,NR_Inter) | 4.2.2.2 (Kcarrier * Tdetect,NR_Inter, Kcarrier * Tmeasure,NR_Inter) | 4.2.2.x | 4.2.2.7 (Thigher_priority_search = (60 * Klayers), Klayers * Tmeasure,NR_Inter) | 4.2.2.2 (Kcarrier * Tdetect,NR_Inter, Kcarrier * Tmeasure,NR_Inter) |

### Sub-topic 2-1: need for RAN4 define different UE idle mode CA measurement requirements depending on whether the EMR carrier is measured for mobility or not

Sub-topic description: Is there a need for RAN4 define different UE idle mode CA measurement requirements depending on whether the EMR carrier is measured for mobility or not?

Is there a need for RAN4 define different UE idle mode CA measurement requirements depending on whether the EMR carrier is measured for mobility or not?

- Proposals

  - Option 1: Agree to have same UE idle mode CA measurement requirements for all configured EMR carriers. Hence, UE idle mode CA measurement requirements do not depend on whether the carrier is configured for mobility or not.

  - Option 2: Agree to have different UE idle mode CA measurement requirements for configured EMR carriers, dependent on whether the carrier is additionally configured for mobility or not.

    - For companies preferring this option: list exactly which requirements (overlapping and non-overlapping)

- Recommended WF (original)

  - Agree on option 1: RAN4 agree to have same UE idle mode CA measurement requirements for all configured EMR carriers. Hence, UE idle mode CA measurement requirements do not depend on whether the carrier is configured for mobility or not.

- Based on the comments received moderator propose following modified proposal for agreement:

  - UE idle mode CA measurement requirements do not depend on whether the carrier configured for idle mode CA measurements is configured for mobility or not.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support the Recommended WF, and to make it more accurate, we support it if measurement period requirements are concerned. For measurement capability requirements, we already agreed to have some separate requirements for number of inter-RAT carriers which are not configured for mobility. |
| MTK | Our understanding is that option 1 can only be agreed when “the number of non-overlapping inter-freq. and inter-RAT carriers are be counted and scaled up;” otherwise, the measurement accuracy can be guaranteed. |
| Apple | Recommended WF is agreeable provided the carriers with EMR only (non-overlapping) are also counted in scaling factor for all idle measurement delay requirements, including overlapping, non-overlapping and mobility only carriers. Otherwise, we have concern on power consumption and accuracy. |
| ZTE | We support the recommended WF. |
| Nokia | We can support the recommended WF. Our understanding is that this means that the idle mode CA measurement requirements will be the same irrespective if the carrier configured for early measurement reporting is also configured for mobility or not. Hence, Idle mode CA measurement requirements for overlapping and non-overlapping carriers would be the same. |
| Qualcomm | Share the same views as other companies. And we want to suggest removing “RAN4 agree to have same UE idle mode CA measurement requirements for all configured EMR carriers. Hence,” because it can be read that “the same requirements for the following cases”: - Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ - Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ |
| Ericsson | Agree with the recommended WF |

### Sub-topic 2-2: UE measurement requirements and scaling according to measured carriers

Sub-topic description: UE measurement requirements and scaling according to measured carriers.

Scaling of the UE idle mode CA measurement requirements depending on the actual number carriers being measured has been addressed in several contributions (R4-2015742, R4-2016573, R4-2014361).

Based on the discussions and earlier principles applied in RAN4 it is proposed by the moderator that the same carrier scaling principles are applied for the configured with UE idle mode CA measurement requirements.

- Proposals

  - Option 1: The earlier principles applied in RAN4 concerning UE requirements and carrier scaling principles are applied for the UE configured with UE idle mode CA measurement requirements.

  - Option 2: Other.

- Recommended WF (original)

  - Agree on option 1: RAN4 agree that the earlier principles applied in RAN4 concerning UE requirements and carrier scaling principles are applied for the UE configured with UE idle mode CA measurement requirements.

The actual scaling (e.g. how to decide the scaling factor) is FFS.

- Based on the comments from companies, moderator propose to agree on the following:

  - RAN4 agree to scale the UE Idle mode CA measurement requirements based on the actual measured target carrier, regardless if the carrier is configured for Idle mode CA measurements only, mobility only or both idle mode CA and mobility measurements.

  - A carrier shall only be counted once in the scaling if the carrier is configured for both idle mode CA measurement requirements and mobility measurements.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We support the Recommended WF, and in our view all the carriers UE measures for mobility and EMR should be accounted in the scaling factor of the measurement period requirements. |
| MTK | We do not understand what is “the same carrier scaling principles.” Clarification is needed. If it means that all the carriers UE measures for mobility and EMR should be accounted in the scaling factor of the measurement period requirements, the clarification should be added. We agree that all the carriers UE measures for mobility and EMR should be accounted in the scaling factor of the measurement period requirements. |
| Apple | We also think it is better to clarify “the same carrier scaling principles”. Our view is that if there is actual measurement on target carrier, regardless if it is EMR only, mobility only or both EMR and mobility measurement, that carrier should be counted in the scaling factor for all idle state measurement requirement. |
| ZTE | The scale factor for ‘overlapping carrier’ should be counted only once and scale factor for ‘non-overlapping carrier’ should be counted either. |
| Nokia | We can agree on the WF. Our view is that carrier scaling is applied according to carriers measured. Our understanding is the same as Apple (which seems also to be what Huawei, MTK and ZTE says). |
| Qualcomm | With the clarifications from companies, we support WF in principle. |
| Ericsson | Agree with MediaTek |

### Sub-topic 2-3: Can the EMR carrier dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds

Sub-topic description: Can the EMR carrier dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds

- Proposals

  - Option 1: A configured EMR carrier can dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds.

  - Option 2: A configured EMR carrier cannot dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds.

- Recommended WF

  - Needs more discussion and depends on Sub-topic 2-1. If RAN4 agrees to have same UE idle mode CA measurement requirements for all configured EMR carriers, there is no clear reason to discuss this topic further.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Technically, as commented by the moderator, if RAN4 agrees to have same UE idle mode CA measurement requirements for all configured EMR carriers (which is also our preference), there is no absolute need to define overlapping and non-overlapping carriers. In our view, the only usage of overlapping and non-overlapping carrier concept is to describe the number of carriers in different requirements. If overlapping and non-overlapping is used in defining the number of carriers for scaling the measurement period requirements, it is meaningful to define the concepts dynamically based on the UE conditions and the s-NonIntraSearch thresholds, e.g. an EMR carrier also configured for EP/LP mobility measurement is a non-overlapping carrier when UE is above the search threshold, and the same carrier is an overlapping carrier when UE is below the search threshold. If overlapping and non-overlapping is used in defining the number of carriers for measurement capability requirements, it is meaningful to define the concepts independent of UE conditions and the s-NonIntraSearch thresholds, e.g. an EMR carrier also configured for EP/LP mobility measurement is always an overlapping carrier. Of course, the number of carriers for scaling the measurement period requirements and measurement capability requirements can also be described without defining overlapping and non-overlapping carriers, and in this case we would not need to define overlapping and non-overlapping carrier concept. To us this is more wording issue, and we are open to all options on whether/how to define overlapping and non-overlapping, as long as the requirements are defined correctly. |
| MTK | We think the discussion also depends the conclusions in Sub-topic 2-4. - If option 1 in Sub-topic 2-4 is applied, then the EMR carrier will be dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds. That is to says, option 1 should be applied. - If option 2 in Sub-topic 2-4 is applied, we don’t have to discuss this issue |
| Apple | Depends on outcome of previous sub-topics. |
| ZTE | We share similar view that it related to possible agreements for previous issues. In general we don’t see much benefit to have requirements that make the EMR carrier depending on search threshold dynamically. |
| Nokia | We actually have concerns with this dynamic approach seen from a network point of view. Depending on Sub-topic 2-4 outcome, this makes it impossible for the network to configure according to the UE capability (number of non-overlapping carriers supported). As we see it this may have the side effect of making use of search thresholds in cells where EMR is used impossible (as also discussed in issue 1-2-4). The visibility to the Srxlev conditions on the UE side is of course velar while this is not the case on the network side. This complicates things. Prefer option 2. But one way to address this (and next sub-topic 2-4) could be to not define two carrier types (overlapping and non-overlapping carriers). In our view this could be done assuming RAN4 can agree that all carriers configured for early measurement reporting will have same measurement requirements (e.g. like defined in the table above). UE would measure the EMR accordingly and scaling (as discussed in sub-topic 2-2) would be applied based on the measured carriers (also according to the table above. Hence, Kcarrier or Klayer). As an example: - if SnonIntraSearchP/Q are configured and one or more high priority carriers are configured, if Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE will at least measure according to high priority carriers and EMR carriers according to section 4.2.2.7 (see table). Scaling is Klayers which would at least be high priority carriers and EMR carriers. - if SnonIntraSearchP/Q are configured and one or more high priority carriers are configured, if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE will measure according to section 4.2.2.4 table 4.2.2.4-1 (see table). Scaling is Kcarrier which would be carriers configured for mobility and EMR. In both cases, if a carrier is configured for EMR and mobility (including high priority carrier) it only counts as one carrier in the scaling. Reason for this is that only one measurement is performed on the carrier but used for two purposes. |
| Qualcomm | If it is not controversial, having some clear definition of overlapping and non-overlapping EMR carrier is preferable at least for technical discussion. And whether to capture the terminology in the spec can be determined later. |
| Ericsson | If we use this terminology in the spec, we need to define what it is. |

### Sub-topic 2-4: Is there a need for the definition of overlapping and non-overlapping carriers

Sub-topic description: Is there a need for the definition of overlapping and non-overlapping carriers

- Proposals

  - Option 1: Yes.

  - Option 2: No.

- Recommended WF:

  - Needs more discussion and depends on Sub-topic 2-1. If RAN4 agrees to have same UE idle mode CA measurement requirements for all configured EMR carriers, there is no clear reason for defining overlapping and non-overlapping carriers.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Please refer to our comments for sub-topic 2-3. |
| MTK | We prefer option 1. We can also compromise to option 2. However, if option 2 is applied, then we have to reverse the agreement for EMR measurement capability (there exists the measurement capability for non-overlapping EMR carrier). |
| Apple | Fundamental issue is to agree on actual UE measurement behaviour and corresponding requirements. If previous sub-topics can be resolved, we believe having such definitions can increase readability of our spec. |
| ZTE | To make UE implementation simple, it seems good not to differentiate overlapping and non-overlapping carrier. Furthermore from EMR measurement functionality perspective we don’t see any benefit to differentiate. |
| Nokia | We see option 2 feasible. Hence, RAN4 does not define overlapping and non-overlapping EMR carriers. Please refer to our discussion in sub-topic 2-3. |
| Qualcomm | We prefer Option 1. We share the same view as Apple “we believe having such definitions can increase readability of our spec”. |
| Ericsson | Option 1 |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2014362 | CR on TS38.133 for measurement capability of IDLE mode DC/CA measurement, MediaTek inc., Huawei, HiSilicon |
|  | Nokia: UE measurement capability, current definition is aligned with current phrasing such that UE should be able to measure at least 7 FDD and at least 7 TDD, with a maximum capability on the total number. We would need then to capture the requirement on the total number (which can be FDD or TDD and between 0 and 7 of each in any mix, but not more than 7 in total). 4.2.2.4: Needs update according to meeting discussion. 4.2.2.5: Needs update according to meeting discussion. 4.2.2.7: Needs update according to meeting discussion. 4.4: we propose to have separate section for inter-frequency and inter-RAT requirements. This would be aligned with the agreements in 36.133 in last meeting. Likely other updates are needed according to the discussion in the meeting. |
|  | Company B |
| R4-2015743 | CR on EMR requirements in 36.133, Huawei, HiSilicon, MediaTek |
|  | Nokia: 4.2.2: Needs update according to meeting discussion. 4.2.2.5.6: It is unclear how to account the carriers configured for EMR and non-HST. We would prefer this to clear. We can discuss further how to capture this which also depend on the ongoing discussion. 4.9.2.4: Needs to be updated according to discussion. We prefer to have the requirements in table format to make it clear. |
|  | Company B |
| R4-2015882 | CR on UE requirement for MR-DC early measurement reporting in 36.133, Nokia, Nokia Shanghai Bell |
|  | Huawei: The definition of overlapping and non-overlapping carrier should depend on outcome of sub-topic 2-3 and 2-4. We do not quite understand the change to 4.9.2.1. Shouldn’t the cell and SSB remain detectable during UE in Idle mode so that it can be measured without cell detection? For 4.9.2.4, it should depend on outcome of sub-topic 1-2, and it may be simpler to define EMR requirements by referring to high priority carrier measurement principle. The SSB index time TtimeIndex,NR_InterRAT should be added to the cell detection time to obtain the total measurement period, and the current wording looks like it is alone the measurement period. |
|  | MTK: we have following comments - [Number of carrier to be scaled up]: Kcarrier, Nlayer, should also consider the number of non-over lapping requirement; otherwise, the UE measurement accuracy can't be guaranteed. (May not provide enough samples) - [Definition of overlapping carrier]: It will violate the agreement in RAN4 #94b meeting. - [Requirement under different conditions]: It seems that below 2 sentence have conflict. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - The UE shall at least measure configured, inter-frequency carriers according to 4.2.2.7. - Otherwise if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ: - The UE shall follow measurement requirements according to 4.2.2.4 (according to Srxlev). - If search thresholds SnonIntraSearchP or SnonIntraSearchQ are configured and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and UE is configured with one or more high priority inter-frequency carriers: UE may measure configured carriers according to 4.2.2.4 |
| R4-2015883 | CR on UE requirement for MR-DC early measurement reporting in 38.133, Nokia, Nokia Shanghai Bell |
|  | Huawei: Is the highlighted part in Change 2 a typo (it should be non-overlapping)? “For a UE configured with early measurement reporting Nlayers is the combined total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information and overlapping carriers.” The scaling factor Kcarriers in 4.2.2.4 also needs to clarified similar as Change 2. The definition of overlapping and non-overlapping carrier should depend on outcome of sub-topic 2-3 and 2-4. We do not quite understand the change to 4.4.2.1. Shouldn’t the cell and SSB remain detectable during UE in Idle mode so that it can be measured without cell detection? For 4.4.2.2, it should depend on outcome of sub-topic 1-2, and it may be simpler to define EMR requirements by referring to high priority carrier measurement principle. The SSB index time TtimeIndex,NR_Inter should be added to the cell detection time to obtain the total measurement period, and the current wording looks like it is alone the measurement period. The second paragraph in 4.4.2.4 seems redundant. |
|  | MTK: Similar comments with the previous one |
| R4-2016388 | Updates in EMR requirements, Ericsson |
|  | Huawei: The definition of overlapping and non-overlapping carrier should depend on outcome of sub-topic 2-3 and 2-4. |
|  | MTK: We have to align the naming. Companies should discuss whether to use EMR in the spec (IDLE mode CA/DC is a naming applied in RAN2 spec) |
|  | Nokia: Agree that the definition of overlapping/non-overlapping (if even needed) depends on ongoing discussion. But agree that RAN4 should clearly capture the outcome. Naming alignment would be preferable. Maybe RAN4 should follow the naming in Rel-15 and use Idle mode CA measurements. |
| R4-2016389 | Updates in EMR requirements, Ericsson |
|  | Huawei: The definition of overlapping and non-overlapping carrier should depend on outcome of sub-topic 2-3 and 2-4. |
|  | MTK: We have to align the naming. Companies should discuss whether to use EMR in the spec (IDLE mode CA/DC is a naming applied in RAN2 spec) |
|  | Nokia: Same comments as for R4-2016388. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-2 | Issue 1-2-3: Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (high priority carrier configured) Tentative agreements: Outcome from GTW: Candidate agreement: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7. Send LS to RAN2 to inform on the agreement and RAN4 observations that the measurement duration can exceed the maximum configurable duration of T331 timer but there is no consensus in RAN4 whether the timer value needs to be increased. It is up to RAN2 whether and how to resolve the issue. Candidate options: - Option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7. - Option 2: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, follow other requirements. Recommendations for 2nd round: - Moderator Propose following compromise agreement: - UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7. - Send LS to RAN2 to inform on the agreement and RAN4 the observation that the UE measurement duration can exceed the current maximum configurable duration of T331 timer. |
| Sub-topic#1-2 | Issue 1-2-4: Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ (high priority carrier not configured) Candidate options: - Proposals - Option 1: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.7. - Option 2: UE measurement requirements for idle mode CA measurements, when SnonIntraSearchP/Q are configured, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the UE is configured with one or more higher priority carrier, at least follow requirements in section 4.2.2.4 table 4.2.2.4-1. Recommendations for 2nd round: More discussion needed. |
| Sub-topic#1-2 | Sub-topic 2-1: need for RAN4 define different UE idle mode CA measurement requirements depending on whether the EMR carrier is measured for mobility or not Candidate options: - Option 1: Agree to have same UE idle mode CA measurement requirements for all configured EMR carriers. Hence, UE idle mode CA measurement requirements do not depend on whether the carrier is configured for mobility or not. - Option 2: Agree to have different UE idle mode CA measurement requirements for configured EMR carriers, dependent on whether the carrier is additionally configured for mobility or not. - For companies preferring this option: list exactly which requirements (overlapping and non-overlapping) Recommendations for 2nd round: UE idle mode CA measurement requirements do not depend on whether the carrier configured for idle mode CA measurements is configured for mobility or not |
|  | Sub-topic 2-2: UE measurement requirements and scaling according to measured carriers Candidate options: - Option 1: The earlier principles applied in RAN4 concerning UE requirements and carrier scaling principles are applied for the UE configured with UE idle mode CA measurement requirements. - Option 2: Other. Recommendations for 2nd round: - RAN4 agree to scale the UE Idle mode CA measurement requirements based on the actual measured target carrier, regardless if the carrier is configured for Idle mode CA measurements only, mobility only or both idle mode CA and mobility measurements. - A carrier shall only be counted once in the scaling if the carrier is configured for both idle mode CA measurement requirements and mobility measurements. |
|  | Sub-topic 2-3: Can the EMR carrier dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds Candidate options: - Option 1: A configured EMR carrier can dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds. - Option 2: A configured EMR carrier cannot dynamically change between overlapping and non-overlapping based on the UE conditions and the s-NonIntraSearch thresholds. Recommendations for 2nd round: - Needs more discussion and depends on Sub-topic 2-1. If RAN4 agrees to have same UE idle mode CA measurement requirements for all configured EMR carriers, there is no clear reason to discuss this topic further. |
|  | 2.2.4 Sub-topic 2-4: Is there a need for the definition of overlapping and non-overlapping carriers Candidate options: - Option 1: Yes. - Option 2: No. Recommendations for 2nd round: Needs more discussion and depends on Sub-topic 2-1. If RAN4 agrees to have same UE idle mode CA measurement requirements for all configured EMR carriers, there is no clear reason for defining overlapping and non-overlapping carriers. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| New | WF on MR-DC RRM requirements for Idle mode CA measurements | Nokia, Nokia Shanghai Bell |
| New | LS on RAN4 agreements for MR-DC Idle mode CA measurements | ZTE |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2014362 | To be merged into revised R4-2015883 |
| R4-2015883 | To be revised |
| R4-2015743 | To be revised |
| R4-2015882 | To be merged into revised R4-2015743 |
| R4-2016388 | To be merged into revised R4-2015883 |
| R4-2016389 | To be merged into revised R4-2015743 |

## Discussion on 2nd round (if applicable)

Based on the progress of 1st round discussions:

- Timer T331 length.

- UE Idle mode CA measurement requirements and UE conditions

- Measurement requirement for UE who supports the beam level EMR reporting

- Cell detected status

### Sub-topic 2-10: Timer T331 length

Sub-topic description: Is there a need to ask RAN2 to increase the timer T331 length? And if agreed, which length?

In R4-2015587 the length of T331 is discussed. It is discussed that with 300s of T331, this is not enough if total number of EMR carriers and high priority carriers is larger than 5, which can be 7 at maximum based on UE capability.

Issue 2-10-1: Whether to increase candidate value for timer T331

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed.

| Company | Comments |
| --- | --- |
| XXX |  |

If RAN4 agrees to ask rAN2 to increase the T331 length – what would RAN4 see as being to upper bound of needed for T331?

Issue 2-10-2: Additional candidate value of timer T331 if to be increased

- Proposals

  - Option 1: additional candidate value for timer T331 is up to 1200s

  - Option 2: other

- Recommended WF

  - More discussion needed.

| Company | Comments |
| --- | --- |
| XXX |  |

### Sub-topic 2-11: UE Idle mode CA measurement requirements and UE conditions

Sub-topic description: Aspects related to UE Idle mode CA measurement requirements and UE conditions

Having idle mode CA measurement requirements based on UE conditions (Srxlev and s-NonIntraSearch) has been discussed in R4-2015881. The problem discussed is that if the measurement requirement depends on the UE conditions (Srxlev and s-NonIntraSearch), this could lead to uncertainty on network side related to the measurement performance and/or accuracy of the EMR measurements reported by the UE.

Issue 2-11-1: Discuss and agree if having the measurement requirement depending on the UE conditions (Srxlev and s-NonIntraSearch), would lead to uncertainty on network side e.g. related to the measurement performance and/or accuracy of the EMR measurements reported by the UE.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed.

| Company | Comments |
| --- | --- |
| XXX |  |

### Sub-topic 2-12: Measurement requirement for UE who supports the beam level EMR reporting

Sub-topic description: Measurement requirement for UE who supports the beam level EMR reporting

In last meeting there was agreement captured: Add in idle mode requirements section 4.4:

- additional time period in measurement requirement for UE who supports the beam level EMR reporting:

- TtimeIndex,NR_Interf = [X]* Tmeasure,NR_Interf,

  - where X is [3, 5].

Issue 2-12-1: Value of X.

- Proposals

  - Option 1: X=3

  - Option 2: X=5

- Recommended WF

  - More discussion needed.

| Company | Comments |
| --- | --- |
| XXX |  |

### Sub-topic 2-13: Cell detected status

Sub-topic description: Cell detected condition depending on future conditions

In R4-20115881 it is discussed that the cell detected condition depends on a condition in the future – when the connection is once again established. Problem raised is that the conditions for a detected cell in connected mode remaining detectable when entering Idle mode need to refer to the conditions at the time of the release.

Issue 2-13-1: RAN4 to discuss and agree if the cell detected status condition needs to be updated or clarified to reflect the cell detected state condition of the cell at the time of and during the actual connection release.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion needed.

| Company | Comments |
| --- | --- |
| XXX |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: Performance – accuracy requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20 15746 | Huawei, HiSilicon | Proposal 1: For LTE-NR inter-RAT EMR, the measurement accuracy requirements for NR inter-frequency EMR are reused. Proposal 2: RAN4 to define same measurement accuracy requirements for all NR EMR carriers. Proposal 3: Measurement accuracy for EMR is relaxed compared to existing connected mode requirements - For RSRP, accuracy is relaxed by 1.5dB and side condition is defined at -4dB - For RSRQ, accuracy is relaxed by 1.5dB Proposal 4: For all NR-LTE inter-RAT EMR carriers, reuse the existing LTE inter-frequency accuracy requirements for CA Idle mode measurements for overlapping carrier. |
| R4-2016017 | Ericsson | Moved to test Topic #4 related to test cases. |

Introduction to the discussion:

It is assumed that RAN4 will need to define accuracy requirements for MR-DC EMR for the newly defined core requirement cases, covering:

- 36.133:

  - NR inter-RAT:

    - FR1

    - FR2

- 38.133:

  - NR inter-frequency:

    - FR1

    - FR2

  - LTE inter-RAT

In earlier meetings RAN4 has agreed following:

Measurement accuracies:

- RAN4 agreed that for overlapping EMR carriers, the UE measurement accuracy requirements for carriers configured for EMR:

  - RAN4 to define relaxed NR measurement requirements for overlapping carrier compared to existing NR inter-frequency requirements in terms of SNR and accuracy

- Sub-topic#2-5: Measurement accuracy requirements for NR Inter-RAT EMR carrier:

  - For Measurement accuracy requirements for NR Inter-RAT EMR carrier same principles as in Issue 1-2-5-2 will be applied.

- Sub-topic#2-5: Measurement accuracy requirements for NR Inter-RAT EMR carrier:

  - For Measurement accuracy requirements for NR Inter-RAT EMR carrier same principles as in Issue 1-2-5-2 will be applied.

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Based on the input RAN4 will need to discuss agree following:

- Introduction of accuracy requirements for all MR-DC EMR cases introduced.

- Re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements.

- Measurement accuracy relaxation compared to existing connected mode requirements

### Sub-topic 3-1 Introduction of accuracy requirements for all MR-DC EMR cases introduced

Sub-topic description: Introduction of accuracy requirements for all MR-DC EMR cases introduced

Open issues and candidate options before e-meeting:

RAN4 will need to define accuracy requirements for MR-DC EMR for the newly defined core requirement cases, covering:

- 36.133:

  - NR inter-RAT:

    - FR1

    - FR2

- 38.133:

  - NR inter-frequency:

    - FR1

    - FR2

  - LTE inter-RAT

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Agree on option 1. RAN4 will define accuracy requirements for MR-DC EMR for the newly introduced core requirement cases listed above.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support the Recommended WF |
| MTK | Agree with recommended WF |
| Apple | Recommended WF is OK. |
| ZTE | We support the recommended WF |
| Nokia | Support WF. |
| Qualcomm | Agree with WF |
| Ericsson | Option 1 |

### Sub-topic 3-2 Re-use of existing connected mode accuracy requirements

Sub-topic description

Open issues and candidate options before e-meeting:

Discuss and agree whether to re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements

- Proposals

  - Option 1: Yes, RAN4 will re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements

  - Option 2: No, RAN4 will not re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements, but will develop new accuracy requirements.

- Recommended WF

  - Agree on option1 and RAN4 will re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | To clarify, RAN4 has agreed to define EMR accuracy requirements more relaxed compared to connected mode inter-frequency accuracy requirements. In this sense, we support to use the connected mode inter-frequency accuracy requirements as baseline and further discuss the relaxation as in sub-topic 3-3. |
| MTK | Agree with option 1. RAN4 can discuss the EMR idle mode CA measurement accuracy requirements based on existing connected mode accuracy requirements. |
| Apple | In general, we agree that existing connected mode accuracy requirements can be a starting point for further discussion. Note that it doesn’t mean we can apply exactly the same requirements. |
| ZTE | We support the recommended WF. |
| Nokia | Agree with the WF. We also agree with Huawei and RAN4 has earlier partly agreed the approach. And we assume such approach can be applied to all cases. We can then discuss further the actual relaxation. |
| Qualcomm | Share the same view as Huawei |
| Ericsson | Option 1 |

### Sub-topic 3-3 Measurement accuracy relaxation compared to existing connected mode requirements

Sub-topic description

Open issues and candidate options before e-meeting:

Discuss and agree the measurement accuracy relaxation compared to existing connected mode requirements

- Proposals

  - Option 1: Measurement accuracy for EMR is relaxed compared to existing connected mode requirements:

    - For RSRP, accuracy is relaxed by 1.5dB and side condition is defined at -4dB

    - For RSRQ, accuracy is relaxed by 1.5dB

- Recommended WF

  - As there is only input from one company in this meeting it is recommended to continue the discussion to allow other companies to provide input.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support option 1, which is aligned with LTE principles. |
| MTK | Agree with option-1. In E-UTRAN, the accuracy requirement in idle mode CA is 1.5 dB higher than that in connected mode. The same principles can be re-used in NR. |
| Apple | We support option 1. Similar methodology has been widely discussed and agreed in LTE requirement. It makes sense to apply same principle here in NR. |
| ZTE | We are fine with option 1. |
| Nokia | We can support using same principle as in Rel-15 (option 1). |
| Qualcomm | Support option 1. |
| Ericsson | Prefer recommended WF |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2015747 | draftCR to introduce accuracy requirements for EMR 38.133, Huawei, HiSilicon |
|  | Nokia: Update according to meeting discussion and whether we get overlapping and non-overlapping carriers defined or not. Need to be clear that these requirements are for Idle CA measurements (not connected mode as is the general coverage of the requirements in the section). Number should initially be in []. |
|  | Company B |
| R4-2015748 | draftCR to introduce accuracy requirements for EMR 36.133, Huawei, HiSilicon |
|  | Nokia: Update according to meeting discussion and whether we get overlapping and non-overlapping carriers defined or not. Reference to sub-clause is preferred. |
|  | Ericsson: prefer same requirements and the reference to the existing requirements. |
| R4-2016378 | Accuracy requirements for MR-DC EMR (36.133), Nokia Corporation |
|  | Huawei: the CR defines separate accuracy requirements for overlapping and non-overlapping carriers. If RAN4 agrees to define same core requirements for all EMR carriers, we do not see a need to define different accuracy requirements. |
|  | Ericsson: prefer same requirements and the reference to the existing requirements. |
| R4-2016386 | Accuracy requirements for MR-DC EMR (38.133), Nokia Corporation |
|  | Huawei: The exact numbers for accuracy needs to based on outcome of sub-topic 3-3. We do not think relative accuracy is needed considering the usage of the EMR reports. Also, relative accuracy is not defined in LTE euCA. |
|  | Ericsson: prefer same requirements and the reference to the existing requirements. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Sub-topic 3-1 Introduction of accuracy requirements for all MR-DC EMR cases introduced Tentative agreements: Agree on option 1. RAN4 will define accuracy requirements for MR-DC EMR for the newly introduced core requirement cases listed above Candidate options: RAN4 will need to define accuracy requirements for MR-DC EMR for the newly defined core requirement cases, covering: - 36.133: - NR inter-RAT: - FR1 - FR2 - 38.133: - NR inter-frequency: - FR1 - FR2 - LTE inter-RAT - Proposals - Option 1: Yes - Option 2: No Recommendations for 2nd round: All companies supported the recommended WF. Recommended to agree: Agree on option 1. RAN4 will define accuracy requirements for MR-DC EMR for the newly introduced core requirement cases listed above |
|  | Sub-topic 3-2 Re-use of existing connected mode accuracy requirements Tentative agreements: Agree on option1 and RAN4 will re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements Candidate options: - Option 1: Yes, RAN4 will re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements - Option 2: No, RAN4 will not re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements, but will develop new accuracy requirements. Recommendations for 2nd round: With the clarification regarding earlier RAN4 agreement all companies support option 1. Agree on the recommended WF with clarification: Agree on option1 and RAN4 will re-use of existing connected mode accuracy requirements as baseline for developing EMR idle mode CA measurement accuracy requirements RAN4 has agreed to define EMR accuracy requirements more relaxed compared to connected mode inter-frequency accuracy requirements |
|  | Sub-topic 3-3 Measurement accuracy relaxation compared to existing connected mode requirements Tentative agreements: As there is only input from one company in this meeting it is recommended to continue the discussion to allow other companies to provide input Candidate options: - Option 1: Measurement accuracy for EMR is relaxed compared to existing connected mode requirements: - For RSRP, accuracy is relaxed by 1.5dB and side condition is defined at -4dB - For RSRQ, accuracy is relaxed by 1.5dB Recommendations for 2nd round: Many companies support the option 1 but some companies prefer the recommended WF (tentative agreement). Moderator suggest: Agree on the principle of option 1. Leave number in [] in this meeting. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2015747 | to be revised |
| R4-2015748 | to be revised |
| R4-2016378 | to be revised |
| R4-2016386 | to be revised |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: Test cases for MD-DC EMR

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2016018 | Ericsson | Proposal 1: RAN4 develops test cases for MR-DC based on the test case list in Table 1. Proposal 2: Time plan for developing MR-DC test cases: - RAN4#97-e (November 2020): - Agree on high-level list for test cases. - Agree on work split between interested companies. - RAN4#98-e (January 2021): - Provide draft CRs for test cases. - RAN4#98bis-e (April 2021): - Provide final CRs for test cases. |
| R4-2015749 | Huawei, HiSilicon | Proposal 1: Define EMR measurement period and measurement accuracy tests for the following cases (totally 8 test cases): - Case 1: NR inter-frequency EMR, target cell is in FR1 and has been detected when UE is in connected mode, without beam level measurement - Case 2: NR inter-frequency EMR, target cell is in FR2 and has not been detected when UE is in connected mode, with beam level measurement - Case 3: LTE-NR inter-RAT EMR, target cell is in FR1 and has not been detected when UE is in connected mode, with beam level measurement - Case 4: LTE-NR inter-RAT EMR, target cell is in FR2 and has been detected when UE is in connected mode, without beam level measurement - In all the tests, the EMR carrier is also configured for mobility, and the serving cell is below the search threshold (meaning the EMR carrier is also actively measured for mobility) |
| R4-2015884 | Nokia, Nokia Shanghai Bell | - RAN4 to discuss and agree on a list of test cases for MR-DC EMR. - RAN4 introduces measurement performance test cases. 38.133: Cell detected state: Carrier; Frequency Range; Carrier type. inter-frequency; FR1; non-overlapping. inter-frequency; FR2; overlapping. inter-RAT; N/A; overlapping. EMR: Carrier; Frequency Range; Carrier type; Cell type; SSB Measurements. inter-frequency; FR1; overlapping; new cell; Yes. inter-frequency; FR1; non-overlapping; known cell; No. inter-frequency; FR2; overlapping; known cell; No. inter-frequency; FR2; non-overlapping; known cell; Yes. inter-frequency; FR2; overlapping; new cell; Yes. inter-frequency; FR2; non-overlapping; new cell; No. Carrier; Frequency Range; Carrier type; Cell type; SSB Measurements. Inter-RAT; LTE; overlapping; known cell; N/A. Inter-RAT; LTE; non-overlapping; new cell; N/A. 36.133: cell detected state: Carrier; Frequency Range; Carrier type. inter-RAT; FR1; overlapping. inter-RAT; FR2; non-overlapping. EMR: Carrier; Frequency Range; Carrier type; Cell type; SSB Measurements. Inter-RAT; FR1; non-overlapping; new cell; Yes. Inter-RAT; FR1; overlapping; known cell; No. Inter-RAT; FR2; non-overlapping; known cell; No. Inter-RAT; FR2; overlapping; known cell; Yes. Inter-RAT; FR2; non-overlapping; new cell; Yes. Inter-RAT; FR2; overlapping; new cell; No. |
| R4-2016017 | Ericsson | Introduce following test case for EMR: 38.133: - Idle Mode measurements of inter-frequency CA/DC candidate cells in FR1 for early reporting - Idle Mode measurements of inter-frequency CA/DC candidate cells in FR2 for early reporting - Idle Mode measurements of inter-RAT EUTRA DC candidate cells for early reporting 36.133: - Idle Mode measurements of inter-RAT NR DC candidate cells for early reporting |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Based on the input RAN4 will need to discuss agree following:

- Time plan.

- Test case list.

### Sub-topic 4-1 Time plan

Time plan for developing MR-DC test cases:

- RAN4#97-e (November 2020):

  - Agree on high-level list for test cases.

  - Agree on work split between interested companies.

- RAN4#98-e (January 2021):

  - Provide draft CRs for test cases.

- RAN4#98bis-e (April 2021):

  - Provide final CRs for test cases

- Proposals

  - Option 1: Agree the time plan.

  - Option 2: N/A

- Recommended WF:

  - Agree on the time plan

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support the Recommended WF |
| MTK | Agree with the time plan |
| Apple | Recommended WF is OK. |
| ZTE | Recommended WF is fine. |
| Nokia | Support the WF and the time plan |
| Qualcomm | Support WF |
| Ericsson | Even though the proposed work plan looks realistic, the current WI completion day is in March 2021, so we need to adjust the time plan accordingly. |

### Sub-topic 4-2 Test case list

- Proposals

  - Option 1: Use following as template baseline for discussing test cases:

| Serving Carrier | Target Carrier | Target cell Frequency Range | Target Carrier type (mobility or not carrier) | Target Cell type | Beam level Measurements | s-NonIntraSearch thresholds |
| --- | --- | --- | --- | --- | --- | --- |
| LTE, NR (FR1, FR2) | LTE, NR | FR1 or FR2 | overlapping or not (pending discussion) | New cell or detected cell | yes/no | yes/no |

  - Option 2:

- Recommended WF:

  - Agree on using the template table for collecting initial views. More discussion is needed concerning which test cases to define.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support the Recommended WF. We will provide our views for TC list separately based on moderator’s instruction. |
| MTK | As we mentioned in Sub-topic 1-1, there exist default values for SnonIntraSearchP/Q. We think this test can be simplified by assuming that SnonIntraSearchP/Q are configured. |
| ZTE | Not sure if we need to agree on something for the template. It seems like just a possible way to discuss test cases. |
| Nokia | We support the WF. Detailed input on the test case will be provided once template is shared. |
| Qualcomm | Okay with Option 1 and WF |
| Ericsson | Ok with the WF |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| xxxx | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 | Sub-topic 4-1 Time plan Tentative agreements: Agree on the time plan according to the current WI completion day is in March 2021. Adjust the time plan accordingly Candidate options: Recommendations for 2nd round: Agree on the following time plan: Time plan for developing MR-DC test cases: - RAN4#97-e (November 2020): - Agree on high-level list for test cases. - Agree on work split between interested companies. - RAN4#98-e (January 2021): - Provide draft CRs for test cases. - Provide big CR by collecting draft CRs. Updated time plan will be captured in WF for test cases |
| Sub-topic#4-2 | Sub-topic 4-2 Test case list Candidate options: Recommendations for 2nd round: Agree on using the template table for collecting initial views. More discussion is needed concerning which test cases to define |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| New | WF on Test cases for MR-DC Idle mode CA measurements | Nokia, Nokia Shanghai Bell |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

### Sub-topic 4-10: Test Case List

Sub-topic description: Companies to fill in the test case list as shared on the server. Discuss and possibly select the test cases to be specified.

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |