TSG-RAN WG4 Meeting # 97-e 				R4-2017089

# Electronic Meeting, 2nd – 13th November, 2020

Agenda item:			7.2.1 and 7.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	WF on NR-U RRM performance

This document contains agreements and discussions in RAN4#97-e in e-mail thread: [97e][206]NR_unlic_RRM_2. The following color code is used:

Agreements in the GTW session

Agreements in the 1st round

Agreements in the 2nd round

# Topic #1: Measurement Accuracy

## Sub-topic 1-1: RSSI & Channel occupancy

Issue 1-1-1: RSSI measurement bandwidth

Agreement: RSSI Measurement Bandwidth is the bandwidth defined in TS 38.215 RSSI measurement definition (i.e. “the measurement bandwidth corresponding to the channel bandwidth defined in Clause 4 of TS 37.213 [17]”)

Issue 1-1-2: RSSI measurement accuracy

Agreement: Define RSSI measurement accuracy requirements in NR-U to be the same as in LTE-LAA.

# Topic #2: NR-U RRM test configuration

## Sub-topic 2-1: Wideband operation

Issue 2-1-1: Whether to test wideband operation in RRM tests

Agreement: It is assumed DL wideband operation Mode 1 is used during RRM tests for NR-U.

Note: DL wideband carrier operation Mode 1 is based on UE single-carrier wideband operation where LBT in the wideband is assumed to be successful when LBT is successful in all LBT sub-bands within the UE DL BWP.

## Sub-Topic 2-2: LBT model during RRM tests

Issue 2-2-1: Differentiation between FBE and LBE

Agreement: Further identify the set of requirements for which LBE and FBE test cases shall be differentiated.

Issue 2-2-2: DL LBT model for LBE operation

FFS: RAN4 to discuss a DL LBT model for LBE channel access.

Options discussed in RAN4 97e:

Option 1: For LBE test cases in non DRX: RAN4 to adopt the following DL LBT model: 1) Define a probability equal to P1  for the transmission of the DRS in the first candidate position. 2) In case of LBT failure for transmission in the first candidate position, define a probability equal to P2 for the transmission in the second candidate position for a given SSB index.

•	FFS: the value of P1 and P2, or if P1 = P2.

•	FFS: how to treat the different UE behaviours in the test cases, depending on whether Lmax values are exceeded (this might need to be discussed on a case by case approach).

Option 2:  DL LBT model, in  LBE non-DRX test cases: adopt the DL LTE LBT modelling approach as baseline

•	FFS: The probability value, P, considered for the transmission of SSBs in different candidate positions

•	FFS: how to treat the different UE behaviours in the test cases, depending on whether Lmax values are exceeded (this might need to be discussed on a case by case approach).

Other options are not precluded.

Issue 2-2-3: DL LBT model for FBE operation

FFS: RAN4 to discuss a DL LBT model for FBE channel access.

Options discussed in RAN4 97e:

Option 1: DL LBT model, in FBE non-DRX test cases: RAN4 to define a DL LBT model that considers a probability of P for the transmission of each DRS. Only the first SSB candidate position for a given SSB index shall be considered in these tests.

- FFS: The probability value, P, for the transmission of SSBs

- FFS: how to treat the different UE behaviours in the test cases, depending on whether Lmax values are exceeded (this might need to be discussed on a case by case approach).

Option 2: DL LBT model, in FBE non-DRX test cases: adopt the DL LTE LBT modelling approach as baseline, considering the fact that there is only 1 candidate position.

- FFS: The probability value, P, for the transmission of SSBs

- FFS: how to treat the different UE behaviours in the test cases, depending on whether Lmax values are exceeded (this might need to be discussed on a case by case approach).

Other options are not precluded.

Issue 2-2-4: DL LBT model when DRX is in use

FFS: RAN4 to discuss the DL LBT model when DRX is in use after the definition of the model for non-DRX cases.

Issue 2-2-5: Exceeding Lmax values during RRM tests

Candidate options discussed in RAN4 97e:

Option 1: For RRM test cases for NR-U, exceeding Lmax should be avoided

Option 2: Companies are encouraged to bring a list of requirements that would trigger different behaviours depending on whether the Lmax is exceeded or not. Discuss for each of those requirements:

•	whether they can be tested without exceeding Lmax.

•	How to design the test case to capture the different behaviors if needed.

Issue 2-2-6: Consecutive DL LBT failures during cell-reselection test cases

Options discussed in RAN4 97e:

- Option 1:  For the cell-reselection test cases, Mp consecutive DRX cycles with LBT failures of the serving cell should be avoided.

- Option 2:  For the cell-reselection test cases, Mp consecutive DRX cycles with LBT failures of the serving cell should be also tested.

Issue 2-2-7: UL LBT model

Agreement: RAN4 to discuss a methodology to test UL LBT failures in RRM tests.

FFS: Should RAN4 choose one typical test case to check this functionality?

•	Option 1: Yes, RAN4 can choose one typical test case to check this functionality.

•	Option 2: No, the UL LBT functionality should be tested in all requirements that depend on UL LBT failures.

•	Option 3: Companies are encouraged to bring a list of requirements that would be impacted by UL LBT failures, so that the group can decide how to test this functionality.

## Sub-topic 2-3: Number of Cells and Frequency range

Issue 2-3-1: Frequency range

Agreement: NR cells in NR-U test cases (e.g., for HO or in scenario A or for measurements) are always in FR1.

Issue 2-3-2: Number of cells

# Topic #3: NR-U RRM test cases

## Sub-topic 3-1: Specification Structure

Issue 3-1-1: Specification structure for test cases

Agreement:

  - Create in TS 38.133 the following new top-level sections for NR-test cases:

    - A.9	NR standalone tests with SCell under CCA and PCell in FR1

    - A.10 	EN-DC tests with NR PSCell under CCA

    - A.11 	NR-U standalone tests with NR PCell under CCA (note: including also NR/E-UTRA measurements and including re-selection in IDLE and HO from NR-U to NR-U/NR/E-UTRA cells and from NR-U/NR to NR-U cells)

    - A.12	 E-UTRA standalone tests with NR-U cells

      - Inter-RAT E-UTRA–NR-U cell re-selection with NR-U target cell

      - Inter-RAT E-UTRA–NR-U HO with NR-U target cell

      - Inter-RAT E-UTRA–NR-U measurements

      - Inter-RAT SFTD with NR-U neighbor cell

Issue 3-1-2: Specification structure for common Configuration Parameters

  - Develop new sections for common test parameters in NR-U RRM test cases according to the table.

| New section | Title |
| --- | --- |
| A.3.1.* | … under CCA |
| A.3.2.3 | Generic OFDMA Channel Noise Generator (OCNG) under CCA |
| A.3.7B | EN-DC test setup with PSCell under CCA |
| A..3.8.4 | PRACH configuration under CCA |
| A.3.10A | SSB configurations under CCA |
| A.3.16A | TCI state configurations under CCA |
| A.3.19 | Discovery Burst Transmission Window configuration under CCA |
| A.3.20 | Signal transmission model under CCA |
| NOTE: “*” denotes different relevant sub sections |  |

## Sub-topic 3-2: RRM tests scope and applicability rules

Issue 3-2-2: RRM tests scope – legacy test cases for SA NR-U

Agreement:

Define legacy test cases for SA NR-U

- Applicability rules are FFS. The test cases will apply at least for UEs supporting SA NR-U only.

- Legacy test cases correspond to requirements which are common to NR and NR-U

Issue 3-2-3: RRM tests scope – NR-U scenarios to be covered by NR-U test cases

Issue 3-2-4: Applicability rules

  - RAN4 will discuss applicability rules when test cases have sufficiently progressed, e.g.:

    - FFS: for a UE capable of multiple scenarios, the UE shall pass the test to verify the same requirements on the same type of cell (e.g. UE timing accuracy) in only one scenario.

    - FFS: for UEs supporting both LBE and FBE.

## Sub-topic 3-3: Test case list

Issue 3-3-1: Test cases overview.

The table discussed in this issue is copied in Topic 4 in this document.

Issue 3-3-2: Cell Reselection Tests

For Cell reselection, at least the following test cases are to be specified in NR-U

- Cell reselection to FR1 intra-frequency NR when CCA is used on the serving and target cell

- Cell reselection to FR1 inter-frequency NR when CCA is used on the serving and target cell

- Cell reselection from E-UTRAN (FDD and TDD) to FR1 when CCA is used on the target cell

Other cases discussed in RAN4 97e:

- Cell reselection from NR-U cell to NR (NR-U -> NR(FR1)

- Cell reselection from NR cell to NR-U (NR(FR1) -> NR-U)

- Cell reselection from NR-U cell to E-UTRAN (NR-U - > E-UTRAN (FDD,TDD))

Issue 3-3-3: Handover (delay and interruption) test cases

For Handover, at least the following test cases are to be specified in NR-U

- Intra-frequency handover from NR-U to NR-U; known target cell

- Intra-frequency handover from NR-U to NR-U; unknown target cell

- Inter-frequency handover from NR-U to NR-U; unknown target cell

- Inter-RAT handover from E-UTRAN (FDD and TDD) to NR-U

Other cases discussed in RAN4 97e:

5.	Inter-frequency handover from NR-U to NR-U; known target cell

6.	Inter-frequency handover from NR to NR-U; known target cell

7.	Inter-frequency handover from NR to NR-U; unknown target cell

8.	Inter-RAT handover from NR-U to E-UTRAN (FDD, TDD)

9.	Inter-frequency handover from NR-U to NR; known target cell

10.	Inter-frequency handover from NR-U to NR; unknown target cell

Issue 3-3-4: RRC Re-establishment test cases

RAN4 to specify RRC Re-establishment test cases in NR-U.

Issue 3-3-5: Random access

RAN4 to discuss the random access tests after the random access core requirements are discussed.

Issue 3-3-6: RRC Connection Release with redirection.

RAN4 to specify at least the following test case:

Redirection from NR in FR1 to NR in FR1 with CCA

Other cases discussed in RAN4 97e:

Redirection from NR in FR1 with CCA to NR in FR1 with CCA

Issue 3-3-7: Timing

RAN4 to specify at least the following test cases:

1.	UE transmit timing with NR-U PCell

2.	UE transmit timing with NR-U PSCell

Other cases discussed in RAN4 97e:

Timing advance test with NR-U PCell and NR-U PSCell

Issue 3-3-8: BWP Switching delay

For BWP Switching delay, RAN4 to specify at least the following test cases:

1.	E-UTRAN – NR-U PSCell UL active BWP switch based on persistent UL LBT failure

2.	NR-U – NR-U PCell UL active BWP switch based on persistent UL LBT failure

Other cases discussed in RAN4 97e:

- DCI/timer/RRC-based BWP switching on NR-U SCell, when configured with:

  - NR PCC (PCC)

  - NR-U PCC

  - NR-U PSCC and E-UTRAN PCC (FDD, TDD)

Issue 3-3-9: Radio Link Monitoring

For RLM, RAN4 to specify at least the following test case:

- Radio Link Monitoring Out-of-sync Test for NR-U PSCell configured with SSB-based RLM RS in non-DRX mode (E-UTRA FDD and TDD)

- Radio Link Monitoring In-sync Test for NR-U PSCell configured with SSB-based RLM RS in non-DRX mode (E-UTRA FDD and TDD)

- Radio Link Monitoring Out-of-sync Test for NR-U PSCell configured with SSB-based RLM RS in DRX mode(E-UTRA  FDD and TDD)

- Radio Link Monitoring In-sync Test for NR-U PSCell configured with SSB-based RLM RS in DRX mode (E-UTRA FDD and TDD)

- Radio Link Monitoring Out-of-sync Test for NR-U PCell configured with SSB-based RLM RS in non-DRX mode

- Radio Link Monitoring In-sync Test for NR-U PCell configured with SSB-based RLM RS in non-DRX mode

- Radio Link Monitoring Out-of-sync Test for NR-U PCell configured with SSB-based RLM RS in DRX mode

- Radio Link Monitoring In-sync Test for NR-U PCell configured with SSB-based RLM RS in DRX mode

Other discussions in  RAN4 97e:

- How to take into account the different SINR side conditions in the OOS cases

Issue 3-3-10: Beam management

RAN4 to specify the following test cases:

- EN-DC Beam Failure Detection and Link Recovery Test for NR-U PSCell configured with SSB-based BFD and LR in non-DRX mode (E-UTRA FDD and TDD)

- EN-DC Beam Failure Detection and Link Recovery Test for NR-U PSCell configured with SSB-based BFD and LR in DRX mode (E-UTRA FDD and TDD)

- Beam Failure Detection and Link Recovery Test for NR-U PCell configured with SSB-based BFD and LR in non-DRX mode

- Beam Failure Detection and Link Recovery Test for NR-U PCell configured with SSB-based BFD and LR in DRX mode

Issue 3-3-11: SCell Activation and Deactivation delay

RAN4 to specify the following test cases for NR PCC, NR-U PCC and E-UTRAN PCC, NR-U PSCC:

- SCell Activation and deactivation of known SCell in NR-U

FFS: the exact measurement cycle (or cycles).

- SCell Activation and deactivation of unknown SCell in NR-U

Issue 3-3-12: PSCell addition/release delay

RAN4 to specify the following test cases:

PSCell addition/ release delay for known NR-U PSCell (E-UTRA FDD and TDD)

Other options discussed in RAN4 97e:

Whether to specify tests for unknown case.

Issue 3-3-13: Active TCI state switching delay

FFS: RAN4 to discuss the following test cases:

E-UTRAN – NR PSCell in FR1 with CCA active TCI state switch for a known TCI state (MAC CE based)

E-UTRAN – NR PSCell in FR1 with CCA active TCI state switch for a known TCI state (RRC  based)

NR PCell FR1 with CCA active TCI state switch for a known TCI state (MAC CE based)

NR PCell FR1 with CCA active TCI state switch for a known TCI state (RRC based)

NR SCell FR1 with CCA (when configured with NR PCell) active TCI state switch for a known TCI state (MAC CE based)

NR SCell FR1 with CCA (when configured with NR PCell) active TCI state switch for a known TCI state (RRC based)

Issue 3-3-14: Interruptions

Issue 3-3-15: Intra Frequency measurement procedure

- RAN4 to define at least the following test cases for the measurement procedures requirements:

  - Intra-frequency SS-RSRP measurements on:

  - NR-U SCC, with NR PCC (FR1)

  - NR-U PCC

  - NR-U SCC, with NR-U PCC

  - NR-U PSCC, with E-UTRAN PCC (FDD,TDD)

  - NR-U SCC measurements, with E-UTRAN PCC (FDD,TDD) and NR-U PSCC

  - L1-RSRP measurements on:

  - NR-U SCC, with NR PCC (FR1)

  - NR-U PCC

  - NR-U SCC, with NR-U PCC

  - NR-U PSCC, with E-UTRAN PCC (FDD,TDD)

  - NR-U SCC measurements, with E-UTRAN PCC (FDD,TDD) and NR-U PSCC

  - Intra-frequency RSSI and CO measurements on:

  - NR-U SCC, with NR PCC (FR1)

  - NR-U PCC

  - NR-U SCC, with NR-U PCC

  - NR-U PSCC, with E-UTRAN PCC (FDD,TDD)

  - NR-U SCC measurements, with E-UTRAN PCC (FDD,TDD) and NR-U PSCC

  - Intra-frequency SS-RSRQ and SS-SINR measurements on

    - NR-U SCC, with NR PCC (FR1)

    - NR-U PCC

    - NR-U SCC, with NR-U PCC

    - NR-U PSCC, with E-UTRAN PCC (FDD,TDD)

    - NR-U SCC measurements, with E-UTRAN PCC (FDD,TDD) and NR-U PSCC

Issue 3-3-16: Inter Frequency measurement procedure

- RAN4 to define at least the following test cases:

- Inter-frequency SS-RSRP measurements on:

•        NR-U inter-frequency, with NR PCC (FR1)

•        NR-U inter-frequency, with NR-U PCC

•        NR-U inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD)

•        NR (FR1) inter-frequency, with NR-U PCC

•        NR (FR1) inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD)

- Inter-frequency RSSI and CO measurements on:

•        NR-U inter-frequency, with NR PCC (FR1)

•        NR-U inter-frequency, with NR-U PCC

•        NR-U inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD)

Other options discussed in RAN4 97e: also specify tests SS-SINR and SS-RSRQ measurements

Issue 3-3-17: Inter RAT measurement procedure

RAN4 to define at least the following test cases:

- Inter-RAT SFTD between:

  - E-UTRAN PCell (FDD,TDD) and NR-U neighbour

- E-UTRA-NR-U SS-RSRP/SS-RSRQ/SS-SINR

  - On NR-U neighbor, with E-UTRA (FDD,TDD) PCC

  - On NR-U neighbor, with E-UTRA (FDD,TDD) PCC and NR-U PSCC

Regarding Inter-RAT SFTD between E-UTRAN PCell (FDD,TDD) and NR-U neighbor, the UE should be capable for NR-U PCell/NR-U PSCell

Other tests discussed in RAN4-97e:

- NR-U-E-UTRA RSRP/RSRQ (needed for HO):

•        On E-UTRA (FDD,TDD), with NR-U PCC

•        On E-UTRA (FDD,TDD), with NR-U PSCC

Issue 3-3-18: Measurement accuracy tests

RAN4 to define at least the following test cases:

Intra-frequency absolute and relative accuracies for SS-RSRP on:

•        NR-U SCC, with NR PCC (FR1)

•        NR-U PCC

•        NR-U SCC, with NR-U PCC

•        NR-U PSCC, with E-UTRAN PCC (FDD,TDD)

•        NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD)

FFS: SS-SINR and SS-RSRQ

Absolute and relative accuracies for L1-RSRP on:

•        NR-U SCC, with NR PCC (FR1)

•        NR-U PCC

•        NR-U SCC, with NR-U PCC

•        NR-U PSCC, with E-UTRAN PCC (FDD,TDD)

•        NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD)

Accuracy for NR-U Intra-frequency RSSI and CO measurements on:

•        NR-U SCC, with NR PCC (FR1)

•        NR-U PCC

•        NR-U SCC, with NR-U PCC

•        NR-U PSCC, with E-UTRAN PCC (FDD,TDD)

•        NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD)

Inter-frequency absolute and relative accuracies for SS-RSRP on:

•        NR-U neighbor, with NR PCC (FR1)

•        NR-U neighbor, with NR-U PCC

•        NR-U neighbor, with NR-U PSCC, with E-UTRAN PCC (FDD,TDD)

FFS:  SS-RSRQ and SS-SINR

Accuracy for NR-U Inter-frequency RSSI measurements and CO on:

•        NR-U neighbor, with NR PCC (FR1)

•        NR-U neighbor, with NR-U PCC

•        NR-U neighbor, with NR-U PSCC, with E-UTRAN PCC (FDD,TDD)

Accuracy for NR-U Inter-RAT SFTD measurements between:

•        E-UTRAN PCell (FDD,TDD) and NR-U neighbor

FFS: NOTE: under the condition of stationary paths

Accuracy for NR-U for E-UTRA-NR-U SS-RSRP measurements

•        On NR-U neighbor, with E-UTRA (FDD,TDD) PCC

•        On NR-U neighbor, with E-UTRA (FDD,TDD) PCC and NR-U PSCC

FFS: SS-SINR and SS-RSRQ

## Sub-topic 3-4: Work Plan

Issue 3-4-1: Division of the work in 2 phases

RAN4 agrees to divide the work in 2-phases.

Issue 3-4-2: Work Plan

  - Conclusion: align the work plan with current WI timelines. Further update in case the extension is granted.

# Test case list and Work Split

| Group of requirements | Test cases |  | Requirements section | Agreed | Volunteer | Phase I | Phase II |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | NR-U -> NR-U | intra-frequency | 4.2A | Yes | Ericsson |  |  |
|  |  | inter-frequency |  | Yes |  |  |  |
|  | NR(FR1) -> NR-U |  |  | FFS |  |  |  |
|  | NR-U -> NR(FR1) |  |  | FFS |  |  |  |
|  | NR-U - > E-UTRAN (FDD,TDD) |  |  | FFS |  |  |  |
|  | E-UTRAN (FDD,TDD) -> NR-U |  | TS 36.133 | Yes |  |  |  |
| RRC_INACTIVE, cell re-selection | Not needed |  | 5.1A | Not needed |  |  |  |
| HO (delay and interruptions) | NR-U-> NR-U | intra-frequency, known | 6.1B | Yes | Huawei |  |  |
|  |  | intra-frequency, unknown |  | Yes |  |  |  |
|  |  | inter-frequency, unkown |  | Yes |  |  |  |
|  |  | Inter-frequency, known |  | FFS |  |  |  |
|  | NR(FR1) -> NR-U | known | 6.1B | FFS |  |  |  |
|  |  | unkown |  | FFS |  |  |  |
|  | NR-U -> NR(FR1) | known |  | FFS |  |  |  |
|  |  | unknown | 6.1.1.2 | FFS |  |  |  |
|  | NR-U - > E-UTRAN (FDD,TDD) |  | 6.1.2.1 | FFS |  |  |  |
|  | E-UTRAN (FDD,TDD) -> NR-U |  | TS 36.133 | Yes | Nokia |  |  |
| RRC Re-establishment | NR-U-> NR-U |  | 6.2.1A | Yes | Nokia |  |  |
|  | NR(FR1) -> NR-U |  |  | FFS |  |  |  |
| Random access | (requirements not available yet, being discussed in thread 205) |  | 6.2.2A [1] | Depends on CORE requirements |  |  |  |
|  | Contention-based and non-contention based RA: |  |  |  |  |  |  |
|  |  to NR-U PCell |  |  |  |  |  |  |
|  |  to NR-U PSCell |  |  |  |  |  |  |
| RRC Connection Release with Redirection |  NR-U-> NR-U |  | 6.2.3.2.3 | Yes | Huawei |  |  |
|  |  NR(FR1) -> NR-U |  |  | FFS |  |  |  |
| Timing (transmit timing) |  NR-U PCell |  | 7.1, 7.3 | Yes | MTK |  |  |
|  |  NR-U PSCell |  |  | Yes |  |  |  |
| Timing (timing advance) |  NR-U PCell |  |  | FFS |  |  |  |
|  |  NR-U PSCell |  |  | FFS |  |  |  |
| BWP switching delay and interruptions |  E-UTRAN – NR-U PSCell UL active BWP switch based on persistent UL LBT failure |  | 8.6 | Yes | Ericsson |  |  |
|  |  NR-U – NR-U PCell UL active BWP switch based on persistent UL LBT failure |  |  | Yes |  |  |  |
|  | Legacy DCI/timer/RRC-based BWP switching on NR-U SCell, with: |  |  |  |  |  |  |
|  |  NR PCC (PCC) |  |  | FFS |  |  |  |
|  |  NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U PSCC and E-UTRAN PCC (FDD, TDD) |  |  | FFS |  |  |  |
| RLM (in-syn and out-of-sync) |  On NR-U PSCC, with E-UTRAN PCC (FDD,TDD) | OOS, non-DRX | 8.1A | Yes | Ericsson |  |  |
|  |  | IS, non-DRX |  | Yes |  |  |  |
|  |  | OOS, DRX |  | Yes |  |  |  |
|  |  | IS, DRX |  | Yes |  |  |  |
|  |  On NR-U PCC | OOS, non-DRX |  | Yes |  |  |  |
|  |  | IS, non-DRX |  | Yes |  |  |  |
|  |  | OOS, DRX |  | Yes |  |  |  |
|  |  | IS, DRX |  | Yes |  |  |  |
| BM |  On NR-U PCC |  | 8.5A | Yes | Ericsson |  |  |
|  |  On NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
| SCell activation/deactivation delay |  NR PCC (FR1) | known | 8.3A | Yes |  |  |  |
|  |  | unknown |  | Yes |  |  |  |
|  |  NR-U PCC | known |  | Yes |  |  |  |
|  |  | unknown |  | Yes |  |  |  |
|  |  NR-U PSCC and E-UTRAN PCC (FDD, TDD) | known |  | Yes |  |  |  |
|  |  | unknown |  | Yes |  |  |  |
| PSCell addition/release delay | NR-U PSCell with E-UTRA PCC | konwn | TS 36.133 | Yes | Huawei |  |  |
|  |  | unknown |  | FFS |  |  |  |
| Active TCI state switching delay | For known and unknown target TCI state in NR-U, on: |  | 8.10A |  |  |  |  |
|  |  NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | FFS |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD, TDD) |  |  | FFS |  |  |  |
| Interruptions | Due to NR-U SCell addition/release, with: |  | 8.2.1, 8.2.2 |  | Ericsson |  |  |
|  |  NR PCC (FR1) |  |  | FFS |  |  |  |
|  |  NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  | Due to NR-U SCell activation/deactivation, with: |  | 8.2.1, 8.2.2 |  |  |  |  |
|  |  NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  | During measurements no deactivated NR-U SCell, with: |  | 8.2.1, 8.2.2 |  |  |  |  |
|  |  NR PCC (FR1) |  |  | FFS |  |  |  |
|  |  NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  | Due to inter-RAT SFTD measurements between: |  | TS 36.133 |  |  |  |  |
|  |  NR-U PCell and E-UTRAN PCell (FDD,TDD) |  |  | Yes |  |  |  |
|  | Due to NR-U PSCell addition/release, with: |  | TS 36.133 | Yes |  |  |  |
|  |  E-UTRA PCell |  |  | Yes |  |  |  |
| Intra-frequency measurement procedure (SS-RSRP, SS-RSRQ, SS-SINR, L1-RSRP, RSSI, CO) | Intra-frequency SS-RSRP, measurements on: |  | 9.2A.5, 9.2A.6 |  | Ericsson |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR-U SCC measurements, with E-UTRAN PCC (FDD,TDD) and NR-U PSCC |  |  | Yes |  |  |  |
|  | Intra-frequency SS-RSRQ, SS-SINR measurements on: |  | 9.2A.5, 9.2A.6 |  |  |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR-U SCC measurements, with E-UTRAN PCC (FDD,TDD) and NR-U PSCC |  |  | Yes |  |  |  |
|  | L1-RSRP measurements on: |  | [9.5.4A] |  | Ericsson |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR-U SCC measurements, with E-UTRAN PCC (FDD,TDD) and NR-U PSCC |  |  | Yes |  |  |  |
|  | Intra-frequency RSSI measurements on: |  | 9.2A.7.1 |  | Ericsson |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR-U SCC measurements, with E-UTRAN PCC (FDD,TDD) and NR-U PSCC |  |  | Yes |  |  |  |
|  | Intra-frequency CO measurements on: |  | 9.2A.7.2 |  |  |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR-U SCC measurements, with E-UTRAN PCC (FDD,TDD) and NR-U PSCC |  |  | Yes |  |  |  |
| Inter-frequency measurement procedure (SS-RSRP, SS-RSRQ, SS-SINR, SFTD, RSSI, CO) | Inter-frequency SS-RSRP measurements on: |  | 9.3A.4, 9.3A.5 |  | Nokia |  |  |
|  |  NR-U inter-frequency, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U inter-frequency, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR (FR1) inter-frequency, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR (FR1) inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  | Inter-frequency SS-RSRQ, SS_SINR measurements on: |  |  | FFS |  |  |  |
|  |  NR-U inter-frequency, with NR PCC (FR1) |  |  | FFS |  |  |  |
|  |  NR-U inter-frequency, with NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  |  NR (FR1) inter-frequency, with NR-U PCC |  |  | FFS |  |  |  |
|  |  NR (FR1) inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  | Inter-frequency RSSI measurements on: |  | 9.3A.8 |  |  |  |  |
|  |  NR-U inter-frequency, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U inter-frequency, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  | Inter-frequency CO measurements on: |  | 9.3A.9 |  |  |  |  |
|  |  NR-U inter-frequency, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U inter-frequency, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
| Inter-RAT measurement procedure (SFTD, E-UTRA-NR-U SS-RSRP/SS-RSRQ/SS-SINR, NR-U-E-UTRA RSRP/RSRQ) | Inter-RAT SFTD between: |  | TS 36.133 |  | Ericsson |  |  |
|  |  E-UTRAN PCell (FDD,TDD) and NR-U neighbor |  |  | Yes |  |  |  |
|  | NOTE: under the condition of stationary paths |  |  | Yes |  |  |  |
|  | NR-U-E-UTRA RSRP/RSRQ (needed for HO): |  | 9.4.2, 9.4.3 |  |  |  |  |
|  |  On E-UTRA (FDD,TDD), with NR-U PCC |  |  | FFS |  |  |  |
|  |  On E-UTRA (FDD,TDD), with NR-U PSCC |  |  | FFS |  |  |  |
|  | E-UTRA-NR-U SS-RSRP/SS-RSRQ/SS-SINR: |  | TS 36.133 |  | Huawei |  |  |
|  |  On NR-U neighbor, with E-UTRA (FDD,TDD) PCC |  |  | Yes |  |  |  |
|  |  On NR-U neighbor, with E-UTRA (FDD,TDD) PCC and NR-U PSCC |  |  | Yes |  |  |  |
| Accuracy for NR-U intra-frequency measurements (SS-RSRP, SS-RSRQ, SS-SINR, L1-RSRP, RSSI, CO) | Intra-frequency absolute and relative accuracies for SS-RSRP on: |  | [10.1.27] |  | Huawei |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  | Intra-frequency absolute accuracies for SS-RSRQ on: |  | [10.1.29] |  |  |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | FFS |  |  |  |
|  |  NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  |  NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  | Intra-frequency absolute accuracies for SS-SINR on: |  | [10.1.31] |  |  |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | FFS |  |  |  |
|  |  NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  |  NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  | Absolute and relative accuracies for L1-RSRP on: |  | [10.1.33] |  | MTK |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  | Intra-frequency RSSI on: |  | [10.1.34.1] |  | Apple |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  | Intra-frequency CO on: |  | [10.1.35.1] |  | Apple |  |  |
|  |  NR-U SCC, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  |  NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
| Accuracy for NR-U inter-frequency measurements (SS-RSRP, SS-RSRQ, SS-SINR, SFTD, RSSI, CO) | Inter-frequency absolute and relative accuracies for SS-RSRP on: |  | [10.1.28] |  | MTK |  |  |
|  |  NR-U neighbor, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U neighbor, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U neighbor, with NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  | Inter-frequency absolute and relative accuracies for SS-RSRQ on: |  | [10.1.30] |  |  |  |  |
|  |  NR-U neighbor, with NR PCC (FR1) |  |  | FFS |  |  |  |
|  |  NR-U neighbor, with NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U neighbor, with NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  | Inter-frequency absolute and relative accuracies for SS-SINR on: |  | [10.1.32] |  |  |  |  |
|  |  NR-U neighbor, with NR PCC (FR1) |  |  | FFS |  |  |  |
|  |  NR-U neighbor, with NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U neighbor, with NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  |  NR (FR1) inter-frequency, with NR-U PCC |  |  | FFS |  |  |  |
|  |  NR (FR1) inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | FFS |  |  |  |
|  | Inter-frequency RSSI on: |  | [10.1.34.2] |  | Apple |  |  |
|  |  NR-U neighbor, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U neighbor, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U neighbor, with NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
|  | Inter-frequency CO on: |  | [10.1.35.2] |  | Apple |  |  |
|  |  NR-U neighbor, with NR PCC (FR1) |  |  | Yes |  |  |  |
|  |  NR-U neighbor, with NR-U PCC |  |  | Yes |  |  |  |
|  |  NR-U neighbor, with NR-U PSCC, with E-UTRAN PCC (FDD,TDD) |  |  | Yes |  |  |  |
| Accuracy for inter-RAT measurements (SFTD, E-UTRA-NR-U SS-RSRP/SS-RSRQ/SS-SINR, NR-U-E-UTRA RSRP/RSRQ) | Inter-RAT SFTD between: |  | TS 36.133 |  | Ericsson |  |  |
|  |  E-UTRAN PCell (FDD,TDD) and NR-U neighbor |  |  | Yes |  |  |  |
|  | NOTE: under the condition of stationary paths |  |  |  |  |  |  |
|  | E-UTRA RSRP/RSRQ (needed for HO) with: |  | 10.2.2, 10.2.3 |  |  |  |  |
|  |  NR-U PCC |  |  | FFS |  |  |  |
|  |  NR-U PSCC |  |  | FFS |  |  |  |
|  | E-UTRA-NR-U SS-RSRP/SS-RSRQ/SS-SINR: |  | TS 36.133 |  |  |  |  |
|  |  On NR-U neighbor, with E-UTRA (FDD,TDD) PCC |  |  | Yes |  |  |  |
|  |  On NR-U neighbor, with E-UTRA (FDD,TDD) PCC and NR-U PSCC |  |  | Yes |  |  |  |