3GPP TSG-RAN WG4 Meeting # 97-e												R4-200XXXX

Electronic Meeting, 2 – 13 Nov., 2020

Agenda item:			5.2 and 6.5.2

Source:	Skyworks Solution Inc.

Title:	Email discussion summary for [97e][105] LTE UE RF maintenance

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: 5 topics to be discussed for agenda:

- 5.2: LTE UE RF maintenance up to R15 + 6.4.2: LTE UE RF maintenance up to R16

  - Topic 1: R15&16 Corrections to Bands an CA

  - Topic 2: R15&16 Corrections to co-ex and AMPR

  - Topic 3: NB-IoT FCC emission issues

  - Topic 4: R15&16 Corrections to MBMS

- 2nd round: TBA

# Topic #1: Corrections to bands and CA

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2014045 Correction of B88 UL EARFCN | Huawei,HiSilicon | In the current spec the UL starting EARFCN of band 88 equals to the UL end EARFCN of band 87 => The UL EARFCN of band 88 is increased by one. Moderator: comment directly in CR section 1.3.2 |
| R4-2014511 Band 88 and 87 bracket removal | Nokia | RAN4 M2 REFSENS requirement has brackets which means that the requriement is untestable => Brackets removed. Moderator: comment directly in C section 1.3.2 |
| R4-2016450 CR for 36.101: Corrections for UL CA_41D | T-Mobile USA | There is an incorrect reference to a void section => Delete the reference to 6.6.3.3A.9, and add and CA_41D to the header of 6.6.3.3A.8 Moderator: comment directly in CR section 1.3.2 |
| R4-2014510 LTE CA corrections | Nokia | R4-2006725 was not implemented properly, CA_13A-48A-48A-66A disappeared and is still in clauee 7 and errors to other configurations emerged. CA_2A-48E-66A-66A has wrong aggregated BW. CA_1A-18A-41C has invalid BCS reference. Moderator: comment directly in CR section 1.3.2 |
| R4-2016340 Rel-16 CR editorial corrections 36.101 | Ericsson | CA_48B is defined, now corrected included in Table 5.6A.1-1 Corrected UL for CA_3A-41A-42C Table 5.6A.1-2a Correcting CA_2A-4A-7C in Table 5.6A.1-2a Correcting CA_5A-48D-66A and CA_5A-48D-66A-66A, CA_2A-5A-48D-66A and CA_2A-5A-48D-66A-66A References to CA 66A-66A changed to CA_66A-66A References to CA 48C change to CA_48C References to CA 48D change to CA_48D Corrected UL bands for CA_2A-13A-48A, CA_2A-48A-66A and CA_2A-14A-66A in MSD Table 7.3.1A-0g Moderator: comment directly in CR section 1.3.2 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description: Band 87 and 88 corrections: please directly comment in the CR section 1.3.2

### Sub-topic 1-2

Sub-topic description : Correction to CA_41D: please directly comment in the CR section 1.3.2

### Sub-topic 1-3

Sub-topic description :: CA_13A-48A-48A-66A , CA_2A-48E-66A-66A, CA_1A-18A-41C corrections: please directly comment in the CR section 1.3.2

### Sub-topic 1-4

Sub-topic description : Corrections in Table 5.6A.1-1, 5.6A.1-2a and Table 7.3.1A-0g from R4-2016340: please directly comment in the CR section 1.3.2

## Companies views’ collection for 1st round

### Open issues

Moderator: Comment directly in CR section 1.3.2

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2014045 | Company A |
|  | Company B |
| R4-2014511 | Company A |
|  | Company B |
| R4-2016450 | Company A |
|  | Company B |
| R4-2014510 | Company A |
|  | Company B |
| R4-2016340 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: R15&16 Corrections to co-ex and AMPR

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2014311 Clarifications and corrections on UE co-ex requirements(R15) | SoftBank Corp. | - NS related requirements (including Note 7, 8, 15,16) are deleted. - A sentence to clarify applicability of additional requirements is added - Some errors are corrected: - In 2UL-CA, note numbers of single band co-ex table seem to be copied in some of “Note 15” -> These should be Note 3(delta FOOB) in the CA table. - In single band table, B74 -> n77-n79 protections are missed. Moderator: PHS band protection removed? Please comment in CR section 2.3.2 |
| R4-2014312 Clarifications and corrections on UE co-ex requirements(R16) | SoftBank Corp. | Moderator: Release 16 mirror CR (uploaded) will be agreed after R15 CR |
| R4-2014896 Coexistence cleanup for 36101 Rel15 | Apple Inc. | Correction of protected band list: CA_1-26, CA_3-28, CA_7-26, CA_11-26, CA_11-18, CA_19-21 and CA_11-26 Moderator: Please comment in CR section 2.3.2 |
| R4-2014897 Coexistence cleanup for 36101 Rel16 | Apple Inc. | Moderator: Release 16 mirror CR (uploaded) will be agreed after R15 CR |
| R4-2016426 LTE CA_NS_04 PC2 256QAM AMPR | Qualcomm Incorporated | Observation 1: The LTE 256QAM CA_NS_04 back-off should be at least be allowed the same back-off as the single CC NR DFT-s-OFDM 256QAM back-off within the similar RB boundary condition. Both back-off is calculated as max (MPR, AMPR). Proposal: Modify Power Class 2 LTE CA_NS_04 AMPR as in Table 2.1 Moderator: 256 QAM column: 8dB and 6.5dB depending on RB allocation. Proposal can be commented directly with CR R4-2014164 in CR section 2.3.2 |
| R4-2014164 LTE CA_NS_04 PC2 256QAM AMPR | Qualcomm Inc. | Revise 256QAM AMPR for PC2 CA_NS_04 in Table 6.2.4A.4-2 from FFS to 8dB and 6.5dB depending on RB allocation. Moderator: related to discussion paper R4-2016426 Please comment in CR section 2.3.2 |
| R4-2014163 LTE CA_NS_04 PC2 256QAM AMPR | Qualcomm Inc. | Moderator: Withdrawn or mirror CR (R16 CR too)? |
| R4-2014162 LTE CA_NS_04 PC2 256QAM AMPR | Qualcomm Inc. | Moderator: Withdrawn or mirror CR (R16 CR too)? |
| R4-2016008 LTE CA_NS_08 A-MPR Correction | Skyworks Solutions Inc. | Clarification on CA_NS_08 region where CA_MPR should still apply for QPSK and 16 QAM in support for relevant section in CR R4-2016035 The comments can be done directly in CR section 2.3.2 |
| R4-2016035 CR Correction to B72 coex - CA_NS_08 - Band 10 protection 36.101 Rel15 | Skyworks Solutions Inc. | - Restore Band 72 list of protected bands, ie B72 and B31, - Band 10 protection removal has been agreed in R4-2011521. This CR applies this correction to Release 15, - Allow CA A-MPR for inner region CA_NS_08 allocations (discussed in R4-2016008) Moderator: Please comment in CR section 2.3.2 |
| R4-2016040 CR Correction to B72 coex - CA_NS_08 - Band 10 protection 36.101 Rel16 | Skyworks Solutions Inc. | Moderator: Release 16 mirror CR (uploaded) will be agreed after R15 CR |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description: Corrections to Coex tables: R15 CRs R4-2014311 and R4-2014896 and associated mirror CRs: comment in CR section 2.3.2

### Sub-topic 2-2

Sub-topic description: NS_04 PC2 256 QAM AMPR proposal

Issue 2-2: NS_04 PC2 256 QAM AMPR

- Recommended WF

  - Moderator: 256QAM CA_NS_04 back-off should be at least be allowed the same back-off as single CC NS_04

  - WF: Since this is straight FW the comments can be collected for the CR R4-2014164 directly in section 2.3.2

### Sub-topic 2-3

Sub-topic description: CA_NS_08 0dB AMPR region

Issue 2-2: CA_NS_08 0dB AMPR region

- Recommended WF Some allocations in the region where A-MPR is not defined needs to benefit of CA_MPR instead of 0dB, comments can be collected for the CR R4-2016035 directly in section 2.3.2

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: …. Others: |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2014311 | Company A |
|  | Company B |
| R4-2014896 | Company A |
|  | Company B |
| R4-2014164 | Company A |
|  | Company B |
| R4-2016035 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: NB-IoT FCC emission issues

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2015807 Test frequencies for NB-IOT UE in standalone operation | Sony | Observation 1: TS 36.104 test conditions (test frequencies) for both stand-alone and guard-band NB-IoT operation may conflict with FCC band-edge spectrum emission requirements. Observation 2: 100 kHz offset for NB-IoT network deployments may solve the violation of the FCC regulation. Proposal 1: Send an LS to RAN5 with proposal to exclude the first and last EARFCNs in TS 36.104 test frequencies for both stand-alone and guard-band IoT operation modes for all frequency bands were FCC regulation applies. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Follow up from RAN4#95/96 on FCC emission requirement for NB-IoT, LS to FCC was sent last meeting without answers yet. Proposal seems similar to the NS signaling approach that was proposed by other companies.

### Sub-topic 3-1

Sub-topic description: Is removing NB-IoT edge channels from RAN5 specification and test sufficient to avoid the issue at UE certification in FCC labs?

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 3-1: |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: R15&16 Corrections to MBMS

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2015549 CR for 36.101 to clarify the SCS supports for LTE MBMS (Rel-14) | Huawei, HiSilicon | Based on the agreement in R4-2012604, UE doesn’t have to support all of the SCS, if UE support LTE MBMS For MBMS feature, there is no need to meet the minimum requirements of transmitter characteristics for UE. Moderator, overlapping CR tackling the same issue than R4-2016129 , suggest to merge CRs, co-source and agree on wording for SCS support |
| R4-2015550 CR for 36.101 to clarify the SCS supports for LTE MBMS (Rel-15) | Huawei, HiSilicon | R15 mirror CR |
| R4-2015551 CR for 36.101 to clarify the SCS supports for LTE MBMS (Rel-16) | Huawei, HiSilicon | R16 mirror CR |
| R4-2016129 CR to TS 36.101 clarifications on supported SCS for UE supporting LTE MBMS | ZTE Corporation | There was some ambiguity existing for UE supporting LTE MBMS that whether all SCS should be supported. Based on the agreement in R4-2012604, MBMS UE doesn’t have to support all of the SCS, if UE support LTE MBMS. Moderator, overlapping CR tackling the same issue than R4-2015549: suggest to merge CRs, co-source and agree on wording for SCS support |
| R4-2016130 CR to TS 36.101 clarifications on supported SCS for UE supporting LTE MBMS | ZTE Corporation | R15 mirror CR |
| R4-2016131 CR to TS 36.101 clarifications on supported SCS for UE supporting LTE MBMS | ZTE Corporation | R16 mirror CR |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1

Sub-topic description: UEs that only support MBMS do not need to support all SCS and do not need to support UL.

Open issues and candidate options before e-meeting:

Issue 4-1: MBMS SCS and UL support

- Recommended WF

  - Moderator note: overlapping CRs from Huawei and ZTE, suggest to merge CRs, co-source and agree on wording for SCS support

  - WF: Chose one company set of CR, co-source and agree on wording. This will need revision, proponent to agree on best starting CR.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 4-1: …. Others: |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2015549 And R4-2016129 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |