3GPP TSG-RAN WG4 Meeting #97-e                         　　　　　　　　　       R4-2016664

Online, 2nd Nov-13th Nov, 2020

Source: 	KDDI

Title:	TP for DC_1-18_n28

Agenda item:	10.4.2

Document for:	Approval

# Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_1A-18A_n28A and according to the request in [1].

# References

[1]	RP-202060, Revised WID on Rel-17 Dual Connectivity (DC) of 2 bands LTE inter-band CA  (2DL/1UL) and 1 NR

band (1DL/1UL).

# Text Proposal

< Start of Text Proposal>

## 5.x	DC_1-18_n28

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-18A_n28A | DC_1A_n28A DC_18A_n28A |

### 5.x.2	Co-existence studies

Based on co-existence studies of DC_1_n28 and DC_18_n28, 5th order IMD generated by dual uplink of Band 18 + Band n28 may fall into own Rx of band 1. The MSD value of this configuration can follow CA_1A-18A-28A with UL CA_18A-28A case. Since band 18 is only used by KDDI, no additional MSD requirement need to be specified for this configuration considering KDDI spectrum (band n28 UL is limited to 718MHz – 728MHz).

### 5.x.3	∆TIB and ∆RIB values

For DC_1-18_n28, the TIB,c and RIB,c values are given in the tables below.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | n28 | 0.5 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18_n28 | 1 | 0 |
|  | 18 | 0 |
|  | n28 | 0 |

### 5.x.4	Reference sensitivity exceptions

As stated in 5.x.2, there is no additional MSD requirement for this configuration.

< End of Text Proposal>