3GPP TSG-RAN4 Meeting #97-e	R4-2016833

# Online, , 2nd Nov 2020 - 13th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0509 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TR 37.716-21-11: Addition of missing lower order fallbacks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | These configurations have relating higher order configurations already in REL16 specs. This CR captures necessary analysis into the TR. DC_2A-66A_n2A DC_30A-66A_n66A DC_2A-30A_n2A DC_29A-30A_n2A DC_30A-66A_n66A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Missing lower order fallbacks are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lower order fallbacks are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of Changes ******************************

## 5.1.X	DC_2-66_n2

### 5.1.X.1	Configurations for DC

Table 5.1.X.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-66A_n2A | DC_2A_n2A1 DC_66A_n2A |
| NOTE 1: Only single switched UL is supported |  |

### 5.1.X.2	Co-existence studies

When uplink is DC_66A_n2A there is interfering IMD3 and IMD5 to E-UTRA band 2 downlink.

### 5.1.X.3	∆TIB and ∆RIB values

It is proposed to take the maximum relaxation values from E-UTRA CA_2-66

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n2 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n2 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.X.4	Reference sensitivity exceptions

It is prosed to re-use the IMD3 and IMD5  MSD from already specified configuration DC_2A-66A_n25A which is similar to DC_2A-66A_n2A.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-66A_n2A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |

## 5.1.X	DC_29-30_n66

### 5.1.X.1	Configurations for DC

Table 5.1.X.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_29A-30A_n66A | DC_30A_n66A |

### 5.1.X.2	Co-existence studies

When uplink is DC_30A_n66A there is IMD5 interfering band 29 downlink.

### 5.1.X.3	∆TIB and ∆RIB values

It is proposed to take the maximum relaxation values from E-UTRA CA_29-30-66

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-30-n66 | 30 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-30-n66 | 30 | 0.5 |
|  | n66 | 0.4 |

### 5.1.X.4	Reference sensitivity exceptions

It is prosed to re-use the IMD5 MSD from already specified configuration DC_1A-28A_n7A which is similar to DC_29A-30A_n66A.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_29A-30A_n66A | 29 | N/A | 5 | 25 | 719.5 | 4.5 | IMD5 |
|  | 30 | 2307.5 | 5 | 25 | 2352.5 | N/A | N/A |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |

## 5.1.X	DC_2-30_n2

### 5.1.X.1	Configurations for DC

Table 5.1.X.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-30A_n2A | DC_2A_n2A1 DC_30A_n2A |
| NOTE 1: Only single switched UL is supported |  |

### 5.1.X.2	Co-existence studies

When uplink is DC_30A_n2A there is no interfering IMD to E-UTRA band 2 downlink.

### 5.1.X.3	∆TIB and ∆RIB values

It is proposed to take the maximum relaxation values from E-UTRA CA_2-30

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-30_n2 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-30_n2 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |

### 5.1.X.4	Reference sensitivity exceptions

No reference sensitivity exceptions needed.

## 5.1.X	DC_29-30_n2

### 5.1.X.1	Configurations for DC

Table 5.1.X.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_29A-30A_n2A | DC_30A_n2A |

### 5.1.X.2	Co-existence studies

When uplink is DC_30A_n2A there is no interfering IMD to E-UTRA band 29 downlink.

### 5.1.X.3	∆TIB and ∆RIB values

It is proposed to take the maximum relaxation values from E-UTRA CA_30-66

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-30_n2 | 30 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-30_n2 | 30 | 0.3 |
|  | n2 | 0.5 |

### 5.1.X.4	Reference sensitivity exceptions

No reference sensitivity exceptions needed.

## 5.1.X	DC_30-66-n66

### 5.1.X.1	Configurations for DC

Table 5.1.X.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_30A-66A_n66A | DC_30A_n66A DC_66A_n66A1 |
| NOTE 1: Only single switched UL is supported |  |

### 5.1.X.2	Co-existence studies

When uplink is DC_30A_n66A there is no interfering IMD to E-UTRA band 66 downlink.

### 5.1.X.3	∆TIB and ∆RIB values

It is proposed to take the maximum relaxation values from E-UTRA CA_30-66

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30-66-n66 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_30-66-n66 | 30 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.4 |

### 5.1.X.4	Reference sensitivity exceptions

No reference sensitivity exceptions needed.

***************************** End of Changes ********************************