TSG-RAN Working Group 4 (Radio) meeting #97-E	R4-2016902

Electronic Meeting,  2nd – 13th November 2020

Source:	Ericsson

Title:	TP to TR 38.921: Correction to antenna parameter table in clause 3 and sub-clause 8.1

Agenda item:	11.1.4

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#96-E) a text proposal to TR 38.921 with antenna parameters was agreed. Unfortunately, it was decided to update the definition of power feed to the antenna at a very late stage of the meeting.

In this contribution a text proposal has been created to update TR 38.921, subclause 8.1 according to the reply LS sent to ITU-R WP 5D at last meeting. Also, clause 3 is updated with all for the antenna model relevant definitions.

This is a revised version of R4-2014979.

# Discussion

From reply LS [2] defined antenna parameters are listed in Table 2-1.

Table 2-1: Beamforming antenna characteristics for IMT in 6425 – 10500 MHz

|  |  |  | Rural |  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |  |  |  |  |
| 1.1 | Antenna pattern |  | Refer to Recommendation ITU-R M.2101 |  |  |  |  | N/A |  |
| 1.2 |  | Element gain (dBi) (Note 1) | N/A |  | 6.4 | 5.5 | 5.5 | N/A |  |
| 1.3 |  | Horizontal/vertical 3 dB beamwidth of single element (degree) | N/A |  | 90º for H 65º for V | 90º for H 90º for V | 90º for H 90º for V | N/A |  |
| 1.4 |  | Horizontal/vertical front-to-back ratio (dB) | N/A |  | 30 for both H/V | 30 for both H/V | 30 for both H/V | N/A |  |
| 1.5 |  | Antenna polarization | N/A |  | Linear ±45º | Linear ±45º | Linear ±45º | N/A |  |
| 1.6 |  | Antenna array configuration (Row × Column) (Note 2) | N/A |  | 16 × 8 elements | 16 × 8 elements | 8 × 8 elements | N/A |  |
| 1.7 |  | Horizontal/Vertical radiating element spacing | N/A |  | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V | N/A |  |
| 1.8 |  | Array Ohmic loss (dB) (Note 1) | N/A |  | 2 | 2 | ## 2 | N/A |  |
| 1.9 |  | Conducted power (before Ohmic loss) per antenna element (dBm) | N/A |  | 22 (Note 5) | 22 (Note 5) | 16 (Note 6) | N/A |  |
| 1.10 |  | Base station maximum coverage angle in the horizontal plane (degrees) | N/A |  | +/- 60 | +/- 60 | +/- 60 | N/A |  |
| 1.11 |  | Base station vertical coverage range (degrees) (Note 3, 4) | N/A |  | 90-100 | 90-120 | 90-120 | N/A |  |
| 1.12 |  | Mechanical downtilt (degrees) (Note 4) | N/A |  | 6 | 10 | N/A | N/A |  |

Note 1:	The element gain in row 1.2 includes the loss given in row 1.8.

Note 2:	m × n means there are m vertical and n horizontal radiating elements. In the sub-array case, one implementation is 2 vertical radiating elements combined in a 2x1 sub-array

Note 3:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as in ITU-R M.2101.

Note 4:	The vertical coverage range in row 1.11 includes the mechanical downtilt given in row 1.12.

Note 5:	The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element).

Note 6:	The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element).

Text marked “red” was updated before approval. This text proposal updated the power in corresponding table in TR 38.921, subclause 8.1.

# Conclusion

At the end on this contribution a text proposal TR 38.921, clause 3 and sub-clause 8.1 is attached for approval.

# References

[1]	R4-2011830, “TP to TR 38.921: Addition of BS antenna model and parameters in subclause 4.2.3 and subclause 8.1”, Ericsson

[2] 	R4-2011932, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)”, Ericsson

TEXT PROPOSAL to clause 3:

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

array element: subdivision of a passive antenna array, consisting of a single radiating element or a group of radiating elements, with a fixed radiation pattern

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

front-to-back ratio: ratio of maximum directivity of an antenna to its directivity in a specified rearward direction

gain: ratio of the radiation intensity, in a given direction, to the radiation intensity that would be obtained if the power accepted by the antenna were radiated isotropically

NOTE:	If the direction is not specified, the direction of maximum radiation intensity is implied.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

A	Peak normalized array element pattern in dB

AA	Composite antenna array pattern in dBi

AE	Array element pattern in dBi

Am	Front to back ratio in dB

dh	Horizontal element separation in meters

dv	Vertical element separation in meters

GE,max	Array element peak gain in dBi

SLAv	Side lobe suppression in dB

3dB 	Horizonal half power beamwidth

3dB	Vertical half power beamwidth

etilt	Electrical down-tilt angle in degrees (defined from antenna array normal and downwards)

escan	Electrical scan angle in degrees

	Horizontal angle (defined between -180° and 180°).

	Vertical angle of the signal direction (defined between -0° and 180°,90° represents the direction perpendicular to the antenna array

NRB	Transmission bandwidth configuration, expressed in resource blocks

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS	Active Antenna System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AWGN	Additive White Gaussian Noise

BS	Base Station

BW	Bandwidth

FDD	Frequency Division Duplex

FR	Frequency Range

FRC	Fixed Reference Channel

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

MR	Medium Range

NR	New Radio

OTA	Over The Air

RB	Resource Block

RF	Radio Frequency

SCS	Sub-Carrier Spacing

TDD	Time division Duplex

WA	Wide Area

TEXT PROPOSAL to subclause 8.1:

## 8.1	BS antenna characteristics

### 8.1.1		Array antenna model

In Table 8.1.1-1, the parameters used by the parameterized array antenna model are described.

Table 8.1.1-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.1-2.

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πn-1dvλcosθ+m-1dhλsinθsinφ wm,n=1MNexpj2πn-1dvλsinθetilt-m-1dhλcosθetiltsinφescan | dBi |

### 8.1.2		Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain.

Table 8.1.2-1: BS array antenna parameters

| Parameter | Rural | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- | --- |
| Am (dB) | N/A | 30 | 30 | 30 |
| SLAv (dB) | N/A | 30 | 30 | 30 |
| 3dB (deg.) | N/A | 90 | 90 | 90 |
| 3dB (deg.) | N/A | 65 | 90 | 90 |
| GE,max (dBi) | N/A | 6.4 | 5.5 | 5.5 |
| LE (dB) | N/A | 2.0 | 2.0 | 2.0 |
| (M, N) | N/A | (16, 8) | (16, 8) | (8,8) |
| dh (m) | N/A | 0.5 | 0.5 | 0.5 |
| dv (m) | N/A | 0.7 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | N/A | +/- 60 | +/- 60 | +/- 60 |
| Vertical coverage range (deg.) | N/A | 90 to 100 | 90 to 120 | 90 to 120 |
| Conducted power (before ohmic loss) per antenna element (dBm) | N/A | 22 | 22 | 16 |
| Mechanical downtilt (deg.) | N/A | 6 | 10 | N/A |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). |  |  |  |  |

## 8.2	UE antenna characteristics

The outcome of the RAN4 study for collecting technical background information relevant for the frequency range 7 to 24 GHz indicated that the frequency range 7.25-[10-13] GHz would have “FR1 like” requirements, and as such we can assume that in the 10-10.5GHz range this applies. The UE will most likely therefore have a conducted interface with an assumed isotropic radiation pattern antenna and no beam forming.