3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2017490

Electronic Meeting, 2-13 Nov., 2020

Title: 	WF on dynamic range, power control (LA) and frequency error for IAB-MT

Source: 	CATT

Agenda item:	7.4.3.1

Document for:	Discussion

- Background

RAN4#97e is the first meeting to discuss IAB conformance RF test, the following contributions are related to the test of dynamic range, power control and frequency error.

R4-2014391, “Discussion on the reference conditions of IAB-MT output power dynamics”, CATT

R4-2015441, “Radiated conformance testing, Tx requirements”, Nokia, Nokia Shanghai Bell

The 3 issues were discussed in the 1st round email discussion and also at the GTW meeting. The discussion and comments were captured in the email discussion summary R4-2017617.

- Way forward

2.1 WF on dynamic range test

Agreement: RAN4 will introduce conformance test cases for dynamic range requirements for both wide-area and local-area IAB-MT classes.

- RAN4 will further discuss the uncertainty impact on the feasibility of introducing test case

The following test points were identified as candidates in RAN4#97e and they will be down selected in next RAN4 meeting and should be reviewed based on agreement on updated DR requirement if any

- [1] Maximum output power with full RB allocation.

- [2] Minimum output power (as set by 5/10 dB dynamic range requirement) with full RB allocation.

- [3] Narrow RB allocation with the same PSD as [2].

- [4] Partial RB allocation with same PSD as [1].

- Others proposals, for example maximum output power with partial RB allocation, if any reasonable justifications

The following aspects will be considered/decided when the test points are concluded.

- The reference side condition agreed in R4-2008775.

- If some test point can be covered by other requirements.

- The typical scenarios of IAB-MT backhaul function.

- The exact RB number for narrow/partial RB allocation based on reference condition definition in core requirements

- Whether test point set/combination for dynamic range is different for Wide Area and Local Area IAB-MT

- Others if any.

2.2 WF on power control test for Local Area IAB-MT

Agreement: Dynamic range and power control tests are defined separately.

- Further discussion on test applicable rules among these test cases is not excluded.

For relative power accuracy test,

- The smallest power control step size is considered.

- The followings will be discussed in next meeting.

  - How to accommodate this test with the test set-up structure.

    - If two way signal is necessary.(this also relates to the generic test setup discussion)

  - Power control step size, RB allocation, etc.

2.3 WF on frequency error test

The following will be discussed in next meeting

- How to accommodate this test with the test set-up structure to be agreed for IAB-MT test environment setup

  - If two way signal is necessary.