3GPP TSG-RAN WG4 Meeting #97-e	R4-2017555

# Electronic Meeting, 2 - 13 Nov, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | 0166 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.141-1: Introduction of conformance testing for NR HST PRACH under fading channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_Perf |  |  |  |  |  | Date: |  |  | 2020-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 agree to introduce PRACH requirements of fading channel and the aligned requirements need to be added into the specfication |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the PRACH requirements of fading channel |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRACH requirements of fading channel are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2016596 |  |  |  |  |  |  |  |  |

<Start of First change>

8.4.1.6	Test requirement for high speed train

Pfa shall not exceed 0.1%. Pd shall not be below 99% for the SNRs in tables 8.4.1.6-1 to 8.4.1.6-4.

Table 8.4.1.6-1: PRACH missed detection requirements for high speed train, burst format 0, restricted set type A, 1.25 kHz SCS

| Number of TX | Number of RX | Propagation conditions | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
| antennas | antennas | and correlation matrix (annex G) |  | Burst format 0 |
| 1 | 2 | AWGN | 625 Hz | -11.7 |
|  |  | AWGN | 1340 Hz | -13.5 |
|  |  | TDLC300-100 Low | 0 Hz | [-5.8] |
|  | 4 | AWGN | 625 Hz | -14.2 |
|  |  | AWGN | 1340 Hz | -15.9 |
|  |  | TDLC300-100 Low | 0 Hz | [-11.3] |
|  | 8 | AWGN | 625 Hz | -16.2 |
|  |  | AWGN | 1340 Hz | -18.1 |
|  |  | TDLC300-100 Low | 0 Hz | [-15.8] |

Table 8.4.1.6-2: PRACH missed detection requirements for high speed train, burst format 0, restricted set type B, 1.25 kHz SCS

| Number of TX | Number of RX | Propagation conditions | Frequency offset | SNR (dB) |
| --- | --- | --- | --- | --- |
| antennas | antennas | and correlation matrix (annex G) |  | Burst format 0 |
| 1 | 2 | AWGN | 625 Hz | -11.3 |
|  |  | AWGN | 2334 Hz | -12.8 |
|  |  | TDLC300-100 Low | 0 Hz | [-5.5] |
|  | 4 | AWGN | 625 Hz | -13.7 |
|  |  | AWGN | 2334 Hz | -15.1 |
|  |  | TDLC300-100 Low | 0 Hz | [-11.2] |
|  | 8 | AWGN | 625 Hz | -16.0 |
|  |  | AWGN | 2334 Hz | -17.1 |
|  |  | TDLC300-100 Low | 0 Hz | [-15.6] |

Table 8.4.1.6-3: PRACH missed detection requirements for high speed train, 15 kHz SCS

| Number of | Number of | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 1740 Hz | -11.0 | -14.0 | -10.8 |
|  | 4 | AWGN | 1740 Hz | -13.2 | -16.4 | -13.1 |
|  | 8 | AWGN | 1740 Hz | -15.3 | -17.9 | -15.2 |

Table 8.4.1.6-4: PRACH missed detection requirements for high speed train, 30 kHz SCS

| Number of | Number of | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 3334 Hz | -10.9 | -14.3 | -10.7 |
|  | 4 | AWGN | 3334 Hz | -13.1 | -16.4 | -13.1 |
|  | 8 | AWGN | 3334 Hz | -15.1 | -18.1 | -15.1 |

<end of First change>