3GPP TSG-WG4 Meeting #97-e	R4-2017581

# Electronic Meeting, 2nd – 13th November, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.827 | CR | 0002 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Time Domain Alternative for Spatial Correlation Validation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_MIMO_OTA_test |  |  |  |  |  | Date: |  |  | 2020-10-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Time Domain Techniques to validate Spatial Correlation have been agreed during R4#96e. In [1], the FR1 spatial correlation validation procedure based on time domain techniques is presented. This revision CR addresses the coments received during the first round. The settings of the signal analyser are chosen as follows: Sampling Frequency: Sampling a random process needs to fulfil one requirement, namely, it must be be greater than twice the maximum frequency content (i.e. maximum Doppler shift). Observation Time: To characterize random processes, enough samples must be observed. The two parameters above are related to the UE speed (v) and center frequency (fc), as those two determine the maximum Doppler frequency shift fd=(v•fc)/c, where c is the speed of light. When the UE speed is low, and the center frequency is low, the maximum Doppler shift is also low. One could set the sampling rate to a low value, but the observation time would need to be long to observe enough samples. When the UE speed is high, and the center frequency is high, the maximum Doppler shift is also high. One needs to set the sampling rate to a high value, and the observation time to a low value as well. But the observation time should not be too low because enough samples as needed to obtain statistical significance. Since the values of center ferequency for FR1 are relatively wide, the observation time is lower bounded to cover the cases of low center frequencies to at least 16 seconds. Larger observation times are allowed to obtain better statistical convergence. The sampling frequency is also lower bounded to at least 15 times the maximum Doppler to obtain a smoothly sampled set of data. Higher sampling rates are allowed to cover high UE speeds and/or higher center frequencies. The above discussion makes it clear that both parameters are frequency dependent, but lower bounded to obtain enough samples and smooth signals. References [1] R4-2014289 Addition of Time Domain Alternative for Spatial Correlation Validation, RAN4#97e November 2020. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition to Spatial Correlation validation section |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Time domain techniques are faster by a factor of 37.5. It would take days to validate spatial correlation using the current techniques instead of hours. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

#### 7.4.1.3	Spatial correlation

This measurement checks whether the measured correlation curve follows the theoretical curve. For spatial correlation validation measurement, both Vertical and Horizontal validation are required. Spatial correlation validation is only adopted for FR1 MIMO OTA.

The spatial correlation validation measurement setup is illustrated in Figure 7.4.1.3-1. The network analyser transmits signals through the fading emulator and probes. The 16 probes radiate the signals within the anechoic chamber and a receiving test antenna is placed within the test zone. The test antenna is attached to a positioner that can move the antenna to pre-defined spatial locations on a fixed radius from the centre of the quiet zone. The received signal is measured with the network analyser.

The measurement and analysis procedure is as follows:

- Set the target channel model to fading emulator.

- For each position of the test antenna in the test zone, step & pause the emulator to different time instances. Measure the frequency responses Hf,t=HmΔf,nΔT, m=0,…,M-1 for all stepped channel snapshots n=0,…,N-1, where the interval between frequency and time samples is Δf and ΔT, respectively. The number of channel snapshots N and frequency samples M should be sufficiently high so that the matrix can be estimated reliably.

- Move the measurement antenna with a positioner to another location k and repeat step 2 to record frequency responses HkmΔf,nΔT of all stepped channel snapshots.

- Repeat step 3 to record frequency responses at all k=1,…,K spatial sample points.

- Stack measured time and frequency samples to a vector and calculate correlation between the first spatial sample point (i.e. k=1) and other spatial points k=1,…K

- ρk=corrvecH1mΔf,nΔT,vecHkmΔf,nΔT

- Take the theoretical reference spatial correlation of the corresponding spatial sample points. Plot both the measured and theoretical curves.

- Calculate the weighted RMS correlation error between the measured and the reference.

Figure 7.4.1.3-1: Configuration for spatial correlation validation

Time and frequency samples

The number of temporal snapshots N and frequency samples M is TBD. They must be chosen to minimize the validation measurement time, but sufficiently high to keep an adequate correlation estimation accuracy. It is beneficial to choose the time sampling interval ΔT larger than the coherence time of the channel model. such that the recorded time samples represent independent fading occasions. The same principle applies also to frequency sampling interval Δf and the channel coherence bandwidth.

Spatial samples

The spatial samples for the correlation validation measurement are on the circumference of the quiet zone, as illustrated in Figure 7.4.1.3-2. The test zone is a circle with 20 cm diameter in the horizontal plane. The reference point (denoted by a red marker) is in AoA 270. The mean AoAs of the CDL-A and CDL-C models are slightly different, but the underlying geometry for the CDL model indicates that the mean AoA (or assumed LoS direction) of the model is 180°. The reference point orientation of the validation measurement is proposed to be with 90° offset to the channel model reference AoA to enable accurate sampling of the main lobe of the spatial correlation curve. The reference point orientation must be defined in the channel model coordinate system instead of the chamber/probe coordinate system to enable optimization of OTA model implementation to achieve better alignment with the cluster AoAs and probe directions. In order to have spatial samples that yield reasonable measurement times and adequately capture the main lobe of the correlation curve, a non-uniform sampling is used where the first quadrant i.e., 270-180, is sampled with dense sampling compared to the rest of the circle. The spacing of the spatial samples is summarized in Table 7.4.1.3-1 for test frequencies less than 1800 MHz and equal to or greater than 1800 MHz.

Table 7.4.1.3-1: Spacing of Spatial Samples

| Test Frequencies [MHz] | First quadrant of test zone circumference (270o-180o) | Remaining quadrants |
| --- | --- | --- |
| 617, 722, 836.5 1575.42 | /15 | /4 |
| 1800, 2132.50, 2450, 3600, 4700 | /10 | /2 |

Figure 7.4.1.3-2: Spatial sampling for spatial correlation validation measurement for test frequencies less than and equal to or greater than 1800 MHz: 617 MHz spatial sampling (left) and 4700 MHz spatial sampling (right).

Reference Spatial Correlation Curves

The spatial correlation validation reference curves are tabulated in Tables 7.4.1.3-2 and 7.4.1.3-3 for CDL-A UMi and in Tables 7.4.1.3-4 and 7.4.1.3-5 for CDL-C UMa for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes.

Table 7.4.1.3-2: Spatial correlation reference curves for CDL-A UMi model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies below 1800 MHz

| 617 MHz |  | 722 MHz |  | 836.5 MHz |  | 1575.42 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref |
| 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 |
| 251.4 | 0.999 | 254.1 | 0.999 | 256.3 | 0.999 | 262.7 | 0.999 |
| 232.9 | 0.997 | 238.3 | 0.997 | 242.6 | 0.996 | 255.5 | 0.996 |
| 214.3 | 0.992 | 222.4 | 0.993 | 228.9 | 0.993 | 248.2 | 0.992 |
| 195.8 | 0.981 | 206.6 | 0.984 | 215.2 | 0.987 | 240.9 | 0.987 |
| 110.40 | 0.809 | 190.7 | 0.969 | 201.6 | 0.975 | 233.7 | 0.982 |
| 40.80 | 0.823 | 120.52 | 0.778 | 187.9 | 0.955 | 226.4 | 0.977 |
| 331.21 | 0.96 | 61.05 | 0.731 | 128.66 | 0.751 | 219.1 | 0.971 |
|  |  | 1.57 | 0.88 | 77.33 | 0.645 | 211.9 | 0.962 |
|  |  | 302.09 | 0.99 | 25.99 | 0.762 | 204.6 | 0.949 |
|  |  |  |  | 334.66 | 0.928 | 197.3 | 0.929 |
|  |  |  |  | 283.32 | 0.998 | 190.0 | 0.903 |
|  |  |  |  |  |  | 182.8 | 0.868 |
|  |  |  |  |  |  | 152.74 | 0.620 |
|  |  |  |  |  |  | 125.48 | 0.363 |
|  |  |  |  |  |  | 98.23 | 0.299 |
|  |  |  |  |  |  | 70.97 | 0.364 |
|  |  |  |  |  |  | 43.71 | 0.460 |
|  |  |  |  |  |  | 16.45 | 0.58 |
|  |  |  |  |  |  | 349.20 | 0.71 |
|  |  |  |  |  |  | 321.94 | 0.86 |
|  |  |  |  |  |  | 294.68 | 0.97 |

Table 7.4.1.3-3: Spatial correlation reference curves for CDL-A UMi model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies equal to or greater than 1800 MHz

| 1800 MHz |  | 2132.5 MHz |  | 2450 MHz |  | 3600 MHz |  | 4700 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref |
| 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 |
| 260.9 | 0.998 | 261.9 | 0.998 | 263.0 | 0.997 | 265.2 | 0.997 | 266.3 | 0.997 |
| 251.7 | 0.991 | 253.9 | 0.990 | 256.0 | 0.990 | 260.5 | 0.990 | 262.7 | 0.990 |
| 242.6 | 0.981 | 245.8 | 0.980 | 249.0 | 0.979 | 255.7 | 0.979 | 259.0 | 0.979 |
| 233.5 | 0.967 | 237.8 | 0.967 | 242.0 | 0.966 | 250.9 | 0.966 | 255.4 | 0.969 |
| 224.3 | 0.951 | 229.7 | 0.952 | 234.9 | 0.951 | 246.1 | 0.950 | 251.7 | 0.956 |
| 215.2 | 0.932 | 221.7 | 0.933 | 227.9 | 0.932 | 241.4 | 0.932 | 248.1 | 0.942 |
| 206.0 | 0.906 | 213.6 | 0.911 | 220.9 | 0.913 | 236.6 | 0.908 | 244.4 | 0.922 |
| 196.9 | 0.877 | 205.6 | 0.883 | 213.9 | 0.888 | 231.8 | 0.881 | 240.8 | 0.896 |
| 187.8 | 0.845 | 197.5 | 0.854 | 206.9 | 0.862 | 227.1 | 0.857 | 237.1 | 0.872 |
| 134.3 | 0.748 | 189.5 | 0.823 | 199.9 | 0.833 | 222.3 | 0.832 | 233.5 | 0.842 |
| 88.6 | 0.729 | 181.4 | 0.795 | 192.9 | 0.805 | 217.5 | 0.815 | 229.8 | 0.817 |
| 43.0 | 0.833 | 139.7 | 0.737 | 185.9 | 0.783 | 212.7 | 0.800 | 226.1 | 0.792 |
| 357.3 | 0.953 | 99.5 | 0.725 | 144.9 | 0.742 | 208.0 | 0.792 | 222.5 | 0.775 |
| 311.6 | 0.978 | 59.2 | 0.753 | 109.9 | 0.754 | 203.2 | 0.785 | 218.8 | 0.760 |
|  |  | 18.9 | 0.884 | 74.8 | 0.727 | 198.4 | 0.782 | 215.2 | 0.753 |
|  |  | 338.6 | 0.970 | 39.8 | 0.778 | 193.7 | 0.781 | 211.5 | 0.750 |
|  |  | 298.4 | 0.982 | 4.7 | 0.901 | 188.9 | 0.786 | 207.9 | 0.753 |
|  |  |  |  | 329.7 | 0.974 | 184.1 | 0.795 | 204.2 | 0.760 |
|  |  |  |  | 294.6 | 0.980 | 156.1 | 0.886 | 200.6 | 0.775 |
|  |  |  |  |  |  | 132.3 | 0.952 | 196.9 | 0.792 |
|  |  |  |  |  |  | 108.4 | 0.949 | 193.3 | 0.817 |
|  |  |  |  |  |  | 84.6 | 0.906 | 189.6 | 0.840 |
|  |  |  |  |  |  | 60.7 | 0.830 | 185.9 | 0.865 |
|  |  |  |  |  |  | 36.9 | 0.741 | 182.3 | 0.888 |
|  |  |  |  |  |  | 13.0 | 0.774 | 161.7 | 0.978 |
|  |  |  |  |  |  | 349.1 | 0.894 | 143.5 | 0.945 |
|  |  |  |  |  |  | 325.3 | 0.966 | 125.2 | 0.926 |
|  |  |  |  |  |  | 301.4 | 0.969 | 106.9 | 0.926 |
|  |  |  |  |  |  | 277.6 | 0.994 | 88.6 | 0.948 |
|  |  |  |  |  |  |  |  | 70.4 | 0.948 |
|  |  |  |  |  |  |  |  | 52.1 | 0.896 |
|  |  |  |  |  |  |  |  | 33.8 | 0.747 |
|  |  |  |  |  |  |  |  | 15.5 | 0.682 |
|  |  |  |  |  |  |  |  | 357.3 | 0.799 |
|  |  |  |  |  |  |  |  | 339.0 | 0.912 |
|  |  |  |  |  |  |  |  | 320.7 | 0.956 |
|  |  |  |  |  |  |  |  | 302.4 | 0.968 |
|  |  |  |  |  |  |  |  | 284.2 | 0.973 |

Table 7.4.1.3-4: Spatial correlation reference curves for CDL-C UMa model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies below 1800 MHz

| 617 MHz |  | 722 MHz |  | 836.5 MHz |  | 1575.42 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref |
| 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 |
| 251.4 | 0.999 | 254.1 | 0.999 | 256.3 | 0.999 | 262.7 | 0.999 |
| 232.9 | 0.997 | 238.3 | 0.997 | 242.6 | 0.996 | 255.5 | 0.996 |
| 214.3 | 0.992 | 222.4 | 0.993 | 228.9 | 0.993 | 248.2 | 0.992 |
| 195.8 | 0.981 | 206.6 | 0.984 | 215.2 | 0.987 | 240.9 | 0.987 |
| 110.40 | 0.809 | 190.7 | 0.969 | 201.6 | 0.975 | 233.7 | 0.982 |
| 40.80 | 0.823 | 120.52 | 0.778 | 187.9 | 0.955 | 226.4 | 0.977 |
| 331.21 | 0.96 | 61.05 | 0.731 | 128.66 | 0.751 | 219.1 | 0.971 |
|  |  | 1.57 | 0.88 | 77.33 | 0.645 | 211.9 | 0.962 |
|  |  | 302.09 | 0.99 | 25.99 | 0.762 | 204.6 | 0.949 |
|  |  |  |  | 334.66 | 0.928 | 197.3 | 0.929 |
|  |  |  |  | 283.32 | 0.998 | 190.0 | 0.903 |
|  |  |  |  |  |  | 182.8 | 0.868 |
|  |  |  |  |  |  | 152.74 | 0.620 |
|  |  |  |  |  |  | 125.48 | 0.363 |
|  |  |  |  |  |  | 98.23 | 0.299 |
|  |  |  |  |  |  | 70.97 | 0.364 |
|  |  |  |  |  |  | 43.71 | 0.460 |
|  |  |  |  |  |  | 16.45 | 0.58 |
|  |  |  |  |  |  | 349.20 | 0.71 |
|  |  |  |  |  |  | 321.94 | 0.86 |
|  |  |  |  |  |  | 294.68 | 0.97 |

Table 7.4.1.3-5: Spatial correlation reference curves for CDL-C UMa model for a vertically polarized MPAC OTA setup with 16 uniformly spaced probes at FR1 test frequencies equal to or greater than 1800 MHz

| 1800 MHz |  | 2132.5 MHz |  | 2450 MHz |  | 3600 MHz |  | 4700 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref | Azimuth [o] | ref |
| 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 | 270.0 | 1.000 |
| 260.9 | 0.998 | 261.9 | 0.998 | 263.0 | 0.997 | 265.2 | 0.997 | 266.3 | 0.996 |
| 251.7 | 0.991 | 253.9 | 0.991 | 256.0 | 0.992 | 260.5 | 0.989 | 262.7 | 0.988 |
| 242.6 | 0.984 | 245.8 | 0.984 | 249.0 | 0.983 | 255.7 | 0.979 | 259.0 | 0.976 |
| 233.5 | 0.976 | 237.8 | 0.975 | 242.0 | 0.975 | 250.9 | 0.969 | 255.4 | 0.966 |
| 224.3 | 0.967 | 229.7 | 0.966 | 234.9 | 0.965 | 246.1 | 0.960 | 251.7 | 0.957 |
| 215.2 | 0.955 | 221.7 | 0.955 | 227.9 | 0.953 | 241.4 | 0.954 | 248.1 | 0.951 |
| 206.0 | 0.936 | 213.6 | 0.940 | 220.9 | 0.939 | 236.6 | 0.947 | 244.4 | 0.945 |
| 196.9 | 0.908 | 205.6 | 0.918 | 213.9 | 0.921 | 231.8 | 0.937 | 240.8 | 0.940 |
| 187.8 | 0.863 | 197.5 | 0.888 | 206.9 | 0.898 | 227.1 | 0.925 | 237.1 | 0.935 |
| 134.3 | 0.309 | 189.5 | 0.846 | 199.9 | 0.867 | 222.3 | 0.903 | 233.5 | 0.928 |
| 88.6 | 0.269 | 181.4 | 0.793 | 192.9 | 0.829 | 217.5 | 0.876 | 229.8 | 0.918 |
| 43.0 | 0.396 | 139.7 | 0.280 | 185.9 | 0.786 | 212.7 | 0.837 | 226.1 | 0.902 |
| 357.3 | 0.619 | 99.5 | 0.252 | 144.9 | 0.245 | 208.0 | 0.798 | 222.5 | 0.882 |
| 311.6 | 0.879 | 59.2 | 0.257 | 109.9 | 0.299 | 203.2 | 0.753 | 218.8 | 0.851 |
|  |  | 18.9 | 0.471 | 74.8 | 0.215 | 198.4 | 0.708 | 215.2 | 0.816 |
|  |  | 338.6 | 0.661 | 39.8 | 0.251 | 193.7 | 0.669 | 211.5 | 0.767 |
|  |  | 298.4 | 0.937 | 4.7 | 0.489 | 188.9 | 0.624 | 207.9 | 0.708 |
|  |  |  |  | 329.7 | 0.652 | 184.1 | 0.580 | 204.2 | 0.651 |
|  |  |  |  | 294.6 | 0.946 | 156.1 | 0.175 | 200.6 | 0.580 |
|  |  |  |  |  |  | 132.3 | 0.565 | 196.9 | 0.516 |
|  |  |  |  |  |  | 108.4 | 0.745 | 193.3 | 0.444 |
|  |  |  |  |  |  | 84.6 | 0.820 | 189.6 | 0.383 |
|  |  |  |  |  |  | 60.7 | 0.750 | 185.9 | 0.310 |
|  |  |  |  |  |  | 36.9 | 0.493 | 182.3 | 0.229 |
|  |  |  |  |  |  | 13.0 | 0.120 | 161.7 | 0.445 |
|  |  |  |  |  |  | 349.1 | 0.272 | 143.5 | 0.750 |
|  |  |  |  |  |  | 325.3 | 0.498 | 125.2 | 0.879 |
|  |  |  |  |  |  | 301.4 | 0.843 | 106.9 | 0.813 |
|  |  |  |  |  |  | 277.6 | 0.991 | 88.6 | 0.733 |
|  |  |  |  |  |  |  |  | 70.4 | 0.740 |
|  |  |  |  |  |  |  |  | 52.1 | 0.873 |
|  |  |  |  |  |  |  |  | 33.8 | 0.944 |
|  |  |  |  |  |  |  |  | 15.5 | 0.643 |
|  |  |  |  |  |  |  |  | 357.3 | 0.250 |
|  |  |  |  |  |  |  |  | 339.0 | 0.178 |
|  |  |  |  |  |  |  |  | 320.7 | 0.375 |
|  |  |  |  |  |  |  |  | 302.4 | 0.726 |
|  |  |  |  |  |  |  |  | 284.2 | 0.929 |

Time Domain Alternative Method:

Time domain techniques can also be used to validate the spatial correlation. The spatial correlation validation measurement setup is illustrated in Figure 7.4.1.3-3. In this case a Signal generator transmits a CW signal through the MIMO test system. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyzer and the measured signal is stored for postprocessing.

Figure 7.4.1.3-3: Configuration for spatial correlation validation based on time domain techniques

For each spatial point, the channel emulator should issue a trigger signal each time fading is started. For each point collect a time domain trace with the signal analyzer, when done, stop fading. Data recording is synchronized with the channel emulator trigger.

Follow the same procedure to postprocess the data and calcalate the spatial correlation by setting m to 1. The settings for the Signal Generator and Signal Analyzer are in Table 7.4.1.3-2 and 7.4.1.3-3 respectively.

Table 7.4.1.3-2: Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table 7.4.1-1 |
| Output power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table 7.4.1.3-3: Signal Analyzer Settings

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table 7.4.1-1 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | At least 16s. Channel Model length should be the same or greater than the observation time. |

## <End of Changes>