3GPP TSG-RAN4 Meeting #97e	R4-2017601

# Electronic Meeting, 2 - 13 Nov, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for eMIMO demod requirements - General and Applicability rule |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Perf |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Under eMIMO WI, PDSCH demodulation requirements are agreed to be defined for multi-TRP multi-DCI and single DCI transmission schemes. The applicability of the newly defined tests needs to be captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add features in section 5.1.1.3 and define applicability for the newly defined tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability of UE demodulation requirements for multi-TRP is not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sections 5.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### 5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] |  | Test type |  |  |  |  | Test list |  |  |  |  |  |  | Applicability notes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver |  |  | FR1 FDD |  | PDSCH |  |  |  | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |  |  |  |
|  |  |  | FR1 TDD |  | PDSCH |  |  |  | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |  |  |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) |  |  | FR1 FDD |  | PDSCH |  |  |  | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |  |  |  |
|  |  |  | FR1 TDD |  | PDSCH |  |  |  | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |  |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) |  |  | NR CA |  | SDR |  |  |  | Clause 5.5A.1 |  |  | 1)Up to 16 DL carriers 2)Same numerology across carrier for data/control channel at a given time |  |
| Single DCI based SDM transmission for multi-TRP (singleDCI-SDM-scheme-r16) |  | FR1 FDD |  | PDSCH |  |  |  | Clause 5.2.2.1.11 Clause 5.2.3.1.11 |  |  |  |  |  |  |
|  |  | FR1 TDD |  | PDSCH |  |  |  | Clause 5.2.2.2.11 Clause 5.2.3.2.11 |  |  |  |  |  |  |
| Multi DCI based multi-TRP support (multiDCI-MultiTRP-r16) |  | FR1 FDD |  | PDSCH |  |  | Clause 5.2.2.1.12 Clause 5.2.3.1.12 |  |  |  |  |  |  |  |
|  |  | FR1 TDD |  | PDSCH |  |  | Clause 5.2.2.2.12 Clause 5.2.3.2.12 |  |  |  |  |  |  |  |
| Single DCI based FDM Scheme-A for multi-TRP(supportFDM-SchemeA-r16) |  | FR1 FDD |  | PDSCH |  |  |  | Clause 5.2.2.1.13 Clause 5.2.3.1.13 |  |  |  |  |  |  |
|  |  | FR1 TDD |  | PDSCH |  |  |  | Clause 5.2.2.2.13 Clause 5.2.3.2.13 |  |  |  |  |  |  |
| Sinle DCI based inter-slot TDM for multi-TRP (supportInter-slotTDM-r16) |  | FR1 FDD |  | PDSCH |  |  |  | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  |  |  |  |  |  |
|  |  | FR1 TDD |  | PDSCH |  |  |  | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |  |  |  |  |  |

End of Change 1

Start of Change 2

#### 5.1.1.6	Applicability of different requirements with Multi-TRP

The applicability rules for requirements with multi-TRP transmission schemes in section 5 are specified in Table 5.1.1.6-1.

Table 5.1.1.6-1: Applicability of requirements with Multi-TRP Transmission

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.12 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.12 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.6 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.12 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.2.2.11 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.12 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.2.2.13 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.6 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.2.2.14 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.12 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.3.1.11 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.12 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.3.1.13 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.6 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.3.1.14 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.12 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.3.2.11 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.12 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.3.2.13 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.6 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.3.2.14 (Test 1-1) |  |

End of Change 2