3GPP TSG-RAN4#98-e	R4-2100135

25th January – 5th February 2021

Source: 	Nokia, Skyworks Inc.

Title: 					n24 / band 24 A-MPR proposal

Agenda Item: 			9.26.1

Document for:	Approval

# 1	Introduction

This contribution proposes A-MPR concept for b24/n24.

# 2	Simulation Scenario and Results

Relevant b24 and n24 simulation scenarios, assumptions and results are presented in [1]. In addition, measurement results are provided in [2]. For the A-MPR presented in the following, both of these result sets are considered.

# 3	A-MPR Concept for n24

Based on the simulation [1] and measurement [2] results, AMPR is necessary in the two following scenarios. Below, also relevant AMPR concept is presented.

Channel BW = 5 MHz, center frequency = 1630 MHz,

IF RBStart <= ceil{3/(SCS/15 kHz)}, AND LCRB <= ceil{17/(SCS/15 kHz)},

THEN AMPR = 14 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz

ELSE IF RBStart <= ceil{3/(SCS/15 kHz)}, AND LCRB > ceil{17/(SCS/15 kHz)},

THEN AMPR = 6 dB

ELSE IF RBStart <= ceil{8/(SCS/15 kHz)},

THEN AMPR = 4 dB.

Figure 1. An informational, illustrative example of the AMPR concept for the 5 MHz channel at center frequency of 1630 MHz, in the AMPR itself also measurement data is considered.

Channel BW = 10 MHz, center frequency = 1632.5 MHz,

IF RBStart < ceil{3/(SCS/15 kHz)}, AND LCRB <= ceil{8/(SCS/15 kHz)},

THEN AMPR = 12 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz

ELSE IF RBStart < ceil{3/(SCS/15 kHz)}, AND LCRB > ceil{8/(SCS/15 kHz)},

THEN AMPR = 8 dB

ELSE IF RBStart <= ceil{18/(SCS/15 kHz)},

THEN AMPR = 6 dB.

Figure 2. An informational, illustrative example of the AMPR concept for the 5 MHz channel at center frequency of 1630 MHz, in the AMPR itself also measurement data is considered.

# 2	A-MPR Table for b24

Based on the simulation [1] and measurement [2] results, AMPR is necessary as shown in Table 1.

Table 2. AMPR for b24

| Channel BW (MHz) | Center freq. (MHz) | Parameter | Region A | Region B |
| --- | --- | --- | --- | --- |
| 5 | 1630 | RBStart | ≤ 8 | ≤ 8 |
|  |  | LCRB | ≤ 8 | > 8 |
|  |  | AMPR [dB] | 3 | 2 |
| 10 | 1632.5 | RBStart | ≤ 18 | ≤ 18 |
|  |  | LCRB | ≤ 12 | > 12 |
|  |  | AMPR [dB] | 5 | 7 |

# 4	Conclusion

We proposed the following A-MPR concept for n24:

Channel BW = 5 MHz, center frequency = 1630 MHz,

IF RBStart <= ceil{3/(SCS/15 kHz)}, AND LCRB <= ceil{17/(SCS/15 kHz)},

THEN AMPR = 14 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz

ELSE IF RBStart <= ceil{3/(SCS/15 kHz)}, AND LCRB > ceil{17/(SCS/15 kHz)},

THEN AMPR = 6 dB

ELSE IF RBStart <= ceil{8/(SCS/15 kHz)},

THEN AMPR = 4 dB.

Channel BW = 10 MHz, center frequency = 1632.5 MHz,

IF RBStart < ceil{3/(SCS/15 kHz)}, AND LCRB <= ceil{8/(SCS/15 kHz)},

THEN AMPR = 12 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz

ELSE IF RBStart < ceil{3/(SCS/15 kHz)}, AND LCRB > ceil{8/(SCS/15 kHz)},

THEN AMPR = 8 dB

ELSE IF RBStart <= ceil{18/(SCS/15 kHz)},

THEN AMPR = 6 dB.

In addition, we proposed the following A-MPR table for b24:

| Channel BW (MHz) | Center freq. (MHz) | Parameter | Region A | Region B |
| --- | --- | --- | --- | --- |
| 5 | 1630 | RBStart | ≤ 8 | ≤ 8 |
|  |  | LCRB | ≤ 8 | > 8 |
|  |  | AMPR [dB] | 3 | 2 |
| 10 | 1632.5 | RBStart | ≤ 18 | ≤ 18 |
|  |  | LCRB | ≤ 12 | > 12 |
|  |  | AMPR [dB] | 5 | 7 |

# 5	References

[1]	R4-2100134	 Simulation results for n24/band 24 A-MPR, Nokia, RAN4#98e

[2]	R4-2100545 discussion Band 24, n24 and n99 A-MPR Skyworks Solutions IncRAN4#98e