3GPP TSG-RAN WG4 Meeting#98             	R4-2100147

E-meeting, 21st Jan – 5th Feb, 2021

Title:	TP to TR 37.717-41-11: DC_1-3-20-40_n78

Source:	Nokia, Telefonica

Agenda item:	9.6.2

Document for:	Discussion

# 1	Introduction

This TP introduces DC_1A-3A-20A-40A_n78A and DC_1A-3A-20A-40C_n78A into the TR 37.717-41-11.

******************************* Start of the TP ***********************************

### 5.1.X	DC_1-3-20-40_n78

#### 5.1.X.1	Configuration for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-20A-40A_n78A DC_1A-3A-20A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_40A_n78A |

#### 5.1.X.2	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from DC_1-3-20_n41-n78 which is very similar combination.

Table 5.1.X.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-20-40_n78 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
|  | 40 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.X.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-20-40_n78 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
|  | 40 | 0 |
|  | n78 | 0.5 |

#### 5.1.X.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

******************************* End of the TP ***********************************