3GPP TSG-RAN4 Meeting #98-e	R4-2100174

# Online, 25 Jan. – 5 Feb., 2021

Agenda item:	4.7

Source:	Apple

Title:	On SSB-less SCell activation

Document for:	Discussion

# Introduction

In last meeting, the SSB-less SCell activation was discussed but no conclusion has been achieved. There are some open issues summarized in [1].

- RAN4 to define SSB-less FR1 SCell activation delay requirement as follows: - If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the Scell on that FR1 band, if the UE [supporting scellWithoutSSB] is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms[, provided - the RS (s) of SCell being activated is (are) QCL-TypeA with TRS (s) of the SCell being activated and the TRS (s) is (are) QCL-TypeC with SSB (s) of one active serving cell on that FR1 band.] - FFS on non-contiguous intra-band CA

In this contribution we further discuss the SSB-less SCell activation.

# Discussion

If UE can support scellWithoutSSB (as duplicated below), UE cannot perform DL synchronization to target SCell by using the SSBs from the target SCell. So, the only timing UE could use is the timing of the existing serving cell which is in the same band as the target cell.

However, if the existing active serving CC and target SCC are in the intra-band non-contiguous BC, the MRTD between existing serving CC and target SCC is 3us, as defined in the following table.

In the note 1 of the above table, it identified that the potential issue that the MRTD might be larger than CP, and therefore UE cannot directly use the timing from existing active serving cell for an intra-band non-contiguous to-be-activated SCell. So, if companies would like to have such SSB-less SCell activation, we prefer to only consider the intra-band contiguous case since it’s too late to change the UE behavior in R15 for the other controversial scenario (e.g., intra-band non-contiguous case).

Proposal 1: If RAN4 agrees to introduce the requirement for FR1 SSB-less SCell activation in R15, only the intra-band contiguous case shall be considered.

If SSB-less SCell activation is to be added in R15, the requirement could be as below,

- If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms.

RAN4 may continue to consider the possibility of FR1 SSB-less SCell activation for intra-band non-contiguous in R16, but some side condition shall be added to apply the activation delay requirement as 3ms. We think the time difference between CCs for intra-band contiguous case could be used as a baseline, which means the time difference condition to apply the SSB-less SCell activation for intra-band NC case in R16 is like TAE in intra-band contiguous case, i.e., 260ns.

Proposal 2: if RAN4 agrees to introduce the FR1 SSB-less SCell activation for intra-band NC case in R16, the 3ms activation delay requirement only applies when time difference between active serving cell and intra-band NC to-be-activated SCell is less than 260ns.

With this discussion, the existing FR2 SCell activation might also need to be clarified with the UE capability. In the current requirement it was defined as below,

We think it’s quite similar as FR1 intra-band contiguous CA case, and the motivation to have such case without SMTC is also because the SSB-less scenario. So, only the UE who can support SSB-less feature could meet this 3ms activation delay requirement in FR2, and we propose to revise the current FR2 activation requirement without SMTC to include the UE capability condition. The CR for this FR2 SSB-less SCell activation requirement is provided in [2].

Proposal 3: the FR2 SCell activation without SMTC shall be revised as,

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

- the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

# Conclusion

In this contribution we further discuss the SSB-less SCell activation.

Proposal 1: If RAN4 agrees to introduce the requirement for FR1 SSB-less SCell activation in R15, only the intra-band contiguous case shall be considered.

Proposal 2: if RAN4 agrees to introduce the FR1 SSB-less SCell activation for intra-band NC case in R16, the 3ms activation delay requirement only applies when time difference between active serving cell and intra-band NC to-be-activated SCell is less than 260ns.

Proposal 3: the FR2 SCell activation without SMTC shall be revised as,

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

- the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

# References

[1] R4-2017035, WF on SSB-less SCell activation delay requirement, Qualcomm, RAN4 #97e

[2] R4-2100175, CR on FR2 SCell activation requirement, Apple, RAN4 #98e