3GPP TSG-RAN4 Meeting #98-e	R4-2100185

# Online, 25 Jan. – 5 Feb., 2021

Agenda item:	7.19.5

Source:	Apple

Title:		On R16 IDLE/INACTIVE RRM requirement with SMTC2-LP

Document for:	Discussion

# Introduction

In last RAN4 #97e meeting, it was agreed in R4-2017274 summary that,

The following proposals are agreeable

- The SMTC2-LP shall be captured in the corresponding baseline RRM requirements of IDLE/INACTIVE in R16

  - More details can be further discussed in RAN4#98e. CR R4-2014281 should be noted.

- The discussion on SMTC2-LP IDLE/INACTIVE RRM requirements starts without NR-U, HST or power saving. Depending on the conclusion, we can further discuss how or whether to extend it to NR-U/HST/PS requirement.

In this contribution, we discuss the RAN4 IDLE/INACTIVE RRM requirement with SMTC2-LP without NR-U, HST or power saving feature and the CR is provided in [1].

# Discussion

In TS38.331, it was defined that,

If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselection), the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2-LP configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc configuration for that frequency. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell or serving cell (for cell reselection) meeting the above condition.

The signalling design is as below for both intra-frequency and inter-frequency measurement in IDLE/INACTIVE mode,

smtc2-LP-r16 in SIB2 Measurement timing configuration for intra-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP-r16. The timing offset and duration are equal to the offset and duration indicated in smtc in intraFreqCellReselectionInfo. The periodicity in smtc2-LP-r16 can only be set to a value strictly larger than the periodicity in smtc in intraFreqCellReselectionInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP-r16 cannot be configured). The pci-List, if present, includes the physical cell identities of the intra-frequency neighbour cells with Long Periodicity. If smtc2-LP-r16 is absent, the UE assumes that there are no intra-frequency neighbour cells with a Long Periodicity.

smtc2-LP-r16 in SIB4 Measurement timing configuration for inter-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP-r16. The timing offset and duration are equal to the offset and duration indicated in smtc in InterFreqCarrierFreqInfo. The periodicity in smtc2-LP-r16 can only be set to a value strictly larger than the periodicity in smtc in InterFreqCarrierFreqInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP-r16 cannot be configured). The pci-List, if present, includes the physical cell identities of the inter-frequency neighbour cells with Long Periodicity. If smtc2-LP-r16 is absent, the UE assumes that there are no inter-frequency neighbour cells with a Long Periodicity.

In most of RRM requirements for IDLE/INACTIVE, we use DRX cycle to define the delay, but in some side conditions and interruption requirements, we used SMTC as the periodicity to determine the requirement. We list all SMTC related RRM requirement as below in IDLE/INACTIVE mode for analysis.

- Measurement and evaluation of serving cell

In TS38.133, the serving cell evaluation requirement for IDLE/INACTIVE mode was defined as,

However, in RAN2 TS38.331, it was defined that smtc2-LP-r16 is measurement timing configuration for inter-frequency neighbour cells with a Long Periodicity (LP). Thus, the legacy serving cell measuerment and evalution requirement in IDLE/INACTIVE mode shall not be changed by introducing SMTC2-LP.

Proposal 1: the legacy serving cell measurement and evaluation requirement in IDLE/INACTIVE mode shall not be changed by introducing SMTC2-LP.

- Measurements of intra-frequency NR cells

In TS38.133, the intra-frequency neighbor cells measurement requirement for IDLE/INACTIVE mode was defined as,

In order to figure out whether the condition of M2 shall be revised by introducing SMTC2-LP or not, we think it needs to consider different scenarios.

If no identified neighbor cell is in the PCI list of SMTC2-LP, it’s not necessary to use the SMTC2-LP in the intra-frequency neighbor cells measurement requirement.

If at least one identified intra-frequency neighbor cell is in the PCI list of SMTC2-LP, there might be three sub-scenarios for consideration:

- when SMTC < SMTC2-LP ≤ 20ms, or

- when 20ms < SMTC < SMTC2-LP, or

- when SMTC ≤ 20ms < SMTC2-LP

The reason of this M2 extension is: when PO(paging occasion) is colliding with target SSB and they cannot be received concurrently, UE may need wake up outside DRX active time to perform intra-frequency neighbor cell measurement. ~20ms is an assumption that UE could keep the RF chain ON to perform intra-frequency neighbor cell measurement even though the DRX active time is less than 20ms, as shown in the following figure 1.

Figure 1. the condition of extension factor M2

When SMTC < SMTC2-LP ≤ 20ms, the UE can keep the RF chain ON to conduct this intra-frequency neighbor cell measurement, and no extension is needed for measurement delay requirement, i.e., M2=1, as shown in figure 2.

Figure 2. When SMTC<SMTC2-LP ≤ 20ms

When 20ms < SMTC < SMTC2-LP, both the SMTC based and SMTC2-LP based SSBs are outside the UE RF ON period for DRX active status, and therefore the M2 extension is needed, i.e., M2=1.5, as shown in figure 3.

Figure 3. When 20ms < SMTC < SMTC2-LP

When SMTC ≤ 20ms < SMTC2-LP, the SMTC2-LP may be outside the UE RF ON period for DRX active status, so the UE needs to wake up again to measurement the neighbor cells in the PCI list of SMTC2-LP. However, for the identified neighbor cells which are not in the PCI list of SMTC2-LP, UE may choose to measure them in the 20ms RF ON period. As commented by other companies in last meeting, in connected mode, we differentiated the cell measurement delay for smtc1 and smtc2 cases. When SMTC ≤ 20ms < SMTC2-LP, for the identified cells in the PCI list of SMCT2-LP, M2=1.5 shall apply, while for the other identified cells M2=1 shall apply.

Summary of analysis:

If no identified neighbor cell is in the PCI list of SMTC2-LP, no change to the legacy intra-frequency cell measurement requirement in IDLE/INACTIVE.

If at least one identified neighbor cell is in the PCI list of SMTC2-LP,

when SMTC < SMTC2-LP ≤ 20ms, M2=1 for all intra-frequency identified cells measurement in IDLE/INACTIVE.

when 20ms < SMTC < SMTC2-LP, M2=1.5 for all intra-frequency identified cells measurement in IDLE/INACTIVE.

when SMTC ≤ 20ms < SMTC2-LP, for the intra-frequency identified cells in the PCI list of SMCT2-LP, M2=1.5, while for the other intra-frequency identified cells M2=1.

Based on the above summary, we propose:

Proposal 2: revise the note 2 in Table 4.2.2.3-1 as:

M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified.

- Measurements of inter-frequency NR cells

In TS38.133, the applicability condition of inter-frequency neighbor cells measurement requirement for IDLE/INACTIVE mode was defined as,

This condition is used to preclude the scenario that inter-frequency cell has no chance to be measured. In the figure 4, if DRX cycle =320ms and both inter-frequency and intra-frequency SMTC=160ms (intra-frequency SSB cannot be measured during PO), then in the second SMTC occasion only intra-frequency SSB could be measured.

In order to figure out whether the applicability condition of inter-frequency measurement shall be revised by introducing SMTC2-LP or not, we think it needs to consider different scenarios as well.

Figure 4. Illustration of the applicability condition for inter-frequency measurement requirement

If no identified neighbor cell is in the PCI list of SMTC2-LP for intra-frequency and inter-frequency carriers, it’s not necessary to use the SMTC2-LP in this inter-frequency neighbor cells measurement requirement.

In RAN2 TS38.331, it defined that “if smtc indicates sf160, smtc2-LP-r16 cannot be configured”. So, following cases are considered:

If at least one identified neighbor cell is in the PCI list of SMTC2-LP for intra-frequency carrier, but no identified neighbor cell is in the PCI list of SMTC2-LP for inter-frequency carrier,

- TSMTC_intra shall be based on SMTC2-LP for intra-frequency carrier, TSMTC_inter shall be based on SMTC for inter-frequency carrier

If no identified neighbor cell is in the PCI list of SMTC2-LP for intra-frequency carrier, but at least one identified neighbor cell is in the PCI list of SMTC2-LP for inter-frequency carrier,

- TSMTC_intra shall be based on SMTC for intra-frequency carrier, TSMTC_inter shall be the actual SMTC periodicity used by the target cell being identified on this inter-frequency carrier.

If at least one identified neighbor cell is in the PCI list of SMTC2-LP for intra-frequency carrier, and at least one identified neighbor cell is in the PCI list of SMTC2-LP for inter-frequency carrier,

- TSMTC_intra shall be based on SMTC for intra-frequency carrier, TSMTC_inter shall be the actual SMTC periodicity used by the target cell being identified on this inter-frequency carrier.

Proposal 3: revise the applicability condition in TS38.133 section 4.2.2.4 as:

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2.2.4-1 under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where

TSMTC_intra is periodicity of the SMTC configured for the intra-frequency carrier if no identified intra-frequency cell is in the PCI list of SMTC2-LP on this intra-frequency carrier; TSMTC_intra is periodicity of the SMTC2-LP configured for the intra-frequency carrier if at least one identified intra-frequency cell is in the PCI list of SMTC2-LP on this intra-frequency carrier.

TSMTC_inter is the actual SMTC periodicity used by the inter-frequency cell being identified.

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS38.304 [1].

- Maximum interruption in paging reception

In TS38.133, the maximum interruption requirement in paging reception for IDLE/INACTIVE mode was defined as,

If the target cell is in the PCI list of SMTC2-LP, then UE shall follow the SMTC2-LP to read the SI from target cell. So, we propose that,

Proposal 4: revise the maximum interruption requirement in paging reception in TS38.133 section 4.2.2.6 as:

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

# Conclusion

In this contribution, we discuss the RAN4 IDLE/INACTIVE RRM requirement with SMTC2-LP and the CR is provided in [1].

Proposal 1: the legacy serving cell measurement and evaluation requirement in IDLE/INACTIVE mode shall not be changed by introducing SMTC2-LP.

Proposal 2: revise the note 2 in Table 4.2.2.3-1 as:

M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified.

Proposal 3: revise the applicability condition in TS38.133 section 4.2.2.4 as:

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2.2.4-1 under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where

TSMTC_intra is periodicity of the SMTC configured for the intra-frequency carrier if no identified intra-frequency cell is in the PCI list of SMTC2-LP on this intra-frequency carrier; TSMTC_intra is periodicity of the SMTC2-LP configured for the intra-frequency carrier if at least one identified intra-frequency cell is in the PCI list of SMTC2-LP on this intra-frequency carrier.

TSMTC_inter is the actual SMTC periodicity used by the inter-frequency cell being identified.

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS38.304 [1].

Proposal 4: revise the maximum interruption requirement in paging reception in TS38.133 section 4.2.2.6 as:

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

# References

[1] R4-2100186, CR on R16 IDLE/INACTIVE RRM requirement with SMTC2-LP, Apple, RAN4 #98e