3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100212

# Electronic Meeting, 25 Jan – 5 Feb, 2021

Agenda item:	7.9.4.3

Source:	Apple Inc.

Title:	PMI reporting requirements with eType II codebook

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#97e PMI reporting requirements with eType II codebook was discussed and way forward [1] was agreed. In this contribution we present simulation results for PMI reporting with eType II codebook.

# 2. Discussion

In RAN4#97e, the following agreements were made for PMI reporting with eType II codebook.

- Introduce Rel-16 eType II codebook requirements with SU-MIMO set-up under the condition that with proper test parameters, test metric/test requirements and test procedure to ensure enough performance difference over than Type I i.e. UE which employ Type I reporting will fail in the test case - Test metric: - Following PMI (eType II )/Random PMI (Type I codebook) (gamma values) based on the assumption that there are ensure enough performance difference over than Type I i.e. UE which employ Type I reporting will fail in the test case - This test metric applied to UE which support eType II codebook feedback irrespective whether supporting Type I codebook feedback or not - FFS: Whether to check UE reported codebook not only within Type I codebook set - Further study and define proper performance requirements if needed under MU-MIMO scenario in Rel-17 performance enhancement WI.

For PMI reporting with Type II codebook with SU-MIMO test setup, the following simulation assumptions were made agreed:

- Number of CSI-RS ports:

  - 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4)

- Number of PMI Sub-bands per CQI Sub-band

  - R = 1

- Codebook parameter configuration

  - paramCombination-r16: 6, with L =4, pν =1/2, β=1/2

- Sub-band Size:

  - 4 for FDD with 15kHz SCS, 10MHz CBW

  - 8 for TDD with 30kHz SCS, 40MHz CBW

- Channel Model

  - TDLA30-5

- MIMO Correlation

  - XP Medium (as baseline)

- MCS and Rank

  - MCS 20 (64QAM Table), Rank 2

- Beam-Steering Model

  - How to specify beam steering model into specification.

    - Same as specified in B.2.3B.4A of TS 36.101

  - Enable two independent beam directions applying the beam for comparison both eType II and Type I cookbook simulation

The simulation results are presented below. The metric used is 90% max TP to be consistent with exiting PMI reporting requirements.

Table 1: Simulation results for eType II vs Type I codebook

| TDLA-30-5Hz XP-Medium |  | eType II |  | Type I |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR @90% Max TP | TP Gain | SNR @90% Max TP | TP Gain |
| FDD | 16x2 | 9.53 | 3.41 | 14.58 | 2.01 |
|  | 16x4 | 7.05 | 2.85 | 9.71 | 2.11 |
| TDD | 16x2 | 9.41 | 3.49 | 14.33 | 2.03 |
|  | 16x4 | 6.40 | 3.00 | 8.94 | 2.31 |

From the results above, we see that eType II has ~5dB performance gain over Type I with 2 RX. The TP gain measured at 90% max TP is also distinguishable between eType II and Type I. With lower test point, for example 70% Max TP, the throughput gain would be higher with eType II and more distinguishable from Type I. For the agreed simulation parameters, test point of SNR @ 90% Max TP is suitable.

Proposal #1: Define PMI reporting requirements with eType II codebook at 90% Max TP.

# 3. Conclusion

In this paper, we simulation results for PMI reporting requirements for eType II codebook. Our proposals are summarized below:

Proposal #1: Define PMI reporting requirements with eType II codebook at 90% Max TP.

# Reference

- R4-2017678, “Way forward on PMI reporting requirement for NR eMIMO”, Ericsson, Samsung.