3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100266

# Electronic Meeting, 25th January – 5th February, 2021

Source:		Verizon, Ericsson

Title:			TP for TR 37.826 for DC_2_n77

Agenda item:		9.20.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.826 to include DC_2_n77 according to the request in [1].

# Reference

[1] RP-20xxxx, Revised WID on High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band

# Text Proposal

<Start of Text Proposal>

## 6.x	DC_2A_n77A

### 6.x.1	Transmitter Characteristics

#### 6.x.1.1	Maximum Output Power

Table 6.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 6.x.1.2	Co-existence study

According to the PC3 CA_n2A-n77A study, the same results are listed in below,

- The 2nd harmonic mixing products from band 2 may fall into band n77 UL frequency range.

- The 2nd, 4th and 5th order IMD products generated by dual uplink from both band 2 and band n77 may fall into band 2 Rx frequency range.

Thus, additional MSD for IMD 2, 4 and 5 should be considered to mitigate the impact of the interference for PC2 DC_2A_n77A combination.

### 6.x.2	Receiver Characteristics

#### 6.x.2.1		MSD exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Table 6.x.2.1-1: MSD exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 2 | 9.1 | 8.0 | 7.0 | 6.7 |  |  |  |  |  |  |  |  |

#### 6.x.2.2	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

Table 6.x.2.2-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD for PC2 | Duplex mode | Source of IMD |
| Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_2A_n77A | 2 | 1855 | 5 | 25 | 1935 | 32 | FDD | IMD2 |
|  |  |  |  |  |  | 34.755 |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 20 | FDD | IMD4 |
|  |  |  |  |  |  | 22.755 |  |  |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 20 | FDD | IMD5 |
|  |  |  |  |  |  | 22.755 |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

<End of Text Proposal>