3GPP TSG-RAN WG4 Meeting #98-e	R4-2100289

# Electronic meeting, 25th Jan. – 05th Feb., 2021

Source:	LG Electronics, LG Uplus

Title:	MPR simulation assumptions for PC2 NR intra-band non-contiguous CA UE

Agenda item:	11.2.2.5

Document for:	Approval

1. Introduction

In last RAN plenary meeting, the PC2 NR intra-band non-contiguous CA was included in Rel-17 FR1 RF enhancement WI. Tthe work scope as follow in the agreed WI [1].

- 3) HPUE for TDD intra-band contiguous and non-contiguous UL CA

  - Take n41, n77 and n78 intra-band contiguous UL CA for examples

    - The two example intra-band contiguous UL CA configurations are under considerations

      - CA_n41C, CA_n78C, CA_n77C

  - Take n77 intra-band non-contiguous UL CA for example

    - One example intra-band non-contiguous UL CA configuration is under considerations: CA_n77(2A)

  - Investigate and specify the 26dBm power class for n41and n78 intra-band contiguous, and n77 intra-band contiguous/non-contiguous UL CA

    - Identify the impact of different UE architectures on the requirements

      - Power class relation between single CC and intra-band contiguous/non-contiguous CA on HPUE band is clarified if any

    - Specify the mechanism to meet SAR requirements if necessary

      - Mechanism for HPUE on single carrier can be a start point considering the same UL-DL configuration assumption

    - A-MPR requirement

  - Specify MPR requirements

In this contribution, we propose the example RF architectures for PC1.5 for n77/n78 NR UE in Rel-17.

2. MPR/A-MPR requirements for PC3 NR intra-band non-contiguous CA in Rel-16

In this section, we take a look at how PC 3 Intra-band NC CA was specified in Rel-16. In here, RAN4 also studied RF architecture as follow

    - 2PA/2LO : Can support as UE capability declaration if Wgap gets too wide

      - instantaneous TX bandwidth: >200MHz can be supported (and below obviously)

      - Number of TX chain: 2 (4TX to support UL MIMO)

    - 1PA/1LO : Restricted Tx bandwidth & relaxed in-gap ACLR requirement

      - instantaneous TX bandwidth: ~200MHz

      - Number of TX chain: 1 (2TX to support UL MIMO)

    - 1PA/2LO: same situation as 2PA/2LO. However, it is not recommended by two reason

      - inherent power backoff (insertion loss) issue [3]

      - One possible solution is to put 3dB combiners and switches in the PA

Finally, RAN4 only specified the MPR requirements based on 2PA/2LO RF architecture for NR intra-band non-contiguous CA in Rel-16.

Also, RAN4 defined some capability signaling for NR intra-band non-contiguous CA to distinguish Maximum UL frequency separation, PA architecture and max. number of UL MIMO layer.

Based on above RAN4 works, we share proposal as follow

Proposal 1: RAN4 should consider 2PA/2LO RF architecture as baseline for PC2 NR intra-band non-contiguous CA same as PC3 intra-band non-contiguous CA UE to derive MPR/A-MPR requirements in Rel-17.

Based on the above simulations assumptions, RAN4 specified new MPR/A-MPR requirements in TS38.101-1.

The following simulations assumptions [2] were considered for MPR requirements for PC3 NR intra-band non-contiguous CA in Rel-16.

3. MPR simulation assumptions for PC2 NR intra-band non-contiguous CA in Rel-17

Based on the above simulation assumptions in section 2, RAN4 can derive PC2 MPR requirements for NR intra-band NC CA UE.

Proposal 2: RAN4 evaluate PC2 MPR requirements based on above simulation assumptions in section2 in Rel-17.

Also, RAN4 need to study about the detail regulation requirements in n77 frequency range in global usage. Then RAN4 derive the A-MPR requirements to comply the specific regulation requirements in some regions. Hence we propose as follow

Proposal 3: RAN4 encourage to share the specific regional requirements in n77 for PC2 NR intra-band NC CA UE to derive A-MPR requirements

4. Conclusion

In this paper, we remind which the basic simulation assumptions are considered for MPR requirements for PC3 NR intra-band NC CA UE.

Based on the detail simulation parameters and baseline RF architecture in section 2, we share our proposals as follow

Proposal 1: RAN4 should consider 2PA/2LO RF architecture as baseline for PC2 NR intra-band non-contiguous CA same as PC3 intra-band non-contiguous CA UE to derive MPR/A-MPR requirements in Rel-17.

Proposal 2: RAN4 evaluate PC2 MPR requirements based on above simulation assumptions in section2 in Rel-17.

Proposal 3: RAN4 encourage to share the specific regional requirements in n77 for PC2 NR intra-band NC CA UE to derive A-MPR requirements

Reference

[1] RP-202799, “WID revision: RF requirements enhancement for NR frequency range,” Huawei

[2] R4-2005661, “WF on intra-band UL non-contiguous CA MPR,” Skyworks, Huawei, Qualcomm

[3] TS38.101-1

[4] TS38.101-3