3GPP TSG-WG RAN4 Meeting #98-e	R4-2100333

# Electronic meeting, 25th January – 5th February, 2021

Title: 	Discussion on the new SUL band n99 BS requirements

Source: 	Huawei, HiSilicon

Agenda item:	9.27.2

Document for:	Discussion

Introduction

At RAN#88e, a new work item related to introduction of SUL band in 1.6 GHz was approved in [1]. A work plan for the WI was also approved in [2] during RAN4#96e. The new proposed SUL band overlaps with the exact frequencies of band n24. With the previous experiences in introducing the SUL band n95 which overlaps with the TDD band n34, it is agreed in RAN4#97e in [3] that RAN4 introduces the corresponding RF requirements to facilitate complete introduction of the new SUL band n99.

For the BS requirements, introducing SUL band n99 involves the BS coexistence requirements: Additional spurious emission and co-location emission requirements. The related specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2.

In this paper, we discuss the necessary BS RF requirements to be specified in order to accomplish introduction of the new SUL band n99.

BS RF Requirements

Several BS specs need to be considered to introduce a new band. Taking TS 38.104 as an example, the corresponding parts shall be introduced for spurious emissions as below and in every other spec.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| NR Band n99 | 1626.5 – 1660.5 MHz | -49 dBm | 1 MHz |  |

Table 6.6.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Basic limits |  |  | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | WA BS | MR BS | LA BS |  |  |
| NR band n99 | 1626.5 – 1660.5 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

The related BS specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2.

Proposal:  Specify BS RF requirements for the new SUL band n99 following UL of band n24 in all the BS specs such as TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2, whenever necessary.

Conclusions

In this paper, we discuss the necessary RF requirements to be specified in order to accomplish introduction of the new SUL band n99. We propose:

Proposal:  Specify BS RF requirements for the new SUL band n99 following UL of band n24 in all the BS specs such as TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2, whenever necessary.

References

[1] RP-201251, New WID proposal on introduction of 1.6 GHz NR SUL band with same uplink frequency range of Band 24, Ligado Networks

[2] R4-2010745, Work plan for Introduction of 1.6 GHz NR SUL Band with same uplink frequency range of Band 24, Ligado Networks

[3] R4-2016897, WF on work item NR_SUL_UL_n24, Ligado Networks, Huawei, Hisilicon