3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100357

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.175 | CR | 0008 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.175 on Exclusion Bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Definition of Exclusion Band sizes is required to guarantee IAB nodes EMC testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing square brackets in order to define the exclusion band sizes applicable for IAB EMC testing. It is also added the concept of exclusion zone (spatial exclusion) for the definition of receiver exclusion bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ther won’t be clarity about how to conduct Radiated Immunity testing of IAB nodes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Start of new section ------------------------------

## 4.4	Exclusion bands

### 4.4.1	Transmitter exclusion band

The transmitter exclusion band for IAB is the frequency range over which no tests of radiated immunity of a transmitter are made. As the IAB node may operate its access and backhaul link in different NR IAB operating band, the transmitter exclusion band for IAB applies separately for the access and backhaul link. The transmitter exclusion band applies to IAB type 1-O.

The transmitter exclusion band is defined as:

FDL,low – ΔfOBUE <f < FDL,high + ΔfOBUE

Where:

-	Values of FDL,low and FDL,high are defined for each NR IAB operating band in TS 38.174 [2], clause 5.2.

-	The value of ΔfOBUE is derived considering the width of the NR IAB operating band, and is defined as in table 4.4.1-1.

Table 4.4.1-1: ΔfOBUE offset values for NR IAB

| IAB type | NR IAB operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| IAB type 1-O | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz FDL,high – FDL,low 900 MHz | 40 |

NOTE:	As the radiated immunity testing is defined in the frequency range 80 MHz to 6 GHz, there is no transmitter exclusion band defined for IAB type 2-O.

### 4.4.2	Receiver exclusion band

The receiver exclusion band for IAB is the frequency range over which no tests of radiated immunity of a receiver are made. As the IAB node may operate its access and backhaul link in different NR IAB operating band, the receiver exclusion band for IAB applies separately for the access and backhaul link. The receiver exclusion band applies to IAB type 1-O.

The receiver exclusion band is defined as:

FUL,low – ΔfRX <f < FUL,high + ΔfRX

Where:

-	Values of FUL,low and FUL,high are defined for each NR IAB operating band in in TS 38.174 [2], clause 5.2.

-	The value of ΔfRX is derived considering the width of the NR IAB operating band, and is defined as in table 4.4.2-1. Value of the ΔfRX also depends on the RI test setup, i.e. whether or not the spatial exclusion zone (as depicted in figure 9.2.2-1) is considered during the RI test.

Table 4.4.1-1: ΔfRX offset values for IAB

| IAB type | IAB operating band characteristics | RI test setup | ΔfRX (MHz) |
| --- | --- | --- | --- |
| IAB type 1-O | FUL,high – FUL,low < 100 MHz | With exclusion zone | 20 |
|  |  | Without exclusion zone | 60 |
|  | 100 MHz FUL,high – FUL,low 900 MHz | With exclusion zone | 60 |
|  |  | Without exclusion zone | 200 |

NOTE:	As the radiated immunity testing is defined in the frequency range 80 MHz to 6 GHz, there is no receiver exclusion band defined for IAB type 2-O.

----------------------------- End of proposed text ------------------------------