3GPP TSG-WG4 Meeting #98-e	R4-2100521

# Electronic meeting, 25th January - 5th February 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.807 | CR | <draft> | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Regulatory update for the 60GHz frequency range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_beyond_52GHz |  |  |  |  |  | Date: |  |  | 14-01-2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP TR 38.807 was instantiated under the RAN SI to collect and capture the corresponding regulatory related information for the frequency range from 52GHz to 116GHz. The corresponding RAN SI was formally completed in December 2019, but after that more related regulatory related decisions were made for the frequency range of 52-71GHz. To facilitate further normative work in RAN WGs, the corresponding updates are introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General corrections in section 4.1 regarding outcome of WRC19. General corrections in section 4.2.1 on the global overview of licensing modes in 52-71GHz frequency range. Updated technical parameters of 52-71GHz in ITU region 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 3GPP TR 38.807 will contain old and outdated regulatory information for the 52-71GHz frequency range. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2.1, 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 4	Operational Requirements

## 4.1	Overview of Global Spectrum Availability

In 2015, the international telecommunication union (ITU) proposed 11 millimetre-wave bands between 24 and 86 GHz to be studied towards WRC-19 for possible identification for IMT (see Table 4.1-1). The WRC-19 [5] concluded that the following frequency bands should have an IMT identification to enable 5G deployments: 24.25– 27.5 GHz, 37–43.5 GHz, 45.5–47 GHz, 47.2–48.2 GHz, 66–71 GHz.

Table 4.1-1 Bands studied under WRC-19 AI 1.13

| Candidate frequency bands which have allocations to the mobile service on a primary basis | Candidate frequency bands which required a co-primary mobile allocation |
| --- | --- |
| 24.25-27.5 GHz [Note1] | 31.8-33.4 GHz, |
| 37-40.5 GHz | 40.5-42.5 GHz |
| 42.5-43.5 GHz | 47-47.2 GHz |
| 45.5-47 GHz |  |
| 47.2-50.2 GHz |  |
| 50.4-52.6 GHz |  |
| 66-76 GHz |  |
| 81-86 GHz |  |
| NOTE 1: When conducting studies in the band 24.5-27.5 GHz, to take into account the need to ensure the protection of existing earth stations and the deployment of future receiving earth stations under the EESS (space-to-Earth) and SRS (space-to-Earth) allocation in the frequency band 25.5-27 GHz |  |

Table 4.1-2 provides information on the allocation within the frequency range 52.6 GHz to 116 GHz in ITU Radio Regulation [37]. The column with comments contains (a subset of) information on protection requirements for incumbent services. For the full details please refer to the Radio Regulations

Table 4.1-2: Frequency bands in the range 52.6 to 116 GHz in radio regulation

| Frequency band (GHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 52.6-54.25 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 54.25-55.78 | No | No | EESS (passive) and SRS (passive) |
| 55.78-59 | Yes | Yes | EESS (passive) and SRS (passive) This band available for high-density applications in the fixed service, footnote 5.547 |
| 59-59.3 | Yes | Yes | EESS (passive) and SRS (passive) Radiolocation |
| 59.3-64 | Yes | Yes | Radiolocation |
| 64-65 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 65-66 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 66-71 | Yes | No | The frequency band 66-71 GHz is identified for use by administrations wishing to implement the terrestrial component of International Mobile Telecommunications (IMT). This identification does not preclude the use of this frequency band by any application of the services to which this frequency band is allocated and does not establish priority in the Radio Regulations. |
| 71-76 | Yes | Yes | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in clause 2.2. |
| 76-81 | No | No | Radiolocation |
| 81-86 | Yes | Yes | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in clause 2.2. |
| 86-92 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 92-94 | Yes | Yes | Radiolocation |
| 94-94.1 | No | No | Radiolocation |
| 94.1-95 | Yes | Yes | Radiolocation |
| 95-100 | Yes | Yes | Radiolocation |
| 100-102 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 102-105 | Yes | Yes | N/A |
| 105-109.5 | Yes | Yes | SRS (passive) |
| 109.5-111.8 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 111.8-114.25 | Yes | Yes | SRS (passive) |
| 114.25-116 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |

Within the range 52.6 to 116 GHz, the frequency bands 66-76 GHz (including 66-71 and 71-76 GHz) and 81-86 GHz were studied under WRC-19 Agenda Item 1.13 for potential IMT identification. The frequency band 66-71 GHz is identified for use by administrations wishing to implement the terrestrial component of International Mobile Telecommunications (IMT). This identification does not preclude the use of this frequency band by any application of the services to which this frequency band is allocated and does not establish priority in the Radio Regulations. Resolution 241 (WRC-19) applies.

For 71-76 GHz, studies were carried out for the FS, RLS and FSS (space-to-Earth) indicating that sharing with FS and FSS is feasible. However, additional limits of the IMT BS and UE unwanted emissions is needed to protect RLS in the adjacent frequency band 76-81 GHz.

For 81-86 GHz, studies were carried out for the FS, FSS (Earth-to-space), RAS (in band and adjacent band), EESS (passive) and RLS. Studies are not needed for the SRS (passive), as this service is dealing with sensors around other planets and no interference issue is expected. Studies were also not carried out for the MSS. The results of those studies indicate that sharing with FS, FSS and RAS (in band and adjacent band) is feasible. Notice that additional limits of the IMT BS and UE unwanted emissions would be needed to ensure protection of EESS (passive) in the adjacent frequency band 76-81 GHz and RLS in the adjacent frequency band 86-82 GHz. Table 4.1-3 and Table 4.1-4 depicts the limits of the IMT BS and UE maximum unwanted emissions levels according to the individual compatibility studies.

Table 4.1-3: Limits of unwanted emission levels from IMT BS and UE operating on 71-76 and 81-86 GHz frequency bands into the 76-81 GHz frequency band according to WRC-19 compatibility studies

| Limits of unwanted emissions into 76-81 GHz from IMT stations | 76-77 GHz dB(W/200 MHz) | 77-81 GHz dB(W/200 MHz) |
| --- | --- | --- |
| BS | [TBD/−29.6/−31.5/] | [TBD/−33] |
| UE | [TBD/−20] | [TBD/−35] |

Table 4.1-4: Limits of unwanted emission levels from IMT BS and UE operating on 81-86 GHz into 86-92 GHz frequency band according to WRC-19 compatibility studies

| Limits of unwanted emissions into 86-92 GHz from IMT stations | dB(W/100 MHz) |
| --- | --- |
| BS | [TBD/-43.6/-49.9/-31.3/-20] |
| UE | [TBD/-43.5/-49.8/-31.2/-19.9] |

## 4.2	Country / Regional Specific Spectrum Availability and Spectrum Regulatory Requirements

### 4.2.1	Country / Regional Specific Licensing situation

Table 4.2-1 and 4.2-2 provide an overall summary of the current licensing situation for data communication (Mobile and Fixed, excluding radar-based telemetry) between frequencies 52.6GHz and 100GHz for various countries under ITU region 1, 2, and 3. Symbol 'U', and 'L', represent unlicensed spectrum and licensed spectrum (including light licensing and other licensing forms), respectively. Notice that Table 4.2-1 and 4.2-2 does not include information about the service allocated to each frequency band. It neither includes information about the frequency bands currently studied in AI 1.13 WRC-19 which are listed in Table 4.1-1. Also, it is worthwhile noticing that type of licensing for each frequency band is a national decision.

Table 4.2.1-1: Current Licensing situation for various countries between 52.6GHz and 71GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 |  | 54.25-55.78 |  | 55.78-56.9 |  | 56.9-57 | 57-58.2 |  | 58.2-59 | 59-59.3 | 59.3-64 | 64-65 |  | 65-66 |  |  | 66-71 |
| ITU Region 1 | Europe/CEPT |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |
| ITU Region 2 | USA |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |
|  | Canada |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |
|  | Brazil |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |
|  | Mexico |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  |  | L |  |  |  |  |  |  |  |  |
|  | Japan |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |
|  | Korea |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |
|  | Singapore |  |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |  |
|  | Australia |  |  |  |  |  |  |  | U (Mobile) |  |  |  |  |  |  |  |  |

Table 4.2.1-2: Current Licensing situation for various countries between 71GHz and 100GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 71-741) | 74-761) | 76-77 | 77-81 | 81-841) | 84-861) | 86-92 | 92-94 | 94-94.1 | 94.1-95 | 95-100 |
| ITU Region 1 | Europe/CEPT | L (Fixed) |  |  |  | L (Fixed) |  |  | L (Fixed) | L (Fixed) |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
| ITU Region 2 | USA | L (Fixed /Mobile) |  |  |  | L (Fixed /Mobile) |  |  | L (Fixed /Mobile) |  | L (Fixed /Mobile) |  |
|  | Canada | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Brazil | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Mexico | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  |  |  |  |  |  |
|  | Japan | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Korea | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan | L(Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Singapore | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Australia |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Candidate frequency band for IMT identification under WRC-19 AI 1.13 |  |  |  |  |  |  |  |  |  |  |  |  |

### 4.2.2	ITU Region 1

#### 4.2.2.1	Europe and CEPT

General information for usages in CEPT can be found in [41]. When it comes to Regulation about Short Range Devices additional information can be found in [42].  Unwanted emission limits can be found in [43] and [44]. The 57-71 GHz range, encompassing the 66-71 GHz range, which has been identified by the WRC-19 for IMT, is already subject to a permanent European Commission mandate to CEPT covering Short-Range Devices (SRD). ERC/REC 70-03 sets out the CEPT position on common spectrum which can be designated for Short Range Devices (SRD) applications and is a reference document to assist in preparing national spectrum regulations. The position set out in this Recommendation (ERC Recommendation 70-03) is subject to continuous review.  ERC Recommendation 70-03 and EC Decision for SRDs (EU) 2019/1345 already have provisions to allow higher powered Wideband Data Transmission use in the 57-71 GHz band. The spectrum regulatory deliverables were both revised and updated during the study period for WRC-19 to include both increased power and spectrum availability to allow use of the band by a more diverse set of uses and applications.

The results of the studies carried out within CEPT between wideband data transmission use and the incumbent services and applications in 57-66GHz were analysed to include the possibility of expanding the existing regulations to cover 66-71 GHz. As a result of this additional analysis both CEPT and EC developed the revised regulations and technology neutral technical conditions contained in both SRDs (EU) 2019/1345 and ERC Recommendation 70-03 that were approved last year. The scope of this work was not to look at introducing licenced mobile services into the band as the assumption was that 5G services in 66-71 GHz would have to share and meet the same technical requirements as the existing services operating in the band under a general authorisation regime.

ECC WGFM have been tasked by the ECC to develop least restrictive harmonised technical conditions suitable for next-generation (5G) terrestrial wireless systems for priority frequency bands above 24 GHz. This task is to review and revise, only if necessary, the EU-harmonised technical conditions for use of the 66-71 GHz frequency band with a view to use of this band for next-generation (5G) wireless broadband electronic communications services, taking into account relevant scenarios for shared spectrum use and the further development of all relevant services within the band. In this regard, ensure coherence of results and reporting to the European Commission, taking into account the ongoing work on the permanent mandate to CEPT on short-range devices, in accordance with the latest Commission guidance.

ECC WGFM at their October 2020 meeting concluded the following:

Based on our analysis the existing technical conditions contained in both SRDs (EU) 2019/1345 and ERC Recommendation 70-03 are sufficient to allow the use of the 66-71GHz frequency band by next generation (5G) wireless broadband electronic communication services. We also consider that the current regulations have been developed taking account of the CEPT Roadmap for 5G and the assumptions are that 5G services in this band would operate under the current general authorisation framework and would implement the appropriate technical conditions to co-exist with other users of the band including current Fixed and mobile Wideband technologies operating under the current regulations.

Table 4.2.2.1-1 and Table 4.2.2.1-2 provide summary of frequency allocations in the range of 52.6 and 114.25 GHz and regulatory requirements for existing Mobile and Fixed services in the frequency bands between 52.6 GHz and 114.25 GHz, respectively.

Table 4.2.2.1-1: Frequency bands in the range 52.6 to 100 GHz

| Frequency range in GHz | Comments | Region |
| --- | --- | --- |
| 52.6 to 54.25 | EESS service, all emissions are prohibited (footnote 5.340 of [37]) | ITU/CEPT |
| 55.78 to 57 | Recommended to be used by fixed services [40](footnote 5.547 of [37]) | ITU/CEPT |
| 57 to 66 | Several usage of Short range devices [42][45] 57-64 GHz recommended by CEPT for fixed service limited to point to point [47] 64-66 GHz recommended by CEPT for fixed service limited to point to point [46] 63-64 GHz ITS band in Europe [47]) | EC/CEPT |
| 66 to 71 | (EU) 2019/1345 and ERC Recommendation 70-03 | EC/CEPT |
| 71 to 76 and 81 to 86 | Recommended by CEPT for fixed services [48] | CEPT |
| 77 to 81 | Automotive RADAR in Europe [49][50] | EC/CEPT |
| 86 to 92, 100 to 102, 109.5 to 111.8 | EESS service, all emissions are prohibited (footnote 5.340 of [37]) | ITU/CEPT |
| 92 to 94, 94.1 to 100, 102 to 109.5 and 111.8 to 114.25 | Recommended by CEPT for fixed services [51]) | CEPT |

Table 4.2.2.1-2: Europe/CEPT – Current Regulatory Requirements for spectrum between 52.6 GHz and 114.25 GHz

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 57 – 71 | C1: 40 dBm mean EIRP, 23 dBm/MHz mean EIRP density. C2: 40 dBm mean EIRP, 23 dBm/MHz mean EIRP density, and maximum transmit power 27dBm and the antenna port or ports. C3: 55 dBm mean EIRP, 38 dBm/MHz mean EIRP density, and maximum transmit power 27dBm and transmit antenna gain >= 30 dBi. | Adequate spectrum sharing mechanism (e.g. Listen-before-Talk, Detect-And-Avoid) shall be implemented by the equipment | Not specified | Short Rage Devices - Wide Data Transmissions [42] | C1: Fixed outdoor installations are not allowed. C3: Applies only to fixed outdooe installations. |
| 71-76, 81-86 | Maximum power limit= 30dBm Maximum EIRP= 85dBm (55 dBW) Minimum antenna gain= Pout (dBm) + 15; or 38 (whichever is the greater) maximum antenna gain= 85-Pout(dBm) Complete set of specification (part 2-2 and part 3 of [52]) | Access to the frequency bands by Authorization regimes or by Block assignment/auction regimes. | An aggregation of 250 MHz channels for wider channels as defined in [53] (channel arrangements for channel sizes ranging from 250 to 4750 MHz) | Fixed Services defined in [53] To assist the planning of fixed links, self-coordination approach, similar to the "light licensing", described in [54] can be considered. |  |
| 92-94, 94.1-100, 102-109.5, 111.8-114.25 | Not defined [51] | Not defined [51] | Radio frequency channel/block arrangements should be derived by aggregation of 250 MHz channels. Inside a specific sub-band no upper limit is given to block size, provided that the blocks´ duplex separation is maintained. Details in Annex 3 of [51] | Fixed Services defined in [51]. The choice of the appropriate assignment method and licensing regime for Fixed Service remains a decision for national administrations, based on several technical and not technical factors | To assist the planning of fixed links, self-coordination approach, similar to the “light licensing”, described in [54] can be considered. |