3GPP TSG RAN WG4 Meeting #98-e	R4-2100674

Electronic Meeting, 25 Jan. - 5 Feb., 2021

Agenda item:	9.7.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-21: EN-DC_1-8-11_n3-n28

Document for:	Approval

1	Introduction

EN-DC of 3B LTE and 2B NR of DC_1-8-11_n3-n28 was approved in RAN#88 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

8.X	DC_1-8-11_n3-n28

8.X.1	Operating bands for DC

Table 8.X.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

8.X.2	Channel bandwidths per operating band for DC

Table 8.X.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 90 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.X.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n3, DC_1-8-11_n28, DC_1-8_n3-n28, DC_1-11_n3-n28 and DC_8-11_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.X.4	∆TIB and ∆RIB values

For DC_1-8-11_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 8.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |

8.X.5	MSD

As mentioned in 8.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-200744  Revised WID on LTE x bands (xDL/1UL, x=1,2,3,4) and NR 2 bands (2DL/1UL) EN-DC in Rel-16, LG Electronics