3GPP TSG-RAN WG4 Meeting #98-e                                   R4-2100769

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda Item:	7.1.5.1

Source:	MediaTek Inc.

Title:	Discussion on terminology for NR-U RRM requirements

Document for:		Discussion

- Introduction

In this paper, we provide our view for the requirements with DRX/gap in use, and it will have impact on the clarification of the following NR-U requirements.

- RLM INS

- CBD

- intra-frequency measurements without/with gaps

- inter-frequency measurements

- L1-RSRP

- Clarification for the requirement with DRX in use

In the last meeting WF [2], the terminology updates for the case with DRX, MGRP was discussed.

Terminology updates for the case without DRX, MGRP, etc. for 36.133

- The terminology needs to be updated in the following sections of TS 36.133, to capture the agreements from R4-2012249:

- 4.2.2.5.7 Measurements of NR cells operating with CCA (CCA requirements for interRAT reselection)

- 5.3.4A	E-UTRAN - NR FR1 Handover when CCA is used in the target cell

- 6.3.2.4A	RRC connection release with redirection to NR with CCA

- 7.31A 	Addition and Release Delay of NR PSCell operating with CCA for E-UTRA - NR Dual Connectivity

- 8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used

- 8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency

- 8.17.4A	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation when CCA is used

FFS: The need for further terminology updates for the case with DRX, MGRP, etc.

In IDLE mode, the terminology for the DRX cycles with LBT failures has been clarified as “ DRX cycles with at least one SMTC occasion where there are no SSBs available at the UE”. The corresponding text the cell reselection requirement are attached below for reference [1]:

- Ms is the number of DRX cycles with at least one SMTC occasion where there are no SSBs available at the UE during Nserv_CCA, and Ms< Ms,max

- Md, Mm, Me are the number of DRX cycles with at least one SMTC occasion where there are no SSBs available during the Tdetect,NR_Intra_CCA, Tmeasure,NR_Intra_CCA and Tevaluate,NR_Intra_CCA, and Mm ≤ Mm,max, Md ≤ Md,max and Me ≤  Me,max

Because when one DRX cycle contains multiple SMTC occasions, as illustrated in Figure 1, it is completely up to UE to choose which SMTC occasion for measurement. The only way to guarantee UE to receive a valid SSB during a DRX cycle is to transmit all SMTC occasions within that DRX cycle. One the other hand, even if there is just one single SMTC occasion no transmitted in a DRX cycle, RAN4 should treat this whole DRX cycle as LBT failure.

Figure 1: one DRX cycle contains multiple SMTC occasions

However, this clarification should be also applied to other RRM requirements with DRX in connected mode, including the following NR-U requirements:

- RLM (8.1A.2)

- CBD (8.5A.3)

- intra-frequency measurements without/with gaps (9.2A.5, 9.2A.6)

- inter-frequency measurements (9.3A.4, 9.3A.5)

- L1-RSRP (9.5.4A)

Nevertheless, the gap should be also considered in connected mode.

Proposal 1: further terminology updates for the case with DRX, MGRP are needed.

For measurements without gap required, it can follow the same approach as IDLE mode requirements.

Proposal 2: For measurements without gap required, when DRX is used and the DRX cycle length is longer than the SMTC periodicity, L is the number of DRX cycles with at least one SMTC where there are no X available outside gap at the UE during … period.

- X shall be replaced depending on the requirement with:

  - RLM-RS SSB in RLM requirements,

  - CBD-RS SSB in CBD requirements,

  - SSB in L1-RSRP measurement requirements,

  - SMTC in measurement without gap requirements, other than RSSI requirements and L1-RSRP

Proposal 3: For measurements with gap required, when DRX is used and the DRX cycle length is longer than the SMTC periodicity, L is the number of DRX cycles with at least one SMTC within Gap where there are no X available at the UE during … period.

- X shall be replaced depending on the requirement with:

  - SMTC in measurement with gap requirements, other than RSSI requirements and L1-RSRP

- Summary

In this paper, the clarification for DRX with unavailable SSBs in NR-U requirements have been discussed.

Proposal 1: further terminology updates for the case with DRX, MGRP are needed.

Proposal 2: For measurements without gap required, when DRX is used and the DRX cycle length is longer than the SMTC periodicity, L is the number of DRX cycles with at least one SMTC where there are no X available outside gap at the UE during … period.

- X shall be replaced depending on the requirement with:

  - RLM-RS SSB in RLM requirements,

  - CBD-RS SSB in CBD requirements,

  - SSB in L1-RSRP measurement requirements,

  - SMTC in measurement without gap requirements, other than RSSI requirements and L1-RSRP

Proposal 3: For measurements with gap required, when DRX is used and the DRX cycle length is longer than the SMTC periodicity, L is the number of DRX cycles with at least one SMTC within Gap where there are no X available at the UE during … period.

- X shall be replaced depending on the requirement with:

  - SMTC in measurement with gap requirements, other than RSSI requirements and L1-RSRP

References

[1] 38.133, V16.6.0.

[2] R4-2017080, WF on NR-U RRM Core Requirements, Ericsson.