3GPP TSG-RAN WG4 Meeting # 98-e	R4-2100833

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source:	ZTE Corporation

Title:	Scope of test cases for NR-U

Agenda item:	7.1.6.3.1

Document for:	Discussion

# 1	Introduction

In RAN4 97-e, companies identified that the latest RAN2 spec TS 38.321 has been modified to reflect changes by LBT results upon 2-step and 4-step based RA process, which is shown below.

5.1.3a MSGA transmission The MAC entity shall, for each MSGA: 1> if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and 1> if the notification of suspending power ramping counter has not been received from lower layers; and 1> if LBT failure indication was not received from lower layers for the last MSGA Random Access Preamble transmission; and 1> if SSB selected is not changed from the selection in the last Random Access Preamble transmission: 2> increment PREAMBLE_POWER_RAMPING_COUNTER by 1. 1> select the value of DELTA_PREAMBLE according to clause 7.3; 1> set PREAMBLE_RECEIVED_TARGET_POWER to msgA-PreambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP; 1> if this is the first MSGA transmission within this Random Access procedure: 2> if the transmission is not being made for the CCCH logical channel: 3> indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission. 2> if the Random Access procedure was initiated for SpCell beam failure recovery: 3> indicate to the Multiplexing and assembly entity to include a BFR MAC CE or a Truncated BFR MAC CE in the subsequent uplink transmission. 2> obtain the MAC PDU to transmit from the Multiplexing and assembly entity according to the HARQ information determined for the MSGA payload (see clause 5.1.2a) and store it in the MSGA buffer. 1> compute the MSGB-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted; 1> instruct the physical layer to transmit the MSGA using the selected PRACH occasion and the associated PUSCH resource of MSGA (if the selected preamble and PRACH occasion is mapped to a valid PUSCH occasion), using the corresponding RA-RNTI, MSGB-RNTI, PREAMBLE_INDEX, PREAMBLE_RECEIVED_TARGET_POWER, msgA-PreambleReceivedTargetPower, and the amount of power ramping applied to the latest MSGA preamble transmission (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); 1> if LBT failure indication is received from lower layers for the transmission of this MSGA Random Access Preamble: 2> instruct the physical layer to cancel the transmission of the MSGA payload on the associated PUSCH resource; 2> if lbt-FailureRecoveryConfig is configured: 3> perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a). 2> else: 3> increment PREAMBLE_TRANSMISSION_COUNTER by 1; 3> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 4> indicate a Random Access problem to upper layers; 4> if this Random Access procedure was triggered for SI request: 5> consider this Random Access procedure unsuccessfully completed. 3> if the Random Access procedure is not completed: 4> if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1: 5> set the RA_TYPE to 4-stepRA; 5> perform initialization of variables specific to Random Access type as specified in clause 5.1.1a; 5> if the Msg3 buffer is empty: 6> obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer; 5> flush HARQ buffer used for the transmission of MAC PDU in the MSGA buffer; 5> discard explicitly signalled contention-free 2-step RA type Random Access Resources, if any; 5> perform the Random Access Resource selection procedure as specified in clause 5.1.2. 4> else: 5> perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a). NOTE: The MSGA transmission includes the transmission of the PRACH Preamble as well as the contents of the MSGA buffer in the PUSCH resource corresponding to the selected PRACH occasion and PREAMBLE_INDEX (see TS 38.213 [6])

The above content in TS 38.321 indicates clearly that 2-step RACH under NR-U is supported in Release 16. However, currently there are no core requirements regarding 2-step RACH for NR-U and no test cases planned for this feature. Companies’ views were summarized in WF [2] and [3], the content copied below:

Issue 3-3-5: Random access RAN4 to discuss the random access tests after the random access core requirements are discussed.

- This paper provides our view on this regard.

2	Discussion

Since the corresponding procedures for 2-step RACH under NR-U have already been captured in RAN2 specifications, it’s clear that such feature is supported in Release 16.

Observation 1: Procedures for 2-step RACH under NR-U have already been captured in RAN2 specifications.

Since the feature is supported, corresponding requirements need to be defined in RAN4. There is ongoing discussion in the core requirements part which will likely to result in a consensus during RAN4 98-e. If RAN4 agrees to specify core requirements for NR-U and 2-step RA, which is already supported by RAN2, corresponding test cases shall be defined.

RAN4 shall define in Rel-16 test cases for 2-step RA in NR-U as it is already a supported feature in R16.

3	Conclusion

Observation 1: Procedures for 2-step RACH under NR-U have already been captured in RAN2 specifications.

Proposal 1: RAN4 shall define in Rel-16 test cases for 2-step RA in NR-U as it is already a supported feature in R16.

References

- TS 38.321

- R4-2017080

- R4-2017089