3GPP TSG-RAN WG4 Meeting # 98-e	R4-2100837

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source:	ZTE Corporation

Title:	Test cases for intra and inter frequency and inter-RAT measurement accuracy

Agenda item:	7.1.6.3.13

Document for:	Discussion

# 1	Introduction

In RAN4 97-e, the first meeting to discuss performance part of NR-U, companies has some preliminary discussions on the scope of test cases, test configurations and the work plan. During the discussions, three of the open issue are the scope of test cases regarding intra-f, inter-f and inter-RAT measurement accuracy requirements. Companies had different views on the scope test cases while some consensus was captured in the WF [1] as shown below.

Issue 3-3-18: Measurement accuracy tests RAN4 to define at least the following test cases: Intra-frequency absolute and relative accuracies for SS-RSRP on: • NR-U SCC, with NR PCC (FR1) • NR-U PCC • NR-U SCC, with NR-U PCC • NR-U PSCC, with E-UTRAN PCC (FDD,TDD) • NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) FFS: SS-SINR and SS-RSRQ Absolute and relative accuracies for L1-RSRP on: • NR-U SCC, with NR PCC (FR1) • NR-U PCC • NR-U SCC, with NR-U PCC • NR-U PSCC, with E-UTRAN PCC (FDD,TDD) • NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) Accuracy for NR-U Intra-frequency RSSI and CO measurements on: • NR-U SCC, with NR PCC (FR1) • NR-U PCC • NR-U SCC, with NR-U PCC • NR-U PSCC, with E-UTRAN PCC (FDD,TDD) • NR-U SCC, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) Inter-frequency absolute and relative accuracies for SS-RSRP on: • NR-U neighbor, with NR PCC (FR1) • NR-U neighbor, with NR-U PCC • NR-U neighbor, with NR-U PSCC, with E-UTRAN PCC (FDD,TDD) FFS: SS-RSRQ and SS-SINR Accuracy for NR-U Inter-frequency RSSI measurements and CO on: • NR-U neighbor, with NR PCC (FR1) • NR-U neighbor, with NR-U PCC • NR-U neighbor, with NR-U PSCC, with E-UTRAN PCC (FDD,TDD) Accuracy for NR-U Inter-RAT SFTD measurements between: • E-UTRAN PCell (FDD,TDD) and NR-U neighbor FFS: NOTE: under the condition of stationary paths Accuracy for NR-U for E-UTRA-NR-U SS-RSRP measurements • On NR-U neighbor, with E-UTRA (FDD,TDD) PCC • On NR-U neighbor, with E-UTRA (FDD,TDD) PCC and NR-U PSCC FFS: SS-SINR and SS-RSRQ

- This paper provides our view on this topic.

2	Discussion

For intra, inter-frequency and inter-RAT measurement accuracy requirements, some companies suggested not to define test cases for SS-SINR and SS-RSRQ but only specify test cases for SS-RSRP. The proposal was backed by the argument that SS-RSRP already reflects the UE capability and essential functions and if the UE can pass the tests for SS-RSRP then the UE is assumed to be able to meet requirements for SS-SINR and SS-RSRQ as well, and thus, no need to specify test cases for SS-SINR and SS-RSRQ.

We don’t agree with the above argument and think that test cases for SS-SINR and SS-RSRQ shall be specified. First of all, as a principle when defining core requirements / performance requirements / test cases, NR-U related requirements or test cases always take that of R15 UEs as baseline. This is because the goal of NR-U WI is to allow the network to work on unlicensed frequency bands while still provide services at a same standard with NR (whereas possible, for instance when no LBT failure occurs).

When defining test cases for features in NR-U, R15 UE test cases shall be taken as baseline.

If the above proposal is agreeable then the test cases for SS-SINR and SS-RSRQ shall also be specified. This is simply due to the fact in test cases for legacy UEs, test cases for SS-SINR and SS-RSRQ are specified along with SS-RSRP. All of the three sets of test cases are essential.

Another angle to look at this issue would be if the UE can be assumed to meet requirements for SS-SINR and SS-RSRQ conditioned that the UE passes tests for SS-RSRP, then it doesn’t mean much burden for the UE to also be tested for SS-SINR and SS-RSRQ since no extra implementation is required.

RAN4 shall define test cases for SS-SINR and SS-RSRQ for intra-frequency, inter-frequency and inter-RAT measurement accuracy.

3	Conclusion

Proposal 1: When defining test cases for features in NR-U, R15 UE test cases shall be taken as baseline.

Proposal 2: RAN4 shall define test cases for SS-SINR and SS-RSRQ for intra-frequency, inter-frequency and inter-RAT measurement accuracy.

References

- R4-2017089