3GPP TSG-RAN WG4 meeting #98-e	R4-2100901

Electronic Meeting, Jan.25-Feb.5, 2021

Agenda Item:	7.9.4.2.3

Source:	Samsung

Title:	Simulation results for URLLC schemes

Document for:	Discussion

1	Introduction

In RAN4 #97e meeting, test cases for URLLC transmission schemes for multi-TRP were agreed in [1], where only FDM scheme A and inter-slot TDM scheme are defined for URLLC performance requirement.

In this contribution, both alignment and impairment results are provided.

2	Simulation results

The simulation results of PDSCH with single-DCI based FDM scheme A and inter-slot TDM are shown in this section.

2.1 Single-DCI based FDM scheme A

Fig. 2.1 SNR vs. Throughput for Single-DCI based FDM scheme A in FDD mode

Fig. 2.2 SNR vs. Throughput for Single-DCI based FDM scheme A in TDD mode

The SNR point on condition that 70% of max throughput are collected in table 2.1.

Table 2.1 SNR point for Single-DCI based FDM schemed A (Alignment results)

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric | SNR point |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-1 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput | 8.5 |
| 5-2 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput | 4.9 |
| 6-1 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput | 8.4 |
| 6-2 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput | 5.1 |

2.2 Single-DCI based inter-slot TDM

Fig.2.3 URLLC TDM inter-slot BLER for FDD mode

Fig.2.4 URLLC TDM inter-slot BLER for TDD mode

Table 2.2 SNR point for Single-DCI based inter-slot TDM (Alignment results)

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric | SNR point |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7-1 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 200Hz / 2us | 1% BLER | To be update |
| 7-2 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 200Hz / 2us | 1% BLER | To be update |
| 8-1 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 300Hz / 1us | 1% BLER | To be update |
| 8-2 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 300Hz / 1us | 1% BLER | To be update |

3	Summary

We provide alignment results in above section for URLLC scheme test cases. The impairment results for these test cases were provided in below table.

Table 3.1 Impairment results for required SNR [dB] at reference test points

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-1 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput | 10.5 |
| 5-2 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput | 6.9 |
| 6-1 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput | 10.4 |
| 6-2 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput | 7.1 |
| 7-1 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 200Hz / 2us | 1% BLER | To be update |
| 7-2 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 200Hz / 2us | 1% BLER | To be update |
| 8-1 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 300Hz / 1us | 1% BLER | To be update |
| 8-2 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 300Hz / 1us | 1% BLER | To be update |

References

- R4-2017529, “WF on NR eMIMO PDSCH requirement”, Intel Corporation, RAN4 #97-e Electronic meeting.

- R4-2017530, “Simulation assumption for PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes”, Ericsson, Intel, RAN4 #97-e Electronic meeting

- R4-2014741, “Views for Multi-Panel/TRP transmission schemes”, Samsung, RAN4 #97-e Electronic Meeting

- R4-2015628,” Summary of simulation results of NR UE demod with higher BLER (FR1)”, Huawei, HiSilicon, RAN4 #97-e Electronic Meeting

Annex: Test assumptions for Single-DCI URLLC scheme

For URLLC schemes, only FDM scheme A and inter-slot TDM scheme are considered, meanwhile the main test assumptions are presented as below:

- PRB precoding granularity

  - Wideband precoding for single-DCI based FDM scheme A

  - 2 PRB precoding granularity for single-DCI based inter-slot TDM scheme

- Antenna port indices and MIMO Rank

  - FDM Scheme A

    - Rank 2: {1000, 1002} DMRS antenna port with TCI state#1 and {1001, 1003} DMRS antenna port with TCI state#2

  - Inter-slot TDM

    - Rank 1: 1000 DMRS antenna port with TCI state#1 and 1001 DMRS antenna port with TCI state#2

    - Two Tx occasions for inter-slot TDM

- Test Metric

  - 70% TP for FDM scheme A

  - 1% BLER for inter-slot TDM scheme (BLER is calculated after all transmission)

In addition, the other parameters are detailed as following.

Table Annex.1 Single-DCI based FDM Scheme A

| Parameters | Simulation cases 5-1 and 5-2 | Simulation cases 6-1 and 6-2 |
| --- | --- | --- |
| Duplex mode | FDD | TDD |
| TDD UL/DL configuration | N/A | FR1.30-1 (7DS2U, 6D+4G+4U) PDSCH is scheduled in the special slots |
| Timing offset of the second TRP from the first TRP (Note 1) | Negative offset: -0.5 μs | Negative offset: -0.25μs |
| Frequency offset between two TRPs (Note 1) | 200 Hz | 300 Hz |
|  | Common serving cell parameters |  |
| Physical Cell ID | 0 |  |
| SSB position in burst | First SSB in Slot #0 |  |
| SSB periodicity | 20ms |  |
| Transmit TRP of SSB | Only from TRP1 |  |
|  | TRS#1 (configuration for TCI state #1) |  |
| Transmit TRP | TRP#1 |  |
| CSI-RS periodicity | 20 slots | 40 slots |
| CSI-RS offset | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=0 | k0=0 |
| QCL Info | TCI state #0 | TCI state #0 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  | TRS#2 (configuration for TCI state #2) |  |
| Transmit TRP | TRP#2 |  |
| CSI-RS periodicity | 20 slots | 40 slots |
| CSI-RS offset | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=1 | k0=1 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
| QCL Info | TCI state #0 | TCI state #0 |
|  | TCI State configuration #0 |  |
| Type 1 QCL information | SSB index #0, QCL type C |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #1 |  |
| Type 1 QCL information | CSI-RS resource: TRS#1, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #2 |  |
| Type 1 QCL information | CSI-RS resource: TRS#2, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | PDCCH configuration |  |
| CORESETPoolIndex | 0 (or not configured) |  |
| Symbols for PDCCH | 0, 1 |  |
| K0 | 0 |  |
| AL | 8 (576REs) |  |
|  | PDSCH configuration |  |
| PDSCH resource mapping type | Type A |  |
| Resource allocation type | Type 1 |  |
| DMRS configuration | Type 1, 1+1 |  |
| Antenna ports index ​ | {1000,1001} for TCI#1 and {1000,1001} for TCI#2 |  |
| Rank | Rank 2 |  |
| Start symbol (S) | 2 |  |
| Time duration (L) | 12 |  |
| PRB allocation | PDSCH#1: PRB#0 to RPB#25 PDSCH#2: PRB#26 to RPB#51 | PDSCH#1: PRB#0 to RPB#52 PDSCH#2: PRB#53 to RPB#105 |
| PRB precoding granularity | Wideband |  |
| Note 1: Using TRP1 is Reference to define timing and frequency offset Note 2: PDSCH is not scheduled in slots where SSB is transmitted (slot #0 with periodicity of 20ms) Note 3: PDSCH is scheduled in slots where TRSs are transmitted. |  |  |

Table Annex.2 Single-DCI based inter-slot TDM

| Parameters | Simulation cases 7-1 and 7-2 | Simulation cases 8-1 and 8-2 |
| --- | --- | --- |
| Duplex mode | FDD | TDD |
| TDD UL/DL configuration | N/A | FR1.30-1 (7DS2U, 6D+4G+4U) PDSCH is NOT scheduled in the special slots |
| Timing offset of the second TRP from the first TRP (Note 1) | Positive offset: 2 μs | Positive offset: 1 μs |
| Frequency offset between two TRPs (Note 1) | 200 Hz | 300 Hz |
|  | Common serving cell parameters |  |
| Physical Cell ID | 0 |  |
| SSB position in burst | First SSB in Slot #0 |  |
| SSB periodicity | 20ms |  |
| Transmit TRP of SSB | Only from TRP1 |  |
|  | TRS#1 (configuration for TCI state #1) |  |
| Transmit TRP | TRP#1 |  |
| CSI-RS periodicity | 20 slots | 40 slots |
| CSI-RS offset | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=0 | k0=0 |
| QCL Info | TCI state #0 | TCI state #0 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  | TRS#2 (configuration for TCI state #2) |  |
| Transmit TRP | TRP#2 |  |
| CSI-RS periodicity | 20 slots | 40 slots |
| CSI-RS offset | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=1 | k0=1 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
| QCL Info | TCI state #0 | TCI state #0 |
|  | TCI State configuration #0 |  |
| Type 1 QCL information | SSB index #0, QCL type C |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #1 |  |
| Type 1 QCL information | CSI-RS resource: TRS#1, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #2 |  |
| Type 1 QCL information | CSI-RS resource: TRS#2, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | PDCCH configuration |  |
| CORESETPoolIndex | 0 (or not configured) |  |
| Symbols for PDCCH | 0, 1 |  |
| K0 | 0 |  |
| AL | 8 (576REs) |  |
|  | PDSCH configuration |  |
| PDSCH Tx occasions | 2 |  |
| PDSCH resource mapping type | Type A |  |
| Resource allocation type | Type 1 |  |
| DMRS configuration | Type 1, 1+1 |  |
| Antenna ports index ​ | {1000} for TCI#1 and {1001} for TCI#2 |  |
| Rank | Rank 1 |  |
| Start symbol (S) | 2 |  |
| Time duration (L) | 12 |  |
| PRB allocation | PDSCH#1: PRB#0 to RPB#51 PDSCH#2: PRB#0 to RPB#51 | PDSCH#1: PRB#0 to RPB#105 PDSCH#2: PRB#0 to RPB#105 |
| PRB precoding granularity | 2 |  |
| Note 1: Using TRP1 is Reference to define timing and frequency offset Note 2: PDSCH is not scheduled in slots where SSB is transmitted (slot #0 with periodicity of 20ms) Note 3: PDSCH is scheduled in slots where TRSs are transmitted. |  |  |