3GPP TSG-RAN WG4 Meeting #98-e              	R4-2100922

Electronic Meeting, 25th Jan– 5th Feb, 2021

Agenda item:	7.15.2.2

Source:	Samsung

Title:	Updated simulation results for NR HST PUSCH

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, all the remaining issues for NR HST BS PUSCH demodulation requirement were finalized and the test parameters for related requirement were captured in the WF [1].

In this contribution, the updated simulation results are provided for performance requirement derived.

# 2	Simulation Results

2. 1 Requirements for 350km/h

In this subsection, the updated ideal and impairment results for both open and tunnel scenario under 350km/h are provided for alignment.

Table 1: Simulation results for NR HST PUSCH with CP-OFDM for open scenario with 350km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal ) | SNR(@70%) (impair ) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -5.87 | -3.87 |
| 15KHz, 5MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | 6.96 | 8.96 |
| 15KHz, 5MHz | 2 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -11.18 | -9.18 |
| 15KHz, 5MHz | 16 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | 1.14 | 3.14 |
| 30KHz,10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -5.92 | -3.92 |
| 30KHz,10MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | 6.62 | 8.62 |
| 30KHz,10MHz | 2 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -11.18 | -9.18 |
| 30KHz,10MHz | 16 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | 0.87 | 2.87 |

Table 2: Simulation results for NR HST PUSCH with CP-OFDM for tunnel scenario with 350km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impair) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -2.81 | -0.81 |
| 15KHz, 10MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -2.83 | -0.83 |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | -5.80 | -3.80 |
| 15KHz, 5MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1340Hz | 7.4 | 9.4 |
| 30KHz,10MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -2.88 | -0.88 |
| 30KHz, 40MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -2.82 | -0.82 |
| 30KHz, 10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | -5.87 | -3.87 |
| 30KHz, 10MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 2334Hz | 7.00 | 9.00 |

2. 2 Requirements for 500km/h

In this subsection, the updated ideal and impairment results for both open and tunnel scenario under 500km/h are provided for alignment.

Table 3: Simulation results for NR HST PUSCH with CP-OFDM for open scenario with 500km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -5.83 | -3.83 |
| 15KHz,5MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | 7.33 | 9.33 |
| 15KHz, 5MHz | 2 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -11.11 | -9.11 |
| 15KHz,5MHz | 16 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | 1.44 | 3.44 |
| 30KHz, 10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -5.87 | -3.87 |
| 30KHz, 10MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | 7.01 | 9.01 |
| 30KHz, 10MHz | 2 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -11.14 | -9.14 |
| 30KHz, 10MHz | 16 | 1T8R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | 1.18 | 3.18 |

Table 4: Simulation results for NR HST PUSCH with CP-OFDM for tunnel scenario with 500km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -2.68 | -0.68 |
| 15KHz,10MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -2.69 | -0.69 |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | -5.68 | -3.68 |
| 15KHz, 5MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 1740Hz | 8.53 | 10.53 |
| 30KHz,10MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -2.77 | -0.77 |
| 30KHz,40MHz | 2 | 1T1R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -2.67 | -0.67 |
| 30KHz, 10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | -5.76 | -3.76 |
| 30KHz,10MHz | 16 | 1T2R | 1+1+1(2 or 3,7,11) | AWGN 3334Hz | 7.48 | 9.48 |

2. 3 Requirements for fading channel with high Doppler

In the last meeting, RAN4 agreed to introduce the PUSCH requirement with CP-OFDM waveform for fading channel under high Doppler scenario. In this subsection, the ideal and impairment results are provided for alignment.

Table 5: Ideal and impairment results for NR HST PUSCH with CP-OFDM for fading channel

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | Frequency Offset (Hz) | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | TDLC300-600 | 0 | -3.37 | -1.37 |
| 30KHz, 10MHz | 2 | 1T2R | 1+1+1(2 or 3,7,11) | TDL300-1200 | 0 | -3.40 | -1.40 |

# 3	Conclusion

In this contribution, the updated simulation results for NR HST PUSCH requirement are provided.

# References

R4-2012676 WF on Rel-16 NR HST BS demodulation requirements, Nokia