3GPP TSG-RAN WG4 Meeting #98-e                                                            R4-2100982

Online, 25th Jan - 5th Feb, 2021

Source:	Samsung, TELUS, Bell mobility

Title:	TP for TR 38.717-03-02: CA_n13-n25-n66

Agenda item:			9.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n13A-n25A-n66A according to the request in [1].

2. Reference

- RP-202201, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL.

# 3. Text Proposal

<Start of Text Proposal>

5.1.x	CA_n13-n25-n66

5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n13-n25-n66 | n13 | 777 MHz | – | 787 MHz | 746 MHz | – | 757 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n13A-n25A-n66A | CA_n13A-n25A CA_n13A-n66A CA_n25A-n66A | n13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

5.1.x.3	Co-existence studies

Co-existence studies of CA_n13-n25-n66 with 2UL are already covered in the constituent fall-back modes, it can get that:

- IMD4 of band n13 UL and band n25 UL falling to band n66 DL

- IMD4 of band n13 UL and band n66 UL falling to band n25 DL

5.1.x.4	REFSENS requirements

Table 5.1.x.4-1 shows the required MSD levels for the CA configuration, its value can reuse the value of DC_2A-13A_n66A and DC_13A-66A_n2A in TS 38.101-3.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n13A-n25A-n66A | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | 1736 | 5 | 25 | 2156 | 7..2 | FDD | IMD4 |
|  | n25 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n13 | 780 | 10 | 50 | 749 | N/A | FDD | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 6.2 | FDD | IMD4 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |

<End of Text Proposal>