3GPP TSG-RAN WG4 Meeting #98-e	R4-2101002

Electronic meeting, 25th January – 5th Feburary, 2021

Source:	Ericsson

Title:	Simulation results for NR-U PUCCH demodulation requirements

Agenda item:	7.1.7.4.3

Document for:	others

# Introduction

In RAN4#97-e meeting, companies got agreements on PUCCH simulation assumptions [1]. This contribution delivers some simulation results based on our analysis and proposals [2].

# Simulation assumptions and results

### 2.1 PUCCH Format 0

Table 2.1.1 Simulation assumptions for PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of symbol | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | Option 1: TDLC300-100 low |
| Test metric | - SNR@Prob(ACK miss)≤10^(−2) - SNR@Prob(NACK→ACK)≤10^(−3) |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

Table 2.1.2 Simulation results for ACK miss detection and NACK to ACK miss detection for PF0

|  | SNR @ ACK missed detection probability = 10^(-2) | SNR @ NACK to ACK detection probability = 10^(-3) |
| --- | --- | --- |
| SCS = 15 kHz | -7.44 dB | -10.11 dB |
| SCS = 30 kHz | -7.24 dB | -9.78 dB |

### 2.2 PUCCH Format 1

Table 2.2.1 Simulation assumptions for PF1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | Option 1: TDLC300-100 low |
| Test metric | - SNR@Prob(ACK miss)≤10^(−2)) SNR@Prob(NACK→ACK)≤10^(−3)) Note2 |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. Note 2: The DTX to ACK probability shall not exceed 1% |  |

Table 2.2.2 Ideal simulation results for ACK missed detection and NACK to ACK detection for PF1

|  | SNR @ ACK missed detection probability = 10^(-2) | SNR @ NACK to ACK detection probability = 10^(-3) |
| --- | --- | --- |
| SCS = 15 kHz | -18 dB | -17.5 dB |
| SCS = 30 kHz | -18 dB | -17.5 dB |

# Conclusion

In this contribution, we deliver our simulation results based on the WF [1] the last meeting and our discussion paper [2].

# References

[1]	R4-2017467, “Way Forward on NR-U PUCCH demodulation requirements”, RAN4#97-e, Ericsson

[2]	R4-2101001, “Discussion on NR-U PUCCH demodulation requirements”, RAN4#98-e, Ericsson

[3]