3GPP TSG-RAN WG4 Meeting #98-e	R4-2101036

Electronic Meeting, January 25 – February 5, 2021

Source: 	NTT DOCOMO, INC.

Title:	TP for DC_3A-21A_n1A-n78A-n79A for TR37.717-11-31

Agenda item:	9.13.2

Document for:	Approval

- Introduction

This paper provides TP for DC_3A-21A_n1A-n78A-n79A for TR37.717-11-31. Co-existence studies and MSD analysis can be omitted because interference between LTE bands and NR FR1 bands have been already studied in lower order EN-DC combinations.

- Text proposal

< Start of Text Proposal>

7.x		DC_3A-21A_n1A-n78A-n79A

7.x.1	Operating bands for DC

Table 7.x.1-1: EN-DC band combination (five bands).

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_3-21_n1-n78-n79 | CA_3-21 | CA_n1-n78-n79 |

7.x.2	Inter-band DC Configurations

Table 7.x.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-21A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A DC_21A_n1A DC_21A_n78A DC_21A_n79A |

7.x.3	Co-existence studies

Co-existence study of this DL of LTE 2 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of DL of LTE 1 band + NR 3 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.x.4	∆TIB and ∆RIB values

For DC_3A-21A_n1A-n78A-n79A, the ∆TIB and ∆RIB values are given in the tables below.

Table 7.x.4-1: ΔTIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n1A-n78A-n79A | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |

Table 7.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-21A_n1A-n78A-n79A | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0 |

7.x.5	MSD

No additional MSD requirement is needed.

< End of Text Proposal>

- References