3GPP TSG-RAN WG4 Meeting #98-e                       	 R4-2101056

Electronic Meeting, 25 Jan. – 5 Feb., 2021

Agenda Item:	7.3.5.3.5

Source:	MediaTek Inc.

Title:	Remaining issues on NR V2X RRM test case

Document for:		Discussion

- Introduction

In last RAN4 #97e meeting, a WF[1] about NR V2X RRM requirement was approved, but one of the remaining issues wasn’t captured that is applicable scenario for congestion control test case. In this paper, we will discuss this issue.

- Applicable scenario for congestion control test case

In RAN1 feature list, the agreed items for congestion control is shown as follow. It could find that UE report CBR measurement to gNB is not required for the UE which supports PC5 only. The event C1 can only be trigged on Uu link. The UE which don’t have Uu link can’t report this event and can’t be tested based on current test design. Thus, we suggest to add this applicability rule for congestion control test case which only need be tested by the UE supports both Uu link and Sidelink.

| 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB when operating in Mode 1 and mode 2 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates | 15-1 and at least one of 15-2 and 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | FFS: This is the basic FG for NR sidelink Note: component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1 Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1: The congestion control test case is only applicable to UE which supports both SL and Uu Link.

- Summary

In this paper, we give our views on remaining issues for NR SL test case.

Proposal 1: The congestion control test case is only applicable to UE which supports both SL and Uu Link.

- Reference

- R4-2017100, WF on NR V2X RRM requirements, LG Electronics