3GPP TSG-RAN WG4 Meeting # 98-e                	R4-2101134

Electronic Meeting, 25 Jan – 05 Feb, 2021

Source:	Nokia, Nokia Shanghai Bell

Title: 	NR-U RRM test case list updates

Agenda item:	7.1.6.3.1

Document for:	Discussion

Introduction

In the last RAN4 meeting, the NR-U performance work started. An initial test case list was agreed in the WF[1]. In this document, the following test cases are discussed:

- RRC_IDLE, cell re-selection

- Random access

- Inter-frequency measurement procedure

- Accuracy for NR-U inter-frequency measurements

Test case list

RRC_IDLE, cell re-selection

In the last RAN4 meeting, the following test cases were agreed for RRC_IDLE, cell re-selection:

| Group of requirements | Test cases |  | Requirements section | Agreed | Volunteer | Phase I | Phase II |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | NR-U -> NR-U | intra-frequency | 4.2A | Yes | Ericsson |  |  |
|  |  | inter-frequency |  | Yes |  |  |  |
|  | NR(FR1) -> NR-U |  |  | FFS |  |  |  |
|  | NR-U -> NR(FR1) |  |  | FFS |  |  |  |
|  | NR-U - > E-UTRAN (FDD,TDD) |  |  | FFS |  |  |  |
|  | E-UTRAN (FDD,TDD) -> NR-U |  | TS 36.133 | Yes |  |  |  |

In RAN4 97, it was agreed that NR-U -> NR-U cell re-selection test cases will be defined.

| 4.2A.2.2 | Measurement and evaluation when subject to CCA on the serving cell |
| --- | --- |
| 4.2A.2.3 | Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell |
| 4.2A.2.4 | Measurements of inter-frequency NR cells when subject to CCA on the target cell |
| 4.2A.2.5 | Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell |

In clause 4.2A, the following requirements are defined for carrier frequencies with CCA in TS 38.133[2]:

Since new requirements are defined, we think that these requirements also need to be tested. Therefore:

RAN4 to define cell-reselection test cases for: NR(FR1) -> NR-U, NR-U -> NR(FR1) and NR-U - > E-UTRAN (FDD,TDD).

Random Access

The random access core requirements are still under discussion. In the last meeting, during the NR-U core maintenance discussion, it was agreed that:

4-step RA type The requirements are not the same as in Rel-15 The requirements will include requirements for contention-based and non-contention based RA 2-step RA type FFS: RAN4 will define in Rel-16 NR-U RA requirements for 2-step RA RAN4 will investigate and discuss in RAN4#98-e the impacts of 2-step RA on other sections/requirements

It is our view that requirements for at least 4-step RA type should be defined, and test cases as well.

RAN4 to define contention based and non-contention based test cases for at least 4 step RA type in NR-U.

Inter-frequency measurement procedure and accuracy

In the last RAN4 meeting, the inter-frequency measurement procedure test cases for SS-RSRP measurements were agreed.

| Inter-frequency measurement procedure (SS-RSRP, SS-RSRQ, SS-SINR, SFTD, RSSI, CO) | Inter-frequency SS-RSRP measurements on: |  | 9.3A.4, 9.3A.5 |  |
| --- | --- | --- | --- | --- |
|  | · NR-U inter-frequency, with NR PCC (FR1) |  |  | Yes |
|  | · NR-U inter-frequency, with NR-U PCC |  |  | Yes |
|  | · NR-U inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |
|  | · NR (FR1) inter-frequency, with NR-U PCC |  |  | Yes |
|  | · NR (FR1) inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | Yes |
|  | Inter-frequency SS-RSRQ, SS_SINR measurements on: |  |  | FFS |
|  | · NR-U inter-frequency, with NR PCC (FR1) |  |  | FFS |
|  | · NR-U inter-frequency, with NR-U PCC |  |  | FFS |
|  | · NR-U inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | FFS |
|  | · NR (FR1) inter-frequency, with NR-U PCC |  |  | FFS |
|  | · NR (FR1) inter-frequency, with NR-U PSCC and E-UTRAN PCC (FDD,TDD) |  |  | FFS |

The reason why the SS-RSRQ and SS-SINR test cases were not agreed is that some companies discussed that the behavior considering the LBT failures could already be captured in the SS-RSRP test cases. Regarding this discussion, it was agreed for intra-frequency measurements that all test cases should be defined (SS-RSRP, SS-RSRQ and SS-SINR), and it is our view that the same should apply for inter-frequency measurements.

RAN4 to define inter-frequency measurement procedure test cases also for SS-RSRQ and SS-SINR measurements.

Consequently, test cases should also be defined for inter-frequency measurement accuracies:

RAN4 to define inter-frequency measurement accuracy test cases also for SS-RSRQ and SS-SINR measurements.

- Conclusions

In this contribution we have further discussed NR-U test case list. We have made the following proposals:

- RAN4 to define cell-reselection test cases for: NR(FR1) -> NR-U, NR-U -> NR(FR1) and NR-U - > E-UTRAN (FDD,TDD).

RAN4 to define contention based and non-contention based test cases for at least 4 step RA type in NR-U.

RAN4 to define inter-frequency measurement procedure test cases also for SS-RSRQ and SS-SINR measurements.

RAN4 to define inter-frequency measurement accuracy test cases also for SS-RSRQ and SS-SINR measurements.

References

- R4-2017089, “WF on NR-U RRM performance”, Nokia, Nokia Shanghai Bell

- TS 38.133, “NR; Requirements for support of radio resource management”